PREVENTIVE MAINTENANCE

Dirt

What Dirt Does To Hydraulic Components?

Hydraulic component manufacturers agree that dirt is responsible for a majority of malfunctions and unsatisfactory component performance. Here is what dirt does to components:

PUMPS:

- Erodes wear plates.

- Causes sticking of vanes creating erratic action.

- Wears out rotor slots.

- Increases shaft journal and bearing wear.

- Increases gear wear with resultant inefficiency.

- Increases piston and sleeve wear with resultant inefficiency.

Dirt in compensator controls on variable volume pumps causes sticking slow response, and erratic delivery thus affecting the entire system, creating excessive heat and inefficient use of horsepower.

RELIEF VALVES

- Causes chatter.

- Causes seat wear in relief valve.

Accumulated dirt causes relief valves to fail and pressure becomes erratic which affects the SAFETY and FUNCTION of entire system.

DIRECTIONAL VALVES

- Causes plugged orifices.

- Causes wear to spool and body glands creating excess leakage.

- Causes spools to stick with resultant solenoid FAILURE.

Sticking valves can cause excessive shock loads and damage hose, piping fittings and various other components.

CHECK VALVES

- Causes wear on poppets and seats creating leakage.

Dirt in valves permits fluid to by-pass check valves.

CYLINDERS

- Causes excessive wear of cylinder rod, packing and bore.

- Causes cushions to malfunction.

HEAT

Maintaining Proper Oil Temperature

Hot oil in your Coiled Tubing Units hydraulic system is one of the primary causes of poor operation, component failure, and downtime. Here are some pointers on maintaining proper oil temperature.

The oil in your hydraulic system was designed for operation within a specified temperature range. You may be able to run it at hotter temperatures for short periods of time intermittently, without bad effects. If you run continuously with oil that is too hot, however, the hydraulic components on the CTU will operate poorly and eventually key components will fail.

HOW HOT IS “TOO HOT”?

The unit will operate satisfactorily at any temperature between 100oF and 180oF. The lower the temperature that the unit is run at will prolong the life of the equipment and the hydraulic oils. The unit should normally operate in the range of 100oF to 150oF during efficient operations, although they may sometimes go a little higher. Prolonged running over 180oF is not recommended. If high temperatures are encountered, it will most likely be because too much fluid is been dumped across the relief valves. All fluid dumped across the relief valves generates heat. The higher the pressure and/or the volume, the worse the heat generation problem the operator is causing.

The operational procedures should be studied and followed to allow the speed to be run at the lowest pump volume and pressure required, to extend the service life of the equipment.

ISOLATING TROUBLE SPOTS

To determine which components are “running hot” and overheating the oil, feel the outlet fittings and lines at the valves, pumps and motors. If the oil is normal going into a component and hot coming out – that could be one of the troublemakers.

LOOK, SMELL AND FEEL

Checking oil temperature periodically is good preventive maintenance. So too is the practice of periodically syphoning an oil sample from the reservoir, and comparing it with a sample of clean, new oil.

Oil that has been running too hot will look darker and feel thinner than new oil. It will also smell burned. Chances are, it will contain more contaminants because hot oil leads to accelerated wear of components parts.

FLUID MOTORS

- Causes wear similar to pump wear.

Dirt under sequence valves prevents closing.

FLUID

- Acts as a catalyst and breaks down molecular structure causing gummy residue varnish.

Dirt in the tank attracts additives and changes composition of the fluid.