MAINTENANCE AND MODIFICATION ON CT UNITS

During our function test been carried out in the yard on coil tubing equipment, It was observed by the appropriate authority that the following modifications and maintenance should be carried out on certain parts of the equipment in other to enhance the performance and operational functions of the equipment with regards to the parts been maintained/modified, which helps to improve equipment and personnel safety.

- COIL TUBING UNIT (PESL/CTU/002)

SPOTTED AREAS OF CONCERN

- Misaligned injector head chain block

- Absence of pad-eye for chain hook-up

- Misaligned goose neck/rollers

- Reel Misalignment of level wind arm.

- Absence of gauges on air tanks

- Sealed console windows

- Difficult route to access the top of the reel.

REMEDY

- Introduction of precisely measured spacers to the injector motors heads to correct misalignment of chain, after these, it’s confirmed OK and ready to go into operations.

- A pad-eye was constructed and welded in place.

- The goose neck rollers were dismantled and adjusted from their original positions in other to accept 11/4” tubing freely without restrictions.

- The rearrangement of the chain and sprocket to correct misalignment of level wind arm.

- Gauges are now included in each of the equipment air tanks.

- Sealed windows might not be appropriate in case of emergency/changes needed.

- For easy access route, ladders were constructed to enhance access to reel top.

- NITROGEN TANK (001 & 002)

- Installation of climbing ladders, to enable access to the tank top for cleaning and maintenance.

- COIL TUBING UNIT (PESL/CTU/001)

SPOTTED AREARS OF CONCERN:

Absence of the following items:

- Hose panel guard for reel

- Console box on power pack

- Pig launcher for reel

- External gauges on injector

- Chain hooks on injector

- Removable hose reel on injector

- B.O.P stand

- Faulty Air condition in console.

REMEDY

*******Above items have been put in place except for the air condition that remains to be cleared out.

- NITROGEN PUMPER (002)

SPOTTED AREAS OF CONCERN:

- Wrong Positioning of thermostat.

- Faulty Electrical Switch

- Support frames and guards for rotating parts

REMEDY

- Actual positioning of thermostat to enhance adequate heat transfer and cooling effect all around the unit, now the unit is working perfectly well and confirm ok.

- Found broken and was changed, its working perfectly and ok.

- Addition of frame support on the N2 discharge line, with protective guard on the glycol pump.

5. NITROGEN PUMPER (001).

SPOTTED AREAS OF CONCERN:

- Nonfunctional pressure relief valve

- Improper piping and vents in glycol tank

- Support frames and guards for rotating parts

REMEDY

- Full- flow (PRV) was stocked due to rust, causing excess pressure buildup in the glycol tank and causes it to over flow during operational activities and when not in operational activities.

- Hoses to the glycol tank and the vaporizer pot were all wrongly connected, leading to poor fluid circulation when engine is running (heat ex – to pot and pot to glycol tank)

- Introduction of vent in glycol tank to enable to depressurize the tank when the engine is running in other not to cause spoiling of the tank contact on the unit floor.

- Addition of frame support on the N2 discharge line, with protective guard on the glycol pump.