- Close master and wing valves on the string.

- Rig up Riser (if required), Flow-tee and Coil Tubing BOP on the string, function test Blind rams, shear rams, Tubing slips and Pipe rams.

- Test Blind rams of the BOP to 500 psi against the Swab valve of the String and test the delivery line of the pump unit of the BOP to 5000 psi against the Swab valve of the String using water.

- Feed Coil Tubing through Injector Head and assemble tools as follows:

- 1 x 6ft straight bar

- 2 x 1" check valves.

- Displacement or Jetting nozzle

- Rig up the injector head on the BOP stack, pull tools into stuffing box and apply 1000lbs tension to check the weight indicator and tool string connection.

- Position 6ft bar across the stuffing box, tubing rams and slip rams. Apply 700 psi to the chain tensioners and 3000 psi to the stuffing box. Pressure up to 3000 psi through the Coil tubing and test the riser, BOP body, Stuffing box and coil tubing (Bleed off pressure through the wing valve).

- Pull back Coil tubing into the stuffing box.

- Open the Swab valve and run Coil Tubing according to the program.