1.11 Planned Disconnect

A planned disconnect procedure is intended to be used when the barge can no longer keep station and must move away from the platform before there is time to POOH and normally rig down the tubing and connections. A planned disconnect will take approximately 30 – 45 minutes, and may be changed to an emergency disconnect at any time. The following sequence to be followed:

- The Senior Service Supervisor instructs the crew to execute a planned disconnect.

- The injector operator notifies the reel operator, and then stops the injector.

- The injector operator notes the depth of the CT, then sets the pipe and slip rams.

- The Senior Service Supervisor directs all fluid and nitrogen pumping to stop and depressurize, and confirm all pressure is zero.

- All personnel are evacuated from the platform and barge deck except the injector operator, service supervisor, reel operator, and barge supervisor.

- If there is no danger of collapsing the coil, the reel operator bleeds the pressure off the coil and inflow tests the BHA check valves.

Note

If there is danger of wellhead pressure collapsing the coil, the injector operator should close the shear and seal rams to control the well before bleeding off internal pressure.

- Disconnect and stow all data, video, and power cables between the platform and barge

- The barge service supervisor disconnects the coflexip hoses from the barge.

- The area between the platform and tubing reel is cleared of all personnel.

- If the inflow test confirms that the BHA check valves are sealing properly, the reel operator shifts the BOP valve to “close” to cut the tubing then sets the reel brake.

Danger

All personnel must be clear of area between the reel and platform before cutting the tubing. Injury or death may result from being struck by the loose end of the tubing or hazardous fluid discharge.

- The barge service supervisor reports by radio that disconnect is complete.

- The injector operator closes the manual locks on the pipe and slip rams and closes the kill and flow tee valves to secure the well.

- After engaging the BOP, the BOP hoses are manually disconnected from the barge end.

- The Injector power and control hoses are manually disconnected from the barge end.

- The Senior Service Supervisor directs the barge captain to move the barge away from the platform.

1.12 Emergency Disconnect

The emergency disconnect procedure takes place in approximately 30 minutes and is only used if there is no time to follow the planned disconnect procedure. It is important to realize that the planned and emergency disconnect procedures are only designed to release the barge from the platform and vice versa.

Emergencies involving well control should be handled in the conventional manner.

1. 0:00 The Senior Service Supervisor instructs the crew to execute an emergency disconnect.

The barge captain initiates a red flashing light and audible alarm.

2. 0:10 The Senior Service Supervisor directs all fluid and nitrogen pumps to stop and

Depressurize. Needs to confirm that all pressure is bled off and zero.

3. 0:10 The injector operator notifies the reel operator, then stops the injector.

4. 0:20 The injector operator notes the depth of the CT, then closes all BOP rams.

5. 0:30 All personnel are evacuated from the platform and barge deck except the injector operator, service supervisor, reel operator, and barge supervisor. The operator and supervisor on the platform will be recovered later.

6. 0:35 The barge service supervisor disconnects the coflexip hoses and evacuates the rear of the barge.

7. 0:45 The reel operator shifts the BOP valve to “close” to cut the tubing then sets the reel brake.

Danger

All personnel must be clear of area between the reel and injector before cutting the tubing. Injury or death may result from being struck by the loose end of the tubing or hazardous residual fluid discharge.

8. 0:55 All data, video, and power cables between the platform and barge are cut using bolt cutters stowed nearby.

9. 1:00 The barge service supervisor reports by radio that disconnect is complete.

10. The injector operator closes the manual locks on all the BOP rams and closes the kill and flow tee valves to secure the well.

11. After engaging the BOP, the BOP hoses are manually disconnected from the barge end.

12. The Injector power and control hoses are manually disconnected from the barge end.

13. The Senior Service Supervisor directs the barge captain to move the barge away from the platform.

Note

Once an emergency disconnect is initiated, it cannot be reversed. If communications are lost, the disconnect sequence is not affected.

APPROVED BY:	VICTOR UADIALE

SIGNED:		________________

DATE:			________________