CT Equipments

Fill removal operations can induce significant levels of fatigue from repeated cycling of the

tool string over a localized area and from high pressure jetting operations. The CT string must

be able to withstand the predicted fatigue effects.

Use a CT string with the largest possible OD to decrease the friction pressure loss inside the

CT string for a given flow rate. The large OD will serve to increase the annular velocity for a

given flow rate.

For reverse circulation, install a reversing manifold ahead of the reel inlet so that the CT operator

can easily switch the flow path from forward (down the CT string) to reverse (up the CT

string). Use piping with ID of 2 in. or more to minimize pressure loss.

Pressure Control Equipment

Configure the pressure control equipment to avoid circulating corrosive or solids-laden annular

return fluids through the secondary BOP. Install a pump-in tee between the secondary BOP

and hydraulic master valve for fluid returns.

Auxiliary Equipment

Ensure that the fluid mixing, handling, and pumping equipment have adequate capacity. Configure

them to minimize cross contamination of the fluid stages.

For live well operations, use a choke manifold to control annular returns. Ensure that the

CTU, pump, and choke manifold operators have a clear line of communication between each

other and the Customer well-site supervisor.

Ensure that the solids removal equipment can operate efficiently at the fluid flow rate necessary

for the job.

Downhole Tools

The following are some basic requirements for tools for fill removal operations:

- The tool string must not restrict the flow rate required to provide the desired annular

velocity.

- Tools must be capable of operating with high solids content fluid.

- The operation and components of the tool must be compatible with treatment fluids.

- The OD profile of the tool string should be as slim as practical. In addition, the profile

must not contain sudden or large changes in OD, which can induce sticking.

The BHA used for fill removal should include the following components, from the top down:

- CT threaded connector.

- Dual flapper type check valves.

- Straight bar—Use approximately 6 ft. This optional item is especially helpful for running

through upsets in the completion, such as side pocket mandrels.

- Jetting nozzle—There are no special nozzle requirements for conventional fill removal. It

can be as simple as a few holes drilled in a bull plug. For reverse circulation, use a nozzle

with a large hole (about 0.75 in.) on the axis of the CT and four smaller holes (about

0.25 in.) equally spaced around the circumference of the nozzle.

Monitoring and Recording Equipment

The CT unit must include a DAS capable of monitoring the pump pressure, WHP, CT depth,

CT weight, and fluid flow rates. The data acquisition software should provide the operator

with a real-time display of tubing forces, operating limits, remaining working life, fluid flow

rates, and total fluid volumes pumped

Generic Procedure for a Coiled Tubing fill Removal

Operational Consideration

In removing fill from the wellbore, it is necessary to ensure the following:

- Transport materials to an appropriate place for separation and disposal.

- Do not place fill materials in areas where they could interfere with the operation of the

Wellhead, production, or pressure control equipment.

- Maintain wellbore conditions to avoid introducing the fill material into the formation and

damaging the formation.

Preparing the Wellbore

If required for safety, fluid compatibility, or production reasons, kill the well. In most cases

the risk of damaging the near wellbore area by bullheading the wellbore fluid is unacceptable.

Consequently, the well kill operation may be a part of the wellbore preparation conducted

through the CT string.

Remove or secure the SSSV. If securing it, hydraulically isolate it in the open position and

install a protective sleeve.

Preparing the Equipment

- Prepare the wellhead and surface facilities.

- Rig up the surface CT and pressure control equipment

- Assemble the BHA and connect it to the CT

- Conduct wellsite pressure tests

Preparing the Fluids

Either batch prepare fluid in advance, or mix it on the fly as the operation proceeds. Allow

sufficient residence time to enable the desired fluid characteristics to develop (and be tested).

Regardless of the fluid mixing procedure, prepare adequate volumes. Once the operation has

commenced, any interruption to circulation is undesirable.

Performing Coiled Tubing Fill Removal

- RIH while pumping inhibited water at a slow rate. Pay close attention to the maximum running speed and location of Pull Test.

- Gently tag the targeted obstruction. Do not tag the fill unless pumping through the nozzles. Running into the fill without circulation could plug the nozzle.

- Pick up about 25 ft and increase the pumping rate to the planned maximum rate. Wait for the treatment fluid to reach the BHA.

- After the pumping pressure stabilizes, RIH slowly while watching for the signs of sand accumulations around the BHA and CT.

- Keep the BHA moving at all times. However keep in mind that multiple passes over a short interval can rapidly fatigue the CT string. Closely monitor the accumulation of fatigue to avoid a failure in the CT string.

- Pick up the BHA to the original top of the fill to ensure that the tubing is not restricted by sand.

- In deviated wellbores, perform wiper trips as necessary to prevent the formation of excessive beds or dunes in the transition area between vertical and horizontal sections. Gauge the number of wiper trips by the volume of fill recovered for the volume of the wellbore penetrated. If the drop-out volume is small, one can perform fewer wiper trips to avoid .

- Pump high viscosity fluid pills if necessary to suspend entrained solids.

- Continue pumping at a high rate until the required depth interval has been traversed.

- At the bottom of the cleanout interval, continue pumping at a high rate until sand production ceases. Pump high viscosity fluid pills if necessary

- POOH while pumping inhibited water at a high rate if possible.

- If in doubt about the condition of the well, perform drift run.

- If the Well does not flow naturally or aided by gas lift, prepare to conduct Nitrogen lift operation

Monitoring a CT Fill Removal

The following are monitored during fill Removal.

- Fluid flow rate-It is relatively easy to determine the appropriate fluid flow rate for single-

phase fluids. Monitor the returning fluid for fill material. If the returns do not contain          enough solids, increase the fluid flow rate. For two phase fluids, such as foams,co-

ordinate the flow rate of each phase to ensure the correct foam quality is generated  downhole.

- Fluid Composition – Carefully mix and store fluids that are specifically engineered for optimum  solid carrying capacity (e.g Gells ) under conditions that will prevent contamination, dilution, or settling of active component.

- Wellbore returns (liquid)- Monitor the flow rate and volume recovered from the wellbore to confirm the pump fluid is not being lost.

- Wellbore returns (Solid)- Measure and assess the rate and volume of removed fill to confirm progress and to gauge the efficiency of the operation. Compare variations in treatment rate or fluid composition with solids removal efficiency. The production or reservoir engineer ultimately determines how much fill material must be removed and how to evaluate the success of the operation.

Safety issues for a CT fill Removal

Always conduct a pre-job safety meeting prior to RIH.

Potential safety hazards for a fill removal include:

- Erosion of surface components by abrasive materials could create leaks or damage valves

- Some fill material (LSA) could be hazardous to personnel

- Operations involving Nitrogen gas pose the same hazards as unloading a well with Nitrogen.

And just as freedom cannot exist when people are imprisoned for their political views, true opportunity cannot exist when people are imprisoned by sickness, or hunger, or darkness.

Access to electricity is fundamental to opportunity in this age.  It’s the light that children study by; the energy that allows an idea to be transformed into a real business.  It’s the lifeline for families to meet their most basic needs.  And it’s the connection that’s needed to plug Africa into the grid of the global economy.  You’ve got to have power.

And that’s where Robert Kennedy delivered his eloquent address to another generation of young people.  The challenges of our world, he said, demand “the qualities of youth; not a time in life, but a state of mind, a temper of the will, a quality of the imagination, a predominance of courage over timidity, of the appetite for adventure over the love of ease.”

“I am fundamentally an optimist.  Whether that comes from nature or nurture, I cannot say.  Part of being optimistic is keeping one’s head pointed towards the sun, one’s feet moving forward.”

You produce the raw materials, sold cheap, and then all the way up the chain somebody else is making the money and creating the jobs and the value.

There’s a lot of variation.  Some countries are doing great work when it comes to accountability and democracy, and an act of civil society, and a free press, and freedom of assembly.  And some countries are not doing as well.  But what’s exciting right now is you’re starting to see more and more a norm, a standard, take hold in Africa.  And young people, I think especially, have high expectations about how government should function, and it should function for the public good, not for the benefit of just a few.  And people should be able to speak their mind, and they should be able to organize without fear of retribution.  And they should be able to cast a ballot without problem.

It is my strong belief that terrorism is more likely to emerge and take root where countries are not delivering for their people and where there are sources of conflict and underlying frustrations that have not been adequately dealt with.

I think the more that we’re giving people opportunity, the more that we’re giving people education, the more that we’re helping resolve conflicts through regular democratic processes, the less likely they are to take root.

Our founders believed that those of us in positions of power are elected not just to serve as custodians of the present, but as caretakers of the future.

That’s how we learned what leadership really means.  It’s about stepping outside of your comfort zone to explore new ideas.  It's about rising above old divisions.  It's about treating people the way you want to be treated in return.  
 
So many of the qualities that we Americans hold dear we imported from this land -- perseverance, faith, an unbending belief that we make our own destiny, and an unshakable dream that if we work hard and we live responsibly, something better lies just around the bend.
 
So our histories are bound by blood and belief, by culture and by commerce.  And our futures are equally, inextricably linked.

THE PRESIDENT:  Now, ma'am, let me finish.  Let me finish, ma'am.  Part of free speech is you being able to speak, but also, you listening and me being able to speak.  (Applause.)

that lasting prosperity comes not from what a nation consumes, but from what it produces, and from the investments it makes in its people and its infrastructure.

And this place can help change that.  It can help light the fire of imagination, plant the seed of noble ambition in the minds of future generations.

For all the challenges of a changing world, for all the imperfections of our democracy, the capacity to reach across our differences is something that’s entirely up to us.