NIGER DELTA PETROLEUM RESOURCES LIMITED

PROPOSAL

ON

OGBELE 2S STIMULATION

FEB, 2015

| TABLE OF CONTENT |  | PAGE |
| --- | --- | --- |
| 1. | Objective | 3 |
| 2. | Well History | 3 |
| 3. | Well Information | 4 |
| 4. | Well Diagram | 5 |
| 5. | Treatment Recipe (Ogbele 2S) | 6 |
| 6. | Treatment Procedure | 7-8 |
| 7. | BHA Schematic | 9 |
| 8. | CT Schematic | 10 |

NIGER DELTA PETROLEUM RESOURCES LIMITED

OGBELE 2T

WELL OBJECTIVE

To clean/wash out the D6.0 perforations using CT and HCL or HF to enable resumption of water injection via Ogbele 2SS

WELL HISTORY

Ogbele–2 was completed on the 29th of Jan 2007 as a TSD oil producer on the E3.0 sand as long string and a produced water disposal well as short string into the D6.0. Both strings are 3-1/2” ACME tubing.

The long string producing from E3.0 was shut down because of low oil rate and inability to manage high gas rate.

The short string has been used for produced water injection. It operated at an injection rate of about 2,500b/d at an injection pressure of about 100psi. However, production issues from well 3 on November 1, 2012 ensured that no water injection, a sharp rise in injection pressure was observed from 100 to 1200psi. The injection process was therefore terminated.

Scale formation across the perforation is suspected as a result of incompatible mixing of formation water and injection water. Bacteria action might also be responsible for plugging the perforations. The possibility of pump malfunction has been eliminated as the pump and other surface equipment seems to be working properly.

The gradual decline in injectivity can be attributed to two (2) main causes:

- Particle plugging at the perforation: This phenomenon was evidenced by the good reaction to the HCL was during the initial treatment and the water quality hasn’t changed, this has probably been enhanced by the higher velocities in the injection lines that followed stimulation (rate increased).

- Clay destabilization by fresh water (illite migration). Gradual decline is often associated with migration (pressure drop proportional to cumulative volume, usually slower in injectors as the flow is divergent).

Other attributes are;

- Scale: no water analysis was available to assess the potential for auto-scaling of the injection water, scale and corrosion inhibitor are injected in the water so this scaling tendency should be minimal.

- Bacteria: No treatment of bacteria will be included in the program.

Recommendation

It is therefore, recommended to use CT and HCL acid to wash the perforations in other to remove the scale. Since this injector is completed with perforated linen we recommend a HP jetting tool of the perforation and thus allow the well to inject water at the desired injection rate.

WELL INFORMATION

| Field | OGBELE 2T |
| --- | --- |
| Well ID | NPDR-02 |
| Casing | 9 5/8” P-110, 53.5# |
| Tubing | 3 1/2 “ HCS 9.3# N80 (LS and SS) |
| TD | 11,487ft |
| Upper Perforation (D6.0) | (9,000—9,050)ft |
| Lower Perforation(E3.0) | (10,050—10,070)ft |
| Cement plug depths | 10,244ft and 11,00ft |
| Well Pressure | 5000psi |
| Injection rate | 2,500b/d |
| Injection Pressure | 100psi |
| Flow rate | 10ft/min |

Treatment Recipe (Ogbele 2S)

| SAND | D6.0 |
| --- | --- |
| Length of Perforations to be stimulated (ft) | 50 |
| Pumping order and additives details | Gals |
| CLEANING FLUID 1 |  |
| TUBING PICKLE VIA CT(Half Tbg Capacity) |  |
| 5% HCL Acid (50bbls) | 2100 gals |
| Hydrochloric acid 33% HCL | 279.7 gals |
| Fresh water | 1820gals |
| Corrosion inhibitor A260 (2 gals/1000gals acid) | 2 |
| Surfactant F100 (4gals/1000gals acid) | 2 |
| Chelating Agent U042 (20gals/1000gals acid) | 5 |
| Nitrogen(250scfm) |  |
| Pickle displacement – 120 bbls of 3% NH4Cl | 5040 gals |
| Ammonium chloride NH4CL | 1,260lbs (573kg) |
| CLEANING FLUID 2 |  |
| INJECTIVITY TEST FLUID |  |
| 3% NH4cl(10 bbls) | 420 gals |
| Ammonium chloride NH4CL | 105lbs (48kg) |
| CLEANING FLUID 3 |  |
| ACID STIMULATION BY BULLHEAD |  |
| 6000 gals 15% HCL containing(100-150 gals/ft) | 6000 gals |
| 0.2 % A260 Corrosion Inhibitor | 12 |
| 0.4% F75N Surfactant | 24 |
| 1.0%U042 Iron Control Agent | 6 |
| Hydrochloric acid 33% HCL | 2,514.6gals |
| Fresh water | 3,486gals |
| Displacement - Twice Vol of main fluid | 12000 Gals |
| 3%NH4CI | 3003lbs (1,365KG) |
| 1% U066 | 120 |
| 0.2%W054 | 24 |

# TREATMENT PROCEDURE

DAY 2: 5% NITRIFIED FOAM TUBING PICKLE

- Hold safety meeting with all relevant personnel on location. Rig up all Poseidon Well Services equipment as per Poseidon Safety Standards. All personnel not required for the treatment should remain out of the location.

- Perform Upfront Slickline gauge cutter run at XN Nipple at 8937 ft. POOH Slickline and lay down lubricators.

- Perform pull test, pressure test and function tests as per PT-1 and PT-2 requirements with clean filtered water. The coiled tubing 1.68” bottom-hole assembly will include the following: coiled tubing connector, tandem check valve, hydraulic disconnect, and Jetting nozzle.

- Open well to flow and RIH coiled tubing at 80 ft/m to 8918 ft-MD while pumping 5%  Nitrified Foamed HCl at a rate of 0.3bpm and N2 @ 250Scfm to ensure that coiled tubing is full of fluid. Perform pull tests every 1000 ft.

- Once coiled tubing is at 8918-MD, increase liquid rate to 0.51bpm and increase nitrogen pumping rate to 400scfm to achieve the optimum jetting effect at the nozzle. CT at 8937 ft (XN Nipple), RIH CT gradually conducting pull test at every 20 ft increment. Wash tubing and slotted liner from 8918-MD to 9,050ft-MD (10 ft from the float shoe).

- Once coiled tubing is at 9,050ft MD, continue CT pipe movement and washing across the slotted liner to ensure all debris/mud/sand are out of well. Check drags every 10 minutes to ensure CT is free. Displace 5% Nitrified foamed pickle with about 120 bbls 3% NH4cl brine. POOH CT and RD CT Injector head.

Day 3 : 15% HCL TREATMENT (BY BULL-HEADING)

- Rig up pump lines to Xmas tree valves and perform Fluid injection test using 10 bbls 3% NH4CL at varying rates from 0.5bbls per min and gradually increase to the initial injection pressure. If okay, mix and bullhead 6000 gals 15% Hcl.

- If the injectivity test result is poor, Rig up CT and perform the 6000gals 15% HCL treatment through CT. RIH CT exiting the tubing pumping the 15% HCL wash fluid across the perforations.

- Displace 15% HCL treatment fluid in CT with 3% NH4CL twice the volume of main 15% HCL treatment.

- POOH CT to surface.

Day 4: REPEAT INJECTION TEST WITH PLANT TREATED WATER

- Hook-up water line to the well and test formation injectivity.