NIGER DELTA PETROLEUM RESOURCES LIMITED

PROPOSAL

ON

OGBELE 2T SIMULATION

JULY,2014

| TABLE OF CONTENT |  | PAGE |
| --- | --- | --- |
| 1. | Objective | 3 |
| 2. | Well History | 3 |
| 3. | Well Information | 4 |
| 4. | Well Diagram | 5 |
| 5. | Treatment fluids Systems | 6-7 |
| 6. | Treatment Recipe | 8-9 |
| 7. | Recipe Composition — Mud Acid Treatment | 10 |
| 8. | Treatment Procedures | 11-13 |
| 9. | BHA Schematic | 14 |
| 10. | CT Schematic | 15 |

NIGER DELTA PETROLEUM RESOURCES LIMITED

OGBELE 2T

WELL OBJECTIVE

To clean/wash out the D6.0 perforations using CT and HCL or HF to enable resumption of water injection via Ogbele 2SS

WELL HISTORY

Ogbele–2 was completed on the 29th of Jan 2007 as a TSD oil producer on the E3.0 sand as long string and a produced water disposal well as short string into the D6.0. Both strings are 3-1/2” HCS tubing.

The long string producing from E3.0 was shut down because of low oil rate and inability to manage high gas rate.

The short string has been used for produced water injection. It operated at an injection rate of about 2,500b/d at an injection pressure of about 100psi. However, production issues from well 3 on November 1, 2012 ensured that no water injection, a sharp rise in injection pressure was observed from 100 to 1200psi. The injection process was therefore terminated.

Scale formation across the perforation is suspected as a result of incompatible mixing of formation water and injection water. Bacteria action might also be responsible for plugging the perforations. The possibility of pump malfunction has been eliminated as the pump and other surface equipment seems to be working properly.

Recommendation

It is therefore, recommended to use CT and HCL acid to wash the perforations in other to remove the scale. Since this injector is completed with perforated linen we recommend a HP jetting tool of the perforation and thus allow the well to inject water at the desired injection rate.

WELL INFORMATION

| Field | OGBELE 2T |
| --- | --- |
| Well ID | NPDR-02 |
| Casing | 9 5/8” P-110, 53.5# |
| Tubing | 3 1/2 “ HCS 9.3# N80 (LS and SS) |
| TD | 11,487ft |
| Upper Perforation (D6.0) | (9,000—9,050)ft |
| Lower Perforation(E3.0) | (10,050—10,070)ft |
| Cement plug depths | 10,244ft and 11,00ft |
| Well Pressure | 5000psi |
| Injection rate | 2,500b/d |
| Injection Pressure | 100psi |
| Flow rate | 10ft/min |

TREATMENT FLUIDS SYSTEMS

#### CT Pickle

210 gal (5 bbls) of 15% HCl

1.6 gal of A260 (0.75 %)

Pickle the coil tubing at surface using 15% HCl (Note: The CT acid pickle may not be necessary if CT has been recently used).

A Corrosion Inhibitor A260 is added to provide protection for coiled tubing pitting and excess metal loss.

Circulating Fluid

Nitrified 3% KCl

32,000 scf (350 gals) of nitrogen

Nitrified 3% KCl will be used to circulate well while running in hole with coiled tubing to Wash vertical section with rotary Jet nozzle.

### Acid treatment fluid

4200 gals of 15% HCl

2.1 gal of A260 (0.05 %) corrosion inhibitor

21 gal of F75N (0.5 %) EZflo surfactant

109.2 gal of U42 (2.6%) chelating agent

63 lbs of L58 (15 ppt) iron stabilizer

A Corrosion Inhibitor A260 is added to provide protection for steel tubulars against pitting and excess metal loss.

A Surfactant F75N is added to improve the penetration of the acid by reducing any surface tension between the formation fluid and the treating fluid.

A chelating agent U42 is added to prevent ferric hydroxide precipitation.

An iron stabilizer L58 is used to prevent the precipitation of dissolved iron from spent acid.

#### Acid Diverter

1260 gal (30 bbls) of 3 % KCl brine (318.4 lbs of KCl)

18.9 gal of F101 (1.5%) foam diversion additive

14,400 scf (155 gals) of nitrogen

#### Displacement of treatment fluid

1050 gal (25 bbls) of 3% KCl brine (265.3 lbs of KCl)

#### Mud Acid treatment

2100 gals of ½ strenght mud acid (8:1 HCl: HF)

512.8 gal of H033 (33% HCl)

263.5 lbs of Y001 Intensifier

#### Mud Acid Diverter

1260 gal (30 bbls) of 3 % KCl brine (318.4 lbs of KCl)

18.9 gal of F101 (1.5%) foam diversion additive

14,400 scf (155 gals) of nitrogen

#### Displacement of mud Acid treatment

1050 gals (25 bbls) of 3 % KCl brine (265.3 lbs of KCl)

#### Displacement fluid

2100 gal (50 bbls) of 3 % KCl brine (530.6 lbs of KCl)

#### Injection water

100 bbls of injection water

# PRODUCTS NEEDS

# Treatment Recipe (UGD 1S)

| TUBING PICKLE VIA CT | VOLUME IN BBL |
| --- | --- |
| 15% HCl Acid | 10 BBL |
| Fresh Water |  |
| Corrosion inhibitor A260 Surfactant F78 |  |
| Iron control |  |
| TOTAL |  |
| TUBING SPACER– 3% NH4Cl | 8.5 bbl |
| U66 | 1.5 BBL |
| TOTAL |  |
| ACID STIMULATION BY BULLHEAD |  |
| PREFOAM– 3% NH4Cl and additives | 18 BBL |
| F78 Surfactant | 3 bbl |
| TOTAL |  |
| FOAM-3% NH4CL | 6 bbl |
| F78 surfactant | 1.1 bbl |
| Co-mingle liquid with N2 @ 300 scf/min. |  |
| TOTAL |  |
| PREFLUSH (3% HCL) | 26 BBL |
| Fresh Water |  |
| U042 Iron control | 1 bbl |
| A260 Corrosion inhibitor | 0.5 bbl |
| F78 Surfactant | 0.5 bbl |
| Clay Stabilizer (4 gals /1000 gals acid) | 4 gals |
| TOTAL |  |
| MAIN FLUSH 8% Mud Acid | 42 bbl |
| Fresh Water |  |
| A260 Corrosion inhibitor | 0.5 bbl |
| U42 Iron control | 1.5 bbl |
| F78 Surfactant | 0.5 bbl |
| TOTAL |  |

| BACK SPACER -3% NH4CL | 3.5 bbl |
| --- | --- |
| U66 SOLVENT | 3.5 bbl |
| TOTAL |  |
| OVERFLUSH- Half Strength Clay Acid | 42 bbl |
| A 260 Corrosion inhibitor | 0.5 bbl |
| F78 Surfactant | 0.05 bbl |
| U042 Iron control | 1 bbl |
| TOTAL |  |
| Displacement -3% Ammonium Chloride |  |

# RECIPE COMPOSITION — MUD ACID TREATMENT

(a)PREFLUSH & OVERFLUSH ( 25 Gals/ft ).

15 % Hcl

0.5% L55

5% U066 (Mutual Solvent)

0.4% A260

0.5% A201

- 0.2% F75N

- 0.2% W054

- 2% U042

- N2 @ 350-400SCF/bbl.

(b)HALF STRENGTH MUD ACID (15 Gals/ft ).

(7.5% Hcl: 1.5% HF System) plus as under-listed:

0.5% L055

5% U066

0.4% A260

0.5% A201

0.2% F075N

0.2% W054

2% U042

N2 @ 350SCF/bbl.

(c) DIVERTER FLUID ( 5 bbls for example ):

3% NH4CL plus

0.5% F075N

1500SCF/bbl.

(d) DISPLACEMENT ( 25 Gals/ft ) :

8% NH4CL plus

5% U066

0.2% W054.

# TREATMENT PROCEDURE

- Hold safety meeting with all relevant personnel on location. Rig up all Poseidon Well Services equipment as per Poseidon Safety Standards. All personnel not required for the treatment should remain out of the location.

- Perform pull test, pressure test and function tests as per PT-1 and PT-2 requirements with clean filtered water. The coiled tubing bottomhole assembly will include the following: coiled tubing connector, tandem check valve, hydraulic disconnect, filer, centralizer, and Jetting nozzle.

- Open well to flow and RIH coiled tubing at 80 ft/m to 7050 ft-MD while pumping 3% KCl at a rate of 0.3 bpm to ensure that coiled tubing is full of fluid. Perform pull tests every 2000 ft.

- Once coiled tubing is at 7,050ft-MD, increase liquid rate to 0.51 bpm and start pumping nitrogen at 400 scfm to achieve the optimum jetting effect at the nozzle. Wash tubing and slotted liner from 7,050ft-MD to 9,050ft-MD (10 ft from the float shoe).

- Once coiled tubing is at 9,050ft MD pipe movement and pump 280 bbls of 3% KCl brine at 0.51 bpm and nitrogen at 400 scfm to wash the slotted liner twice bottoms up to ensure all debris/mud/sand are out of well. Check drag every 10 minutes to ensure CT is free.

- Mix 4200 gals of 15% HCl acid as detailed in the treatment fluids systems section and pump it inside CT.

- RIH CT to bottom liner at 9,050ft MD. Once acid is at nozzle, start POOH to 8,050ft at 10 ft/m while pumping 1400 gals (0.5 bpm & 400 scfm).

- Pump 2 bbls of 3%KCl brine only at 0.5 bpm. Then start pumping nitrogen at 400 sfcm for 12 minutes to have nitrified 3%KCl brine. After 12 minutes, stop pumping nitrogen and pump 2 bbls of 3%KCl brine only at 0.5 bpm.

- Once CT is at 8,050ft-MD, stop and RIH to 9,050ft-MD at 10 ft/m while pumping another 1400 gals (0.5 bpm & 400 scfm).

- Pump 2 bbls of 3%KCl brine only at 0.5 bpm. Then start pumping nitrogen at 400 sfcm for 12 minutes to have nitrified 3%KCl brine. After 12 minutes, stop pumping nitrogen and pump 2 bbls of 3%KCl brine only at 0.5 bpm.

- Once CT is at 9,050ft-MD, start POOH to 8,050ft at 10 ft/m while pumping the last 1400 gals (0.5 bpm & 400 scfm).

- Pump 2 bbls of 3%KCl brine only at 0.5 bpm. Then start pumping nitrogen at 400 sfcm for 12 minutes to have nitrified 3%KCl brine. After 12 minutes, stop pumping nitrogen and pump 2 bbls of 3%KCl brine only at 0.5 bpm.

- Displace treatment fluid in CT with 3% KCl brine.

- POOH CT and allow acid to react for 1 ½ hours.

- Hook-up water line to the well and test formation injectivity.  Abort job if well injects up to 5 bpm at less than 600 psig.  Otherwise proceed with the Mud acid treatment.

- Mix 2,100 gals of Half Strength mud acid treatment fluid.

- RIH CT to bottom liner at 9,050ft MD. Once acid is at nozzle, start POOH to 8,050ft at 20 ft/m while pumping 700 gals (0.5 bpm & 400 scfm).

- Pump 2 bbls of 3%KCl brine only at 0.5 bpm. Then start pumping nitrogen at 400 sfcm for 12 minutes to have nitrified 3%KCl brine. After 12 minutes, stop pumping nitrogen and pump 2 bbls of 3%KCl brine only at 0.5 bpm.

- Once CT is at 8,050-MD, stop and RIH to 9,050ft-MD at 20ft/m while pumping another 700 gals (0.5 bpm & 400 scfm).

- Pump 2 bbls of 3%KCl brine only at 0.5 bpm. Then start pumping nitrogen at 400 sfcm for 12 minutes to have nitrified 3%KCl brine. After 12 minutes, stop pumping nitrogen and pump 2 bbls of 3%KCl brine only at 0.5 bpm.

- Once CT is at 9,050ft-MD, start POOH to 8,050 at 12 ft/m while pumping the last 700 gals (0.5 bpm & 400 scfm).

- Pump 2 bbls of 3%KCl brine only at 0.5 bpm. Then start pumping nitrogen at 400 sfcm for 12 minutes to have nitrified 3%KCl brine. After 12 minutes, stop pumping nitrogen and pump 2 bbls of 3%KCl brine only at 0.5 bpm.

- Displace treatment fluid from the CT into formation with 3% KCl brine.  Allow Mud acid to react for 1 hour.

- Pump 50 bbls of displacement fluid (3% KCl plus additives) followed by 100 bbls of injection water.

- Open well to injection. Record flow meter readings and wellhead pressures for 3 hours.