NITROGEN SAFETY

Prior to Rig Up

Pre – job safety meeting should be held and attended by all personnel involved with the operation. It should cover job requirements and any special requirements.

Emergency shutdown procedures should be agreed on with contingency plans for Platform/Rig shutdown or job site evaluation.

Procedures for handling major spillage of liquid nitrogen (LN) to be discussed

Carry out full safety check on all slings, shackles, pins and frames.

Check LN Tanks for condition of pipework and valves. Make full pre-check as per Nitrogen Maintenance Manual.

Ensure equipment is spotted in well – ventilated area.

Rig Up

Ensure that Rig Up/Hot Work permits are in order and inform client representative of readiness status

Coordinate with crew for rig up procedure.

Ensure that treating line is rigged up with check valve and bleed off point

Treating line should be tied down at fixed points to prevent it whipping around should the line part. Hoses should only be used in low – pressure applications, and should be tied down at frequent intervals.

Pressure test treating lines at 50% above working pressure using water if possible.

Ensure all return lines and bleed – off are rigid and firmly secured.

In the event of  a sudden release of nitrogen into the atmosphere by venting of tankers or systems there is a potential asphyxiation hazard if venting take  place in a confined space where depletion of atmospheric oxygen by the nitrogen may be possible. Equipment and hoses should be sited to avoid this risk.

Venting cold nitrogen  can produce large clouds fog. Particularly on damp or wet days. This fog may introduce a visibility hazard, which may lead to slips or falls or collision with suddenly obscured equipment. Personnel should be aware of this hazard which can arise suddenly and be aware of their surrounding and access routes.

Where nitrogen pumping is to take place on the stressed steel decks of offshore platforms and subject to agreement with the client, the following provisions shall apply in order to reduce the risk of embrittlement and cracking of decks due to liquid nitrogen spillage:

- When nitrogen pumping units and tanks have been positioned on – site, a bounded area shall be positioned underneath the pump and all tanks in use. The bund shall consist of a wooden frame and tarpaulin (or thick plastic sheet) lay on top of scaffolding boards and designed to contain any small liquid nitrogen spillage that might occur. All liquid nitrogen hoses shall be run with the confines of the bund.

- During all liquid nitrogen operations, a charged fire or water hoses shall be run to the immediate vicinity of the work and be ready to turn on. In the event of any spillage the hose shall be used to flush the area with large amounts of water in a fine spray to rapidly vaporize any liquid nitrogen in the bund.

- All liquid nitrogen tanks shall have blank or blind flanges fitted to the back fill line in order to prevent accidental discharge of liquid to the unprotected rear of the tank.

- During liquid nitrogen operations all the liquid pipework shall be constantly monitored and check for drips and leaks at all joints and connections. Any leaks discovered shall be rectified immediately and the liquid nitrogen flushed away with water.

Nitrogen Properties

Nitrogen is normally stored in liquid form and converted to gas for use in the various service applications. Both liquid and gaseous nitrogen are usually present onsite. Nitrogen has the following properties:

Molecular Weight 							28.01

Boiling Point @ 1atm 							-195.8 oc

Freezing Point @ 1atm 							-209.8 oc

Critical Temperature							-146.85 oc

Critical Pressure 								33.5atm (493psi)

Density, gas at 								21.1 oc

Density, liquid at 								-209.8 oc

Specific Gravity, gas (air= 1) 						0.967

Specific Gravity, liquid 							0.808

Latent Heat of Vaporisation @ BP 						198.9KJ/kg (85.5Btu/lb)

Refrigeration Capacity (1atm, BP to 21oc) 					423.3 KJ/kg(182 Btu/lb)

Expansion Ratio (liq to gas, Bp to 21oc) 						N2

.

Liquid Nitrogen Safety Information

Nitrogen gas is very cold and any part of any system that may come into contact with it must be suitable for the temperatures of the liquid. Unsuitable materials may brittle and lose their strength or resistance to shock loads.

Liquid nitrogen is stored, transported and handled in vacuum insulated tanks to minimize losses through vaporization. All containers and tankers are designed and manufactured according to application codes and specifications for the pressures and temperatures involved.

Personnel should never allow any unprotected part of their body to touch uninsulated pipes or vessel containing liquid nitrogen. The extremely cold material may stick fast and tear the flesh when the part is withdrawn. Continued direct contact to the cold surface will result in frostbite and serous cold burns. Even non-metallic materials are dangerous to touch. Personnel shall wear protective clothing and loose fitting impermeable gloves (to allow easy removal if liquid splashes onto or into them) at all times when working with liquid nitrogen. Special care is required when wearing wet gloves. The low viscosity of liquid nitrogen means that it will penetrate woven or porous clothing materials much faster than, for example water. If severe splashing or spraying is likely to occur eyes should be protected by goggles. Trousers should be worn outside boots.

One volume of liquid of nitrogen at atmospheric pressure vaporizes to 695 volumes of nitrogen gas at ambient temperature. Any liquid nitrogen trapped between valves in nitrogen pipework on equipment has the potential to cause excessive pressure build up and violent rupture of pipework. For this reason, fixed liquid nitrogen pipework is always protected by small, reliable, inline pressure relief valves. Cold gas/liquid is relieved safely into the units drip tray. All personnel on-site should exercise care when in the vicinity of liquid nitrogen equipment in order to avoid contact with venting cold gases or cold equipment.

Similarly, liquid nitrogen must never be allowed to be trapped between valves in temporary pipework and must never be used as a pressure- testing medium.

Ensure fire extinguisher is available by engine.

Ensure treating line is in good condition and pressure test band is in date.

Handling Nitrogen

It is the responsibility of the nitrogen operator to ensure that any person who is likely to come into contact with nitrogen during moving or operation should be aware of the hazards.

Dry insulated gloves and eyes protection should be worn while handling hoses and connections when frozen. The gloves should be removed and placed in a dry place when carrying out normal duties.

Long sleeves coveralls should be worn, with the trouser legs outside boots to prevent LN possibly entering the boot, brass hammers will be used for LN suction hose unions.

Fire hoses should be used on small spillages to disperse Nitrogen and prevent deck cracking. Do not direct hard jet at LN as this may simply deflect liquid to different area. Use fine spray to blanket area. Use fine spray to blanket area with as much water as possible without blowing LN about.

Leaks should be isolated immediately if at all possible. If there is a danger of asphyxiation or extreme loss of heat, do not enter the danger area. Inform the authorities and treat the situation as a fire.

If a leak cannot be sealed, but can be approached occurs on the tank, i.e. between tank and first discharge valve, contain the leak with drip trays and buckets. If leak cannot be safely managed in this way, pick up the tank with the crane and ensuring no personnel are below or downwind of the tank route swing over the sea and lower to within about 5meters of sea level.

Treat exposure of skin or eyes to Liquid nitrogen as a burn. Tissue in the exposed area will be dead.

Asphyxiation

Inhalation of pure nitrogen gas (by looking headfirst through a manhole into a vessel full of nitrogen, for example) can easily produce sudden Asphyxia. When inhalation of nitrogen containing little or no oxygen occurs, unconsciousness is immediate. The individual falls as if struck by a blow and may die within a few minutes unless remedial action is taken. Fortunately many manhole incidents allow air to diffuse into the immediate area and gives time for discovery and resuscitation. Sometimes the individual is less lucky and falls into the vessel while unconscious.

Where oxygen concentrations drops more slowly such as might occur due to continued venting of nitrogen into a confined space, the Gradual Asphyxia is more likely. Cold nitrogen gas is denser than surrounding warm air and will preferentially fill pits, gullies and trenches; care should be exercised when spillage or cold gas venting occurs near these.

Nitrogen is odorless and gives no warning. Gradual asphyxiation starts with oxygen depletion from 21% to 14%. The volume of breathing increases and the pulse rate accelerates. Ability to think clearly and maintain attention diminishes. Co-ordination is reduced.

From 14% oxygen to 10% judgment becomes faulty and severe injuries may cause no pain. Muscular effort leads to rapid fatigue. Emotions, particularly bad temper are easily aroused.

Individuals may still not be aware of what is happening to them.

From 10% oxygen to 6% giddiness, nausea and vomiting may occur. Individuals lose the ability to move rapidly or even move at all. Brain damage may occur even if resuscitation is now applied. Below 6% fainting, convulsive movements and death occur.

Cold Burns

At its extremely low temperature, liquid nitrogen can produce an effect on the skin similar to a burn. When spilled on a surface it tends to cover it completely and therefore cools a large area. The vapors from the liquid are also extremely cold and produce burns as well.

Delicate tissues, such as those of the eyes, can be damaged by exposure to cold gas, which is too brief to affect the skin of the face or hands.

Tissue contact with liquid nitrogen produces damage similar to that associated with thermal burns and causes severe deep freezing with extensive destruction of tissue. Area of the body affected should be flushed with large volumes of tepid water to reduce freezing. Loosen any clothing, which may restrict circulation.

Do not apply heat. Cover the affected area with a sterile protected dressing (or with clean sheets if the area is large) and protect the area from further injury. Seek medical help properly.

Frozen tissues should be treated promptly by immersion in water bath at a temperature between 40- 45oc. Avoid the use of dry heat.

Frozen tissues are painless and appear waxy with a pallid yellow color. Tissues become painful and edematous on thawing and the pale color turns to pink or red as blood circulation is restored. Potent analgesics are often indicated.

Tissues, which have been frozen, show severe widespread cellular injury and are highly susceptible to infections and additional trauma. Therefore, rapid rewarming of tissues in the field is not recommended if transportation to a medical facility will be delayed. If body temperature is depressed, the patient must be warmed gradually. Shock may occur during the correction of hypothermia. Cardiac dysrhythmias may be associated with severe hypothermia.

It’s very important we acknowledge these dangers before the start-up of any LN operation.

Operation

Fire hose should be run out to the work area and pressurized to rapidly disperse any minor spillages of LN (should they occur) by the application of ambient heat.

When nitrogen pump units and tanks have been positioned on-site, a bounded area shall be positioned underneath the pump, all tanks in use and with drip trays under unions. The bund shall consist of a wooden frame and tarpaulin (or thick plastic sheet) laid on top of scaffolding boards and designed to contain any small liquid nitrogen spillage that might occur. All liquid nitrogen hoses shall be run within the confines of the bund.

All N2 discharge connections should remain capped off until required. Check block valves are closed

Over pressure shut down on unit should be set to maximum working pressure +10%

Ensure N2 temperature is approximately ambient. Subzero N2 can lead to frosting and embrittlement of treating lines. High N2 temperature can lead to damage to swivel and union seals.

Back up should always be available in case emergency.

Always use ear protection, especially when operating bleed off valve. Keep head and body clear of bleed off gas path when opening valve.

Rig Down

Carry out safety requirement as per rig down.

Ensure all hoses, treating pipes, pins, shackles, tools are stowed and secured safely.

Coordinate with client representative regarding back loading of equipment from location.

Fill in post job maintenance report.

Highlight all items on the equipment that is faulty or unsafe using labels and paint markers.