|  | Seplat Petroleum Development Company |
| --- | --- |

Security Classification: Restricted

Ovhor-16H Well Stimulation

Well Intervention Programme

October, 2014

| Rev | Date | Description | Issued by | Checked by | Approved by |
| --- | --- | --- | --- | --- | --- |
| A01 | 2014-09-24 | Issued for Approval | Isah Adeiza | Ismael Yahaya | Afolabi Folorunso |
| R01 | 2014-09-24 | Issued for Review | Isah Adeiza | Ismael Yahaya | Afolabi Folorunso |

ADDITIONAL APPROVAL / REVISION HISTORY TABLE

| Party |  | Name |  | Sign | Date |
| --- | --- | --- | --- | --- | --- |
| Principal P.Tech |  | Folorunso Afolabi |  |  |  |
| Well Services Manager |  |  |  |  |  |
| Revision Philosophy All draft documents for review will be issued at R01, with subsequent R02, R03, etc as required. All revisions approved for issue / implementation will be issued at A01, with subsequent A02, A03, etc as required. All revisions approved for design will be issued at D01, with subsequent D02, D03, etc as required. Documents approved for Construction will be issued at C01, C02, and C03 respectively. Documents or drawings revised as “As built” will be issued as Z01, Z02 Z03 etc. Narrative sections revised from previous approved issues are to be noted in the table below and/or highlighted in the RH margin (using the appropriate revision status) thus: ¦ A02 Previous revision highlighting to be removed at subsequent issues. Drawings/diagrams revised from previous approved issues are highlighted by 'clouding' the affected areas and by the use of a triangle containing the revision status. |  |  |  |  |  |
| Revision History |  |  |  |  |  |
| Revision No. | Date of issue |  | Reason for change |  |  |

SAFETY NOTES:-

- Obtain and daily revalidate PTW.

- Complete the required risk assessments forms to ensure safeguards are in place.

- Hold safety meeting before start of operations on each day and repeated for difficult rig-ups or high pressure pumping operations. Identify potential safety hazard, explain emergency procedures, discuss job procedures for pressure testing and assign responsibilities. Ensure that service company engineer has an equipment checklist on location. Personnel must wear the appropriate Personal Protective Equipment (PPE) for this operation.

- Use correct tools and competent personnel during rigging up/down.

- Ensure that the Wire line/E-line/CTU Operator is always in the console room to monitor their units throughout the operation.

- Gas monitoring equipment must be available on site.

- Appropriate fire extinguishing equipment will be positioned in key locations.

- All pressure equipment must be tested to test pressure rating and must have valid certificates.

- Work area must be designated as a hazardous area and barricaded from general access. All non-essential personnel would be informed to stay clear of the area.

- Ensure that all connections for hooking up pump unit to flow lines and wellhead are available and of the correct API pressure rating and metallurgy.

- During logging operations, the logging supervisor or other designated employee shall maintain direct surveillance of the operation to protect against unauthorized or unnecessary entry into a restricted area

- General well services safety practices must be applied throughout the duration of the job.

- Ensure no environmental pollution and the location must be free from trash and cleaned after daily activities and upon completion of the job.

###### DETAILED PROGRAM

1.   EQUIPMENTS SPOTING AT AGBADA 11ST LOCATION

- Spot all equipment on the Barge and tag if necessary according to the equipment layout plan. Sail to Ovhor-16H

- PRE-JOB SAFETY MEETING:

  - Hold a pre-job safety meeting with all personnel involved in the clean-up operation.

Note:

● the available exit routes/boat, shower, eye wash station, first aid station, job procedure, job potential hazards and precautions, safety equipment, smoking area etc, should all be discussed.

● Roles and responsibilities should be defined.

● After the safety meeting, all offline jobs like brine preparation, etc, should commence while the rest of the activities in the sections below are being carried out.

- NRV RETRIEVAL :

  - Retrieve NRV is there

3.2  RU slick line. Connect the hand pump to the Control line port on the Xmas Tree. Cycle the control line, and pump open the TRSCSSSV. (TRSCSSSV is self-equalizing) Increase control line pressure to 5000 psi. Hold pressure for 15mins. Monitor the control line pressure and that of the Wellhead. If ok

- RIGGING UP COILED TUBING OVER WELLHEAD:

4.1   RU CT injector head on the Wellhead. Continue to monitor the Wellhead pressure while RU CT.

5.	PRESSURE TESTING OF CTU AND FLOW LINES:

5.1   Make up High Velocity Nozzle and other clean-up BHA to CT. Pressure test CT assy to 5000 psi, the flow lines between the Wellhead and the Choke manifold to 3000psi, and all other flow lines down stream of manifold to 1000 psi for 15 minutes respectively. Record all pressure tests on charts.

- RUNNING IN OF COILED TUBING:

WELL CLEAN-UP AND LIFTING: WASHING SCREEN  (10, 800-12,090 FT)

PUMP AS PER PUMPING TABLE

Note:

- Ensure that mud (drill-in fluid) is properly circulated out.

- If returns are lost increase the nitrogen circulation rate to 300-350 scf/min, while picking up CT. Continue circulating Nitrogen until returns are re-established.

- Slow CT velocity to 10 ft/min while passing through restrictions ( TRSCSSSV, GLM, nipples, sleeves, etc.)

- Produce all spent inhibited water, acid, and produced oil into ‘flow back tanks.

- Shut in well at the choke manifold.

    - Trip the TRSCSSSV and inflow test  by bleeding off pressure to zero psi. Observe for 30 minutes

    - Flush above the TRSCSSSV, and around the wellhead with brine using the CT.

    - RD the CT from the Wellhead

7.       FLOWING THE WELL CLEAN AND TESTING:

7.1	Rig-up BELLOIL & GAS to swab valve and test same.

Open the well on adjustable choke 32/64" to clean-up to 5% watercut using the BELL & OIL Test package. Test well to 2500bpod on choke 28th/64th in stage of 4/64th.  Ensure stable pressure and flow rates before changing chokes. Start with 12/64th as minimum.

  - Pass flow through separator when the BS&W is/less than 5% and Ph above 7.3Record oil/gas rates, GOR, FTHP, FTHT, and BSW (take sample every 15 minutes for BS&W analysis.)

- DEMOBILIZATION:

  - RD all equipment on the location.

8.2 Demobilize all equipment on the location.

Note:

●   The location must be free of any form of pollution before final vacation of the location- ISO 14001 Compliant.

*	Location situation report must accompany the end well report.

- WELL HANDOVER:

  - Hand over the well to SAPELE FACILITY MANAGER ON CHOKE 28/64TH ..

- COIL TUBING OPERATIONS PUMPING SCHEDULE

| Sta # | Liquid | Liquid Volume (bbls) | Pump Rate (bpm) | N2 Rate (scf/min) | Stage Time (mins) | Cumm Time (mins) | CTU Rate (ft/min) | CT Position |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4% NH4Cl Brine water | 65 | 0.3 | 0 | 216 | 216 | 50.0-80.0 | CT from surface to top of Liner (@ 10.925 ftah) Open CT/Tbg annulus |
| 2 | 4% NH4Cl Brine water | 104 | 0.8 | 300. | 130 | 346 | 20.0 | CT at top of Slotted Liner (10,925 ftah) Start pushing CT to TOE of the Screen at 12,090 ft and back to top of screen. Repeat with CT/Tbg annulus OPEN. Ensure continuous return |
| 3 | Solvent Wash (Diesel + Xylene+NE-940) | 92 | 0.7 | 250 | 131 | 477 | 20-30 | Circulate B/U with CT at 100 ft above slotted liner |
| 3 | 4% NH4Cl Brine water Plus Nitrogen | 62 | 0.8 | 250 | 77 | 554 | 0 | Pull CT to 100 ft above top of liner (10,8001 ft). Circulate bottoms up to lift all cuttings to surface. (Check NTU and report to CE & PTC.) CT/Tbg annulus OPEN |
| 4 | Acid Wash (10 % HCl + additives) | 54 | 0.6 | 300 | 90 | 644 | 20-25 | CT at top of Screen keep ANN Open, start pushing CT from top of screen (10.900 ft) to TD (12,090 ft) (Closed return when CT get to TD) and pull back to top of Screen. Wait for 30 min for filter cake to dissolve. OPEN returns. |
| 5 | Nitrogen |  |  | 150 | 90 | 794 | 50-80 | Move CT from top of slotted liner to 3,000 ftah while circulating Nitrogen. |
| TOTAL |  | 250 bbls NH4Cl + 54 bbl Acid fluid + 4,000 gals. Nitrogen |  |  |  | ~ 14 hrs |  |  |

TREATMENT RECIPES

| Pumping order and additives details | I2.0 |
| --- | --- |
| Length of Perforation/Horizontal section | 1300 ft |
| Solution ( 500 bbls) | 21,000 gals |
| 4% NH4Cl salt (For wash/Circulating) | 100 bags of 50Kg |
| Fluid Rate | 0.3 – 0.8 bpm |
| Nitrogen Rate | 250 scf/min |
| SOLVENT SYSTEM |  |
| Mud Breaker System | 92 bbls |
| Diesle Oil | 73 bbls |
| Aromatic (Xylene ) | 19 bbls |
| AQUAFLOW/NE-940 De-emulsifier (4 gals/1000 gals acid) | 15 gals |
| ACID SYSTEM ( 10% HCl @ 1.9 gal/ft of perf ) |  |
| 10% HCl | 2270 gals |
| Corrosion Inhibitor-CI-22 (3 gals/1000 gals acid) | 7 gals |
| AQUAFLOW/NE-940 De-emulsifier(4 gals/1000 gals acid) | 9 gals |
| US-40 Water Wetting Agent (20 gals/1000 gals acid) | 50 gals |
| CITRIC ACID Iron Sequestrant (30 lbs/1000 gals acid) | 70 Lbs |
| FAW-18W Surfactant/foaming agent (5 gals/1000 gals acid) | 12 gals |
| Fluid Rate | 0.8 bpm |
| Nitrogen Rate | 400 scf/min |

Equipment, Personnel and Material

- Equipment

| Description | ##### QUANTITY |
| --- | --- |
| Dual Skid Pump Truck | 1 |
| 400 bbls brine tanks | Various |
| Flow Back Tank (100bbls) | 1 |
| Choke Manifold with Variable Choke | 1 |
| Stimulation Laboratory ( capable for oil, water and emulsion test) –Multichase/BJ Base | 1 |
| Nitrogen Pumping Unit | 1 |
| 4000 gallons Nitrogen Storage Tanks (Plus standby plus tank in the base) | 1 |
| Coil Tubing Unit complete with 1.5” Tubing Reel, Injector Skid and Control House | 1 |
| Complete wireline unit package – EMVAL | 1 |
| Illumination Package | SET |
| - Acid Transport Tanks | 1 |
| - Water Transport Tanks | 1 |
| All necessary safety equipment | 1 package |

Personnel

| NO | NAME/STATUS | JOB DESCRIPTION |
| --- | --- | --- |
| 1. | Field Engineer (1) | Ensure quality control and liaison with SPDC PE on technical issues. Also, functions as HSE Doctor |
| 2. | Field Supervisor (2) | Co-ordinate all Operations; Report to Wellserve/ P.Tech. Engineer. |
| 3 | Pump Operator (2) | Operates pumps and mixers |
| 4 | Nitrogen Unit Operator (2) | Pumping of Nitrogen |
| 5 | CT Operator (2) | Operating of Coil Tubing |
| 6 | Wireline supervisor/crew (3) | Operating of Wireline Unit |
| 10 | Helpers (8) | Assist in CT/Nitrogen/Mixing/Light and others |

Materials Requirements

| CHEMICAL NAME | QUANTITIES |
| --- | --- |
| 33% HCL ( 10 % HCL) | 620 gals 12drums |
| CI-22 | 17 gals |
| Ferrotrol-900L | 66 gals |
| NE-940 | 17 gals |
| US-40 | 166 gals |
| Claytrol | 17 gals |
| Soda Ash | 2 bags |
| AmmoniunChloride | 100 bags |
| Liquid Nitrogen | 4000 gals |
| Diesel | 73 bbls |
| Xylene | 14drums |

Volume Calculations

- Drainhole Volume Calculation

Vdrainhole = Hole volume (bbl/ft) x pay zone length,  Hole volume for 1300 ft of 8” open hole

Vdrainhole = (0.07018 bbl/ft x 1300 ft)

= 91 bbls

- =  3822 gals

Since the acid is nitrified, there is reduction in acid volume to treat the interval. This is calculated based on fractional volume occupied by nitrogen in the Nitrified acid.

Nitrogen calculations are based on bottom hole conditions of  ±2900 psi bottom hole treating pressure and 140° F.

BHTP	 = 2900 psi, 	                                               BHT	=         1400 F

Bn2 ( N2 in scf/bbl of space at these conditions)		            =	880.5 scf/bbl

qn2 (Nitrogen rate, per min. )			                       =	300 scf/min

Qn2 (Nitrogen rate, bpm = qn2 / Bn2)				=	0.34 bbl/min

Qa (Acid rate, bpm )					            =	0.5 bpm

Total Rate (Qt = Qa + Qn2)  	                                             =       0.84 bpm

Actual volume of acid required when commingling with nitrogen becomes

Qa / Qt  * (V)                                                                            =           0.5/0.84 * 3822

=               gals (54bbls) of 10% HCl

| Completion Interval | I2000N |
| --- | --- |
| Casing size and type | 9⅝" |
| Setting Depth (ftah) | ##### 11058 |
| Top Sand (ftss/ftah) | 9722/10426 |
| Base Sand (ftss/ftah) | 9819/10617 |
| Sand Thickness (ft tvd) | 97 |
| Well TD (ftss/ftah) | 9756/12059 |
| Reserves to be Developed by well @2029 (MMstb/Bscf) | 2.71/0.241 |
| Oil Gravity (0 API) | 44.8 |
| Average OOWC [ftss] | 9783 |
| Proposed Completion Interval (ftss) | 9719 - 9720 (Perforation) & 9738 – 9756 (open hole) |
| Proposed Completion Interval (ftah) | 10693 – 10723 (Perforation) & 10829 – 12059 (open hole) |
| Distance of top completion interval to OGOC (ft) | N/A |
| Distance of bottom completion interval to OOWC (ftss) | 40 (@ heel) 27 (@ toe) |
| Expected Initial Offtake [bopd] | 2500 |
| Expected THP [psia] | 415** |
| Expected CITHP [psig] | 730** |
| Expected drawdown (psi) | 115** |
| Expected BS&W [%] | 0 |
| GOR (scf/stb) | 89 |
| Tubing size [inches]/Material | 2⅞"/Carbon Steel |
| Tubing Accessories | TRSCSSSV & GLMs |
| Sand Exclusion Type | 6 5/8” Reslink Screen (225 micron) |
| Gas Lift Required | Yes (1726, 2570, and 3253 ftah) |

*   Based on 10 ft of perforated interval on the I20000A1 N

** Based on 1000 ft of drainhole on the I2000A2 N interval.

Survey Data for Simulation

MD

(ft)           Incl

(°)         Azim Grid

(°)

0                  0             92.96678945

45.41001356        0.076685651          250.67

450.04              0.76              250.67

535.9              0.12              166.68

629.09              0.39              289.83

720.29              0.18               241.3

815.52              0.09               40.08

910.44              0.22               29.81

1002.77             0.22              332.89

1096.09             0.18              333.18

1190.38             0.38              294.49

1284.9              0.18              294.09

1377.86             0.79              275.03

1471.46             0.93              271.38

1565.66               1               278.01

1659.57             0.96              281.11

1751.39             0.99              272.88

1845.01             0.72               284.7

1936.75             0.82              260.44

2029.35             0.71              295.34

2121.31             0.71              292.26

2215.21             0.81              294.02

2306.41             0.76              300.94

2399.92             0.69              295.29

2494.95             0.89              290.24

2585.48             0.87              306.71

2680.11             0.56              284.86

2775.14             0.85              290.78

2867               0.82              307.46

2963.07             0.54              277.39

3056.04             0.56              283.78

3148.69             0.35              314.71

3244.29             0.44              291.72

3336.89              0.8              312.15

3432.32             0.73              310.18

3526.14             0.96              325.94

3620.18              0.8              323.53

3713.21             1.44              326.83

3809.11             1.18               329.9

3901.66             1.55              315.27

3993.87              1.2              314.24

4088.8              1.95               315.9

4182.87             1.64              318.89

4277.17             1.65              313.63

4372.13             1.55              323.98

4467               1.81              314.74

4559.73              1.8              318.09

4655.11             2.03              319.42

4750.25             1.87              312.36

4845.69             1.74              308.24

4942.72             1.49              315.65

5038.54             1.93              321.31

5131.62             1.75              333.94

5226.94             1.49              332.71

5320.58             1.49              319.48

5415.15              1.2              313.94

5508.79             1.28              310.65

5603.03             0.35              192.88

5697.12             0.95              150.16

5792.01              0.9              130.13

5880.93             1.07              124.37

5980.93             1.04              110.58

6075.93             0.93               118.4

6171.2              1.16              114.04

6265.8              1.13              112.95

6360.41              1.2              113.85

6424.19             1.31              120.28

6523.97             1.11              125.83

6616.57             1.27              114.53

6708.41             1.34              121.31

6801.14             1.76              121.16

6893.29             1.92              124.41

6987.38             1.97              120.45

7079.11             2.29              119.85

7171.74             2.45              123.83

7264.4              2.74              124.48

7359.05             3.56              122.71

7452.57             5.35              122.12

7546.62             6.78              116.19

7639.67             8.36              110.39

7733.53             10.53             103.87

7828.23             12.86              99.86

7921.15             14.73              97.53

8013.97             16.84              96.6

8108.81             19.13              93.82

8202.71             21.03              92.85

8296.91             22.58              92.41

8392.89             24.99              92.2

8486.9              26.63              92.46

8582.57             28.62              93.19

8673.56             30.37              93.53

8767.51             33.23              93.02

8863.24             37.3               93.02

8956.21             40.36              92.41

9050.87             42.91              92.61

9145.98             43.92              91.98

9239.82             44.84              92.96

9334.45             44.92              94.08

9427.45             45.91              94.45

9523.15             49.04              94.16

9618.42             52.1               93.73

9713.9              54.26              93.47

9808.62             56.06              92.98

9905.57             56.49              93.2

10000.46             59.36              94.52

10095.48             62.85              94.58

10189.74             66.57              94.01

10283.17             68.91              92.85

10376.45             70.33               92

10471.55             72.5               91.44

10567.84             75.13              90.98

10661.2             78.21              90.93

10754.81             81.39              90.99

10822.94             83.91              90.95

10935.88             87.33              90.48

11031.76             90.29              90.62

11126.91             89.4               90.56

11219.44             88.28              90.49

11314.26             88.82              90.59

11408.14             89.43              90.57

11501.59             91.64              90.48

11594.97             91.2               89.74

11688.79             91.09              89.13

11785.91             88.02              88.65

11881.85             88.74              88.67

11976.31             90.14              88.7

12100              90.17              87.9

Appendix-h

| ## PSE - WEL JOB HAZARD ANALYSIS WORKSHEET (Including Risk Assessment) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Task Name: ##### CT/ Pumping operations ### OVHOR 16 ACID WASH |  |  | Team Composition & Company: SEPLAT/MULTICHASE |  |  |  |  | Date: 23/10/14 |  |
| Step No. | ### Activity Description of task step. | ### Hazard (potential incident) | ### Threats Who or what might be harmed ? | Severity | Likeli- hood | Risk | #### Control Measures recovery -measures |  | Action Party |
| 1 | Spotting/ Rigging up Equipment | Hitting object / personnel Heavy equipment weights Damage to equipment / cranes Potential for slips/falls | Personnel Environment Equipment Assets | H | L | L | - Pre-site inspection - Intended action observer - Driver awareness - Warning signal - Personnel awareness of intended action - Knowledge of weight to be lifted physically - Correct posture - Number of personnel to aid lift - PPE - Pre job meeting - Certified crane operator/lifting equipment. |  | Certified drivers Job supervisor Job supervisor CT helpers |
| 2 | Making up BHA to Coil tubing below injector head | Falling injector head Injury to personnel Crane failure | - Personnel - Equipment | H | L | L | - Injector stands to support injector head weight - Competent certified crane operator - Correct handling - Visual observation and certified sling / shackles - No working under suspended loads - Use of Signs and signals |  | CT Supervisor Crane operator |
| 3 | Lifting injector head onto wellhead | Falling equipment | - Personnel - Equipment | H | L | L | - Certified colour coded slings / shackles - Certified lifting eyes on lubricator - Certified competent crane operator |  | CT supervisor Crane operator |
| 4 | Stabbing lubricator on well head | Falling personnel Index amputation | Personnel | H | L | H | - Personnel awareness of intended action of crane operator - Correct handling - Certified competent crane operator - Observer |  | CT supervisor CT Helpers |
| Step No. | ### Activity Description of task step. | ### Hazard (potential incident) | ### Threats Who or what might be harmed ? | Severity | Likeli- hood | Risk | #### Control measures recovery -measures |  | Action Party |
| 5 | Xmas tree valve operating during down hole operations | - Stuck xmas tree valves - Loss of equipment | - Personnel - Well Asset | H | L | L | - Operator proficiency - Keep non essential personnel away - Designated Xmas tree valve handler - Personnel awareness of ongoing operation - Monitor valve wheel number of turns when closing |  | CT helpers Job supervisor |
| 6 | Opening well to flowline | Oil spill due to pressure release | - Environment - Community | H | M | L | - Pre-site inspection - Flowline integrity daily monitoring - Communication with flow station personnel - Pressure testing of flow line |  | CT supervisor Job supervisor |
| 7 | Securing well head at the end of the day’s work | Oil spill | - Environment - Community | H | L | L | - Operator proficiency - Designated Xmas tree valve handler - Physical confirmation |  | Job supervisor |
| 8 | Zeroing mechanical counter | Falling off reel Moving parts | Personnel | M | L | L | - Shackle down reel - CT operator to monitor - Buddy system |  | CT supervisor |
| 9 | Pressuring chicsan lines | Uncontrolled release of pressure | Personnel | H | L | L | - Ensure pressure lines are tied down securely - Visual monitoring at start of pressuring - Only designated individual at HP area - Certified/ banded lines - Pre-audited equipment |  | CT Supervisor Job supervisor |
| 10 | Mixing chemicals | - Chemical contact - Spillage Contamination | - Personnel - Environment - Loss of asset | H | L | L | - Correct PPE for task - Emergency response - Shoc cards available on site - Personnel awareness of chemical type hazard - Designated mixing crew - Water shower |  | CT supervisor Job supervisor Field Engineer |

| Step No. | ### Activity Description of task step. | ### Hazard (potential incident) | ### Threats Who or what might be harmed ? | Severity | Likeli- hood | Risk | #### Control Measures recovery -measures | Action Party |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11 | Pumping acid | - Pressure leakage from pin hole - Spills/chemical leakage and equipment failure. | - Personnel - Environment | M | L | L | - Reduce pump pressure - Displace to water with pin hole at lowest part of reel - Keep unnecessary personnel away - Personnel awareness of job in hand - SHOC Cards available - Follow OGP | CT Supervisor Job supervisor CT operator Field Engiineer |
| 12 | Nitrogen lifting | - Oxygen deficiency - Burns - Pump failure - Energized line failure | - Personnel | L | L | L | - N2 operator only near N2 unit - Warning signs - PPE with gloves - Water on site | N2 unit operator Job Supervisor Field Engineer |
| 13 | Tensioning tie down chains | Chain snapping | Personnel | H | L | L | - Certified chains - Personnel awareness | CT supervisor CT Helpers Transport drivers |
| 14 | Hammering unions | Uncontrolled release of pressure Slips | Personnel | H | M | L | - Personnel awareness - Monitor gauges - Direct singular supervision | CT supervisor Pump operator CT helpers |
| 15 | Flowing back well to flow back tank | Fire Leaking lines | - Personnel - Environment - Community | H H L | L L L | L L L | - Constant Visual monitoring of location - Warning signs - Constant monitoring of flow by designated competent person | Job supervisor CT Supervisor |
| 16 | Pumping flow back tank contents to flow station | Uncontrolled release of pressure Pump Unit failure | - Environment - Community | M | M | L | - Competent pump operator - Pre site inspection of flowline integrity - Constant monitoring of flowline integrity - Communication with flow station personnel | Job supervisor Pump operator |

| Name: Company: Signature: | Name: Company: Signature: | Name: Company: Signature: | Name: Company: Signature: |
| --- | --- | --- | --- |

FOAM ACID WASH  SITE LAYOUT (OVHOR 16H)

CT  INJECTOR   HEAD

CT  BOP

BRINE TRANSPORT

L

I

FIRE

EXTINGUISHER

WELL HEAD

T

R

FIRE  EXTINGUISHER

F

F

NITROGEN PUMP

2’’ CHOKE  MANIFOLD

CT  POWER  PACK

DUAL PUMP TRUCKS

3’’ CHOKE  MANIFOLD

U

R

E

S

A

P

M

U

P

R

E

N

A

R

T

S

N

O

I

N

L

I

O

A

W

CT REEL

Site Office

L

M

NITROGEN SECTION

Acid  Transport  TTTTruck

COILED TUBING CONSOLE

OIL AND GAS LINE

N2  LINE

TREATMENT  CHEMICALS  LINE

NITROGEN TANKS

EYE WASH STATION

PUMPING  SECTION

Return Fluids Containment

COILED TUBING SECTION

300BBL

Flow back  TANK

FIRE

EXTINGUISHER