REPORT ON NDPR LOCATION VISIT. 25/07/2014

Mr Jude Ndeh and Mr Ben Ogan left Poseidon base to NDPR base, and left NDPR base with a crew bus at about 0930 hours.

We Arrived at Ogbele flow-station at 1130am, and met with the flow station supervisor Mr. Roosevelt.

On arrival at the place we met the representative of Weatherford and Libroid Mr. West.

We were taking on an induction by the safety coordinator, immediately after that, the station supervisor took us into the facility to check the wellhead and others at Ogbele 2S/S.

Upon moving in, there is an over-head rail along the access road into the well site of about 15ft high. The tree connection is 31/2 ACME. The location is tight, but we will be able to spot the equipment and carry out the operation safely.

The wellhead pressures on Ogbele 2 are as follows:

- Long string = CITHP = 800 psi

- Short string = CI THP = 150 psi

Short string flow-line (pump down line) was blinded-up, needs a 21/16 flange x 1520 thread half connection.

On Ogbele 5, the supervisor took the crew into the location.

The wellhead pressures are as follows:

- Long string = FTHP = 2200psi

- Long string = FLP = 340 psi

- Short string = FTHP = 1300 psi

- Short string = FLP = 340 psi

There is enough space at the well site.

The crew left Ogbele Flow-station at 1430hours with the crew bus and arrived at NDPR base at about 1600 hours.