Hold safety meeting with all personnel involved in this operation-PESL crew, Client’s Rep, third party and security officer. Discuss job potential hazard and precautions, safety equipment, exit route, shower, eyewash, first aid and designated smoking area.

- Rig up CT and pumping equipment and pressure test according to the required standard practices.

- Data acquisition, martin decker must be in good working condition, charted, dated and used throughout the operation for post job evaluation.

- Connect returns line to flow back tank.

- Treatment chemicals are to be prepared in separate tanks and properly labeled for easy identification.

- Attach pressure gauges on the casing/tubing annulus and tubing/CT annulus, and monitor same throughout the operation

- PESL and Client’s Rep will rehearse stimulation treatment on the job.