- The work program requires the use of the following hazardous chemicals:

- Acid-				corrosive

- Nitrogen-				causes severe burns.

- The following precautions must therefore be taken to avoid the hazards:

- Use breathing protective equipment when working with the chemicals to avoid inhalation of vapor.

- Do not eat during the job

- Use eyes and face protectors

- Use rubber hand gloves and aprons to avoid contact with skin. Flush skin with a lot of water immediately if contacted.

- Do not touch any uninsulated pipes or vessels containing liquid Nitrogen with any unprotected part of the body. Note: the extremely cold material sticks fast and tears the flesh when withdrawn.

- Ensure there is sufficient fresh water on site at all times to flush any accidental acid contact to the body.

- Observed site pollution should be cleared up immediately before the commencement of operation.

- Ensure that all require connections for hooking up pump unit to flow lines and wellhead are available and are of the correct API pressure rating and metallurgy.

- Cellar pit should be emptied if covered with water and properly secured with steel gratings before R/U operations.

- Barrier tapes with appropriate warning signs must be posted to alert personnel of High Pressure Zones around and work in progress.

- All pre-job safety meetings must be properly documented.