Sandstone Matrix Acidizing

featuring

BJ Sandstone AcidSM

Prepared by : Jeff Hibbeler

Contributing Authors	:	Kieran O’Driscoll, Jeff Hibbeler

Edited by 		:	Bob Hall, Steve Metcalf, Joel Boles, Randy LaFollette, David Ross

Edition No.		:	Edition-1, November 1996

Preface:

This manual is to be presented to Technical Managers as part of the Product Development Plan for BJ Sandstone AcidSM.  We created this manual as a marketing tool and as a technical reference for BJ Engineers.  During the development of the product, we will issue this manual, on disk and hard copy, explain the contents and set goals for its use.  This manual is not  controlled, however, it is not to be distributed to third parties in whole form until further notice. We hope to incorporate this document into more formal, company wide training courses in the near future.  In effect, this is a training manual for trainers (BJ Technical Managers).  After the BJ Sandstone AcidSM Marketing and Technical Seminar, attendees will be responsible for training and marketing their respective area.

Many thanks to Kieran O’Driscoll for making his manual “Acidizing Concepts and Designs” available and to David Ross for supplying “Understanding Formation Damage”. Please email or fax any comments you have about the information presented to Houston International Marketing Dept. (fax.-713.895-5898, email-jhibbeler@bjservices.com, or 104700,2075@compuserve.com).

We look forward to a second edition of the manual with participation from international RTM’s, Tomball Research and The Woodlands Training departments in the near future.

1. Geologic Background.

2. Discussion of Matrix Acidizing.

2.1 Reaction Rate Parameters.

2.1.1 Surface Area.

2.1.2 Temperature:

2.1.3 Acid Concentrations:

2.1.4 Chemical Composition of the Rock:

2.1.5 Pressure:

2.2 Fracture Rates and Penetration.

3. Formation Damage.

3.1 The Effect of Formation Damage.

3.1.1 Linear Flow Through Rocks.

3.1.2 Radial Flow Through Rocks

3.2 Detection of Formation Damage

3.2.1 Productivity Index

3.2.2 Pressure Drawdown and Buildup Test

3.2.3 Completion Efficiency

3.3 Damage Mechanisms

3.4 Solids invasion:

3.4.1 Actual Micron Sizes of Critical Particles

3.4.2 Relationship of Formation Plugging to Well Productivity

3.4.3 Particle Mixing Theory

3.5 Clay Swelling and Dispersion.

3.6 Indigenous Formation Clays.

3.6.1 Clay Definition.

3.6.2 Clay Structures.

3.6.3 Common Clays.

3.7 Damage Mechanisms Associated with Clays

3.7.1 Hydration of Clays. (Swelling)

3.7.2 Dispersion.

3.7.3 Clay Migration.

3.8 Prevention of Clay Damage.

3.8.1 Clay Stabilizers.

3.8.2 Fines Stabilization.

3.8.3 Removal of Particle Blocks.

3.8.4 Dispersion of Clays.

3.8.5 Flocculation of Clays.

3.8.6 Hydrochloric Acid on Clays.

3.8.7 Hydrofluoric Acid on Clays.

3.9 Wettability Changes

3.10 Water Blocks

3.11 Emulsion Blocks

3.12 Acidizing Damage.

3.12.1 Formation De-Consolidation.

3.12.2 Fines Mobilisation.

3.12.3 Reaction By-Products.

3.12.4 Reaction By-Products Prevention.

3.12.5 Additive Incompatibility / Solubility / Dispersibility.

3.12.6 Chemical Compatibility.

3.12.7 Iron Compounds.

3.12.8 Emulsions and Sludge.

4. Acid Types.

4.1 Inorganic Acids.

4.1.1 Hydrochloric Acid (HCl).

4.1.2 Hydrofluoric Acid (HF).

4.2 Organic Acids.

4.2.1 Acetic Acid (CH3COOH).

4.2.2 Formic Acid (HCOOH).

4.2.3 HCL:HF Mixtures

5. Systematic Approach to Matrix Acid Treatment Design.

5.1 Primary Considerations.

5.1.1 Drilling, Completion and Work-over Design Considerations.

5.1.2 Drilling Mud Damage.

5.1.3 Drilling Fluid Damage in Horizontal Wells.

5.1.4 Matrix Acidizing Horizontal Wells.

5.2 Cementing Considerations.

5.3 Lost Circulation Materials (LCM).

5.4 Perforating Considerations.

5.5 New Completion Considerations.

5.6 Work-over Considerations.

5.7 Production Considerations.

5.8 Production Curves.

5.9 Treating Pressure Design Considerations.

5.10 Injection Rate and Surface Treating Pressure.

5.10.1 Low Injection Rates.

5.10.2 High injection Rates.

5.11 Establishing Pump Rate and Surface Treating Pressure.

5.12 Shut-In Times.

5.12.1 Hydrochloric Acid with Limestone.

5.12.2 Retarded or Emulsified Acid.

5.12.3 Organic Acid and HCl Acid Mixtures.

5.12.4 Sandstone Acidizing with HCl: HF Mixtures.

5.13 Bottom Hole Temperature Design Factors.

5.14 Formation Fluid and Rock Characteristic Considerations.

5.15 Mineralogy Design Factors.

5.16 Permeability Design Factors.

5.17 Porosity Design Factors.

5.17.1 Optimising HCl:HF Acid Strength from Core Flow Studies.

5.18 Reservoir Solubility.

5.19 Preflushes.

5.20 Diverting Treatment Options.

5.20.1 Foam Diverting Techniques.

5.21 Selective Acidizing (Water Stimulation Prevention).

5.22 Post-Flush Design Considerations.

5.23 Over-flushing.

5.24 Retarded Acid Systems.

5.24.1 Reaction Rate Considerations.

5.24.2 Measuring Reaction.

5.24.3 Controlling Reaction Time.

5.25 Organic Acids.

5.25.1 Mixtures of Organic acid and Hydrochloric acid.

5.26 BJ Sandstone AcidSM .

5.26.1 Background Relating to BJSSA

5.26.2 Retarded or Slow reacting acids

5.26.3 Other Retarded Systems.

5.26.4 New HF Acid Mixture

5.26.5 Theoretical Production Increase (Discription of Software)

5.26.6 Stim-Lab Core Flow Testing

5.26.7 BJSSA Conclusions:

5.26.8 Reaction Time -vs- Surface Area

5.26.9 Example Calculation

5.26.10 BJSSA Job Design

5.26.11 Guidelines for Acidizing with BJ Sandstone Acid:

5.26.12 BJSSA Fluid Design / Mixing Notes

5.27 Acid Strength.

6. Applications of Nitrogen In Acidizing.

6.1 Atomised Acid.

6.2 Nitrified Acid.

6.3 Foamed acid

7. Acidizing Additives

7.1 Corrosion Inhibitors

7.2 Alcohols

7.3 Foamers

7.4 Clay Stabilizers

7.5 Iron Control

7.6 Aromatic Solvents

7.7 Mutual Solvents

7.8 Surfactants

8. Treatment Evaluations.

9. Acid Jobs That Do Not Work.

10. Technical Reference

10.1 General Chemistry.

10.2 Reactions of Hydrochloric Acid (HCl).

10.3 Reactions of Hydrofluoric Acid (HF).

10.4 Reactions of Acetic Acid.

10.5 Reactions of Formic Acid.

11. Appendix / References

# Geologic Background.

Sandstones contain approximately 40% of the world’s known oil reserves. Most sandstone reservoirs require treatment either upon primary completion or later in the life of the well. Treatments may range from formation damage removal to true stimulation. Thus, knowledge of sandstone rock-pore network properties is critical.  Before addressing the topic of sandstone matrix acidizing, one should have a clear understanding of how sandstones are formed and to what extent their rock-pore network properties affect job design.  Following is a brief discussion of some key sandstone terms and concepts.

Sandstone - is a sedimentary rock comprised of framework grains, cement(s), clays and possibly other intergranular constituents, and (hopefully) porosity. Framework grains in sandstone are sand particles ranging in size from 0.063 to 2 millimeters across.  The “sand” in the word sandstone refers only to particle size of the framework grains, not to mineral content.  Coarser particles are termed granules, cobbles, pebbles, and boulders.  Finer particles are termed silt- and clay-sized.  Framework grains in sandstones are most commonly quartz, feldspars, and multi-mineral particles of pre-existing rock, termed “lithic” or rock fragments.

Framework grains in sandstones are indurated (bound) together into rock by cements that are chemically precipitated from waters that pass through sand bodies during burial history.  Cements commonly observed in oil and gas reservoir sandstones are silica (quartz overgrowths), carbonate minerals (calcite, siderite, ankerite, and dolomite), sulfates (gypsum and anhydrite), zeolite minerals, and sometimes clay minerals.  Cements reduce porosity in sandstones.  They also increase the hardness of the rock.  Actual hardness achieved depends on the type and quantity of cement present in the sandstone.

The clay minerals commonly encountered in oil and gas reservoirs are kaolinite, chlorite, illite, expandable mixed-layer illite/smectite (MLIS), and smectite group minerals, often referred to as montmorillonite. Kaolinite, illite, and some of the MLIS clays are generally considered to be migrating or sloughing clays. Iron-rich chlorite is the acid-sensitive clay.  Smectite and some of the MLIS materials are the swelling clays.  Clay minerals may occur as “shale laminations,” as shale or mud clasts or as detrital clay fill i.e., as minerals that were deposited mechanically along with the framework sand grains and filling what would otherwise be pore space between the grains.  Clay minerals may also be formed in sandstones either by direct precipitation from solution or by alteration of pre-existing minerals (e.g. feldspar framework grains altering in-situ to kaolinite).  These are termed the authigenic or diagenetic clays.  In general, it is those authigenic or diagenetic clays that coat the framework grains that have the greatest potential for formation damage when contacted by incompatible liquids or by turbulent liquid flow.

Porosity is the void space within reservoir rocks.  In an engineering sense, porosity may be expressed either as a percentage or as a fraction of bulk rock volume.  Porosity is normally measured and reported by the routine core analysis companies as porosity percent (of bulk volume), as determined under a set of standard benchtop conditions.  Measured porosity may range from 0-35% or more, and typically is in the 15-25% range in matrix acidizing candidate wells.  It should be noted that routine core analysis porosity values measure total interconnected porosity in the core plug or whole core sample and do not indicate pore types.

It is inadequate to describe only the total porosity of a sandstone because different pore types have a marked effect on reservoir permeability and may therefore significantly impact treatment options.  Porosity may be described as primary, i.e., pore space existing at the time of deposition, or as secondary, i.e., porosity enhanced by dissolution of pre-existing grains or cement.  Further, porosity may be categorized by type. Intergranular porosity is defined as pore space between framework grains.  Intergranular porosity is usually primary porosity.  Grain-dissolution porosity occurs as a result of unstable framework grains, such as feldspars, being dissolved by waters moving through the sand/sandstone during burial.  Grain-dissolution pores in a sandstone are essentially sand-grain sized holes in the rock matrix.  Natural fracture porosity is another porosity type.  It may occur at any scale from microcracks in the rock visible only under a microscope to large joint systems that may be hundreds of feet in height.

Porosity may also be characterized by the size of individual pore entry and exit points, termed “throats.”  Pore throats having radii less than 0.5 microns were first defined as microporosity by Pittman, 1975.  Pore throats having pore throat radii greater than 0.5 microns are termed macroporosity, with some authors breaking out a middle ground termed mesoporosity.  Smaller pore entry and exit channels (throats) cause permeability to be poor and, in general, microporosity is considered to be ineffective in contributing significantly to oil and gas production in the reservoir.

Permeability is a measure of flow resistance.  Increased permeability implies reduced resistance to flow. As described above, permeability is controlled by pore-throat size range.  It is important to note that several different “permeability” values may exist for a given rock.   A standard routine core analysis permeability is a “specific gas permeability” value.  This value represents permeability to gas on a core plug or cylinder of whole core that has been cleaned, dried to constant weight in an oven at 180 degrees F or higher temperature, and thus contains no liquid.  This value is the highest permeability that can exist for a given sample.  Permeability to either gas or oil will decrease as water saturation is increased in a core plug.  Gas or oil permeability values measured at some water saturation are “relative permeability” values.  Permeability values measured at “benchtop” or routine core analysis conditions will also decrease, sometimes dramatically, as net overburden is increased to simulate reservoir in situ conditions.  Of special interest is percussion sidewall core permeability.  Sidewall core permeability values are not derived from flow resistance measurements.  They are generated by applying a porosity value to a nomograph and should be considered as estimates only.  The most desirable permeability values to use are in situ permeability numbers from pressure transient tests.  However, it should be noted that these values represent average permeability over a considerable thickness of rock through which permeability is most probably non-uniform.

To summarize, the four major constituents of sandstones are framework grains (rock matrix), cements (binding agents), clays and other intergranular constituents, and porosity, which occurs as various types, and with different throat sizes.  When considering permeability, it is critical to know which permeability value is being discussed, what permeability level is present, and how permeability is distributed in the reservoir.

Sedimentary - A sedimentary  rock results from the transformation of a sediment (fluvial or aeolian for sandstones) into a rock through a process called diagensis.  Originally the sediment is a sand composed of silicates (quarts, feldspar, and possibly granitic fragments).  The coarse particles of this sand (0.05 to 2 mm) form the framework of the sediment.  Smaller particles (clays, silts, lime mud) can also be deposited at the same time.  When these associated minerals are deposited along with the original sand they are termed diagenitic.  The original porosity of the sediment is governed by the following parameters:

- Size of the particles

- Shape of the particles

- Packing of the particles

- Particle size distribution (uniformity)

Diagenesis - overburden pressure compacts the sediment  and a cementing process transforms the sediment into a rock.  This is also called lithefication.  The common cementing agents in sandstones include; calcium carbonates, iron oxides, authigenic clays, anhydrite and quartz overgrowth ie. secondary quartz .  This cementation process reduces the porosity and permeability of the original sediment.  Dissolution of cement can occur later in the life of the rock, again changing permeability and porosity.

During the initial cementing process, or at any time in the life of a sanstone, the flow of reactive fluids (formation brines) can cause dissolution and precipitation of various minerals.  Precipitated minerals are termed authigenic (meaning formed in place).  Most clays in sandstones are formed this way.  In fact they are most commonly by products of feldspars’ reaction with brine water under pressure and temperature.  Diagenesis stops when the nonreactive fluids such as hydrocarbons or hydrocarbon gasses fill the permeability of the sandstone.

In general, sandstones of older age are more compacted during diagensis are therefore more consolidated and less permeable.  Younger sandstones (eg. Miocene age) are less consolidated and more permeable.

Sandstones can be organised according to the degree and type of chemical composition, as shown in the sandstone ternary diagram .

Quartzose sandstones are mineralogically simple, containing 90% or more detrital quartz.  Texturally these sandstones are typically even grained becoming sub to well rounded.  Arenite is the term applied to quartz sandstone containing > 95% quartz.  Feldspathic sandstones are more common and are composed of 10-25% feldspar (mainly K-spar).  The cement matrix is commonly quartz overgrowth.

Arkosic sandstones are composed primarily of feldspars and quartz.  Other minerals such as clays and micas generally make up 5 to 15% of the rock.  They are typically less rounded than quartzone sandstones because of the angularity of feldspars.  The matrix cement is usually iron oxides, clay minerals, and fine silts.

Graywackes are sandstones containing between 20 and 75% clay and some ratio of feldspar to quartz.  The cement matrix consists primarily of clays, iron oxides, cherts and micas.

;

# Discussion of Matrix Acidizing.

Matrix acidizing is the process of injecting acid into the formation below fracture pressure in order to remove damage (or skin) and restore the permeability to original reservoir permeability or higher.

In carbonates, where solubility can approach 100%, acid wormholes past damage or it simply enlarges the wellbore.  Thus for incremental increases in acid volume, there is a proportional increase in effective wellbore radius and productivity.  This is not the case for sandstone acidizing.  In the absence of damage, the volume of acid required to improve the immediate wellbore of a sandstone is cost prohibitive.   In fact, without skin damage, sandstones will typically respond to an acid treatment with only a 10-50% increase in productivity, while damaged sands can respond with several folds of increase.  Therefore, sandstone acidizing requires detailed candidate selection along with proper job design.

In matrix acidizing, acid flow is radial and confined to the formation's natural pores and flow channels (Figure 1). The purpose is to remove damage from the permeability of the formation. Because the contact area between the acid and the formation is very large. Matrix acid jobs are therefore conducted at low injection rates (typically 1/4 to 5 bpm).

Figure 1: Matrix Acidizing.

(a)	A matrix acidizing treatment is defined as - slowly injecting acid into the formation so that it penetrates into the pore spaces of the rock without fracturing the formation.

Maximum injection rate can be estimated by the following manipulation of Darcy’s equation for radial flow;

Qimax	=	4.917e6 x kh [gf x H) - (Pf - Pr) 0.9 ]

B ( ln re/rw  + S)

Where Qimax  is the maximum injection rate(BPM); k is permeability (md); h is formation height; Pf  is frac pressure (psi); Pr is reservoir pressure (psi); 0.9 is a safety factor;  is viscosity of fluid (cp); B is formation volume factor (assume = 1); re  is drainage radius (ft); rw is wellbore radius (ft); and S is skin factor (dimensionless) if known.

If possible, treating rate should be increased proportionally to the permeability improvement during the job, thereby keeping a constant downhole injection pressure (See papers by Pacolloni, Economides). This will improve zone coverage and increase acid penetration.  Figure 1-b below shows the micro and maco view of radial flow.  On a “micro” scale, acid is actually moving in flow paths of least resistance within the matrix of the rock - dissolving particles and enlarging pores , but the overall pattern (“macro” scale) of flow is a radial network.

(b)	Acid penetrates the matrix of the formation, moves outward  in radial waves, and dissolves materials that have invaded the rock pores.

## Reaction Rate Parameters.

It is important to understand the physics behind sandstone acidizing as well as the chemical processes.  The reaction rate of hydrofluoric acid within the matrix is governed by the following;

- Surface Area

- Temperature

- HCL % in the HCl:HF blend

- Chemical Composition of the Rock

- Pressure

### Surface Area.

A strong relation exits between the surface area of the rock and the volume of acid injected -versus- reaction rate.  Simply put, as surface area increases - so does acid reaction rate.  This relation accounts for the size of the sand grains, surface area of the matrix material and the porosity of the rock.  The ratio is most sensitive to type and volume of clay in the matrix.  From the table below note that the surface area of Montmorillonite may be 400 sq. mt./gram (depending on degree of swelling) while 20/40 mesh sand is approximately 2 sq. mt./gram.  So on the basis of surface area alone, the rate of reaction would be 200 times faster on clay (per given acid vol.).

Table 2 : Surface Area / Reactivity Analysis

| MINERAL | FORMULA | HF Solubility | HCl Solubility | SURFACE AREA (m2/gm) |
| --- | --- | --- | --- | --- |
| Quartz (20/40m) | Si02 | Low | No | Very Low (2) |
| K-Feldspar | KAISi3O8 | Low - Mod. | Very Low | Low - Mod. (5-29) |
| Montmorillonite | (Ca.5,Na).7(Al,Mg,Fe)4[(Si,Al)8O20](OH)4nH2O | Very High | Low | Very High (80-700) |
| Kaolinite | Al4[Si4O10](OH)8 | High | Low | Low - Mod. (24) |
| Illite | K1-1.5Al4[Si7-6.5Al1-1.5O20](OH)4 | Very High | Low | Very High (100-150) |
| Chlorite | (Mg,Al,Fe)12[(Si,Al)8O20](OH)16 | High | Mod. | High (20-40) |
| Muscovite mica | K2Al4[Si6Al2O20](OH,F)4 | Moderate | Low | Moderate (10-20) |
| Calcite/Dolomite | CaCO3/Ca,Mg(CO3)2 | Very High + | Very High | Varied |

+ Forms precipitate

### Temperature:

Temperature has a marked effect on the reaction rate of hydrofluoric acid with sand and clay. The rate of reaction approximately doubles for each 50° F (28° C) increase in temperature from 100 to 200 o F.

Table 1: 12:3 MudAcid Reaction Rate on Silica -vs- Temp.

| Temperature (F) | Temperature (C) | Reaction Rate (cc / min.) |
| --- | --- | --- |
| 75 | 24 | 3.89 x 10-5 |
| 100 | 38 | 5.59 x 10-5 |
| 150 | 66 | 11.27 x 10-5 |
| 200 | 93 | 20.05 x 10-5 |
| 300 | 149 | 51.46 x 10-5 |

### Acid Concentrations:

The required spending time of HF approximately doubles as the concentration doubles (for a fixed temperature).  For example, a 12.0% : 4.0% solution of hydrofluoric acid reacts twice as long as a 12.0% : 2.0% solution, with clays and silicates.  However, increasing Hcl, does increase the actual reaction rate of HF on silica.

Table 2: Recommended Maximum Acid Strengths at Different  Temperatures.

| Temperature | Maximum HCl Strength | Maximum HCl:HF Strength |
| --- | --- | --- |
| Up to 180° F (82° C) | 15% | 12.0 : 3.0% |
| 180° F to 220° F (82° C to 104 °C) | 10% | 9.0 : 3.0% |
| Above 220° F (104° C) | 7.5% | 7.5 : 1.5% |

### Chemical Composition of the Rock:

Sandstone formations are commonly comprised of quartz, feldspar, and rock fragment grains consolidated by compaction and by chemical cements. The chemical cements may include silica (quartz overgrowths), carbonate minerals, and authigenic or diagenetic clay minerals.  The acid solubility of a typical sandstone formation in Hydrochloric, Acetic or Formic acids is primarily limited by the amount of carbonate material present in the formation.  With the exception of chlorite, the acid-sensitive clay mineral, these three acids have no appreciable reaction with the silicate minerals.  Hydrofluoric acid behaves similarly to these acids in that it reacts with carbonates in the same way as HCl. Hydrofluoric acid also has the ability to dissolve the silicate minerals, including quartz, feldspars, micas, and clay minerals.  Because typical drilling muds contain significant amounts of montmorillonite (bentonite) clay, and because muds are also loaded with drilled and pulverized formation rock solids, (mainly quartz and feldspars) HF acid is the most widely used acid system for removing formation damage from and stimulating sandstone reservoirs that are to be completed in radial flow.

The relative amounts of the different minerals in any given formation will affect the reaction rate. Also affecting reacting rate is the particle size distribution of each different material present. Each material has its own characteristic reaction rate with hydrofluoric acid. For example, hydrofluoric acid reacts with clay at a faster rate  than with quartz, and with calcareous material at a faster rate than with clays.  Furthermore, clays will react at different rates based on both chemistry and surface area. Formations high in porosity filling minerals such as clays and carbonates will react with acids much faster than “clean” sands lacking in reactive clay- and silt-sized particles.  When these high surface area formations are treated with HF acid, only a few inches of penetration is achieved before the acid is completely spent.

Note: The term “silicate” refers to six different classes of minerals which include minerals composed of silica.  Silicates make up 95% of the earth’s crust (60% feldspar, 12% quartz).  Aluminosilicates (feldspars and clays) are by far the most reactive with HF acid.  Other common silicates include; quartz, micas, and chert.

### Pressure:

Surprisingly, reaction rate also increases with pressure, even though most reactions that produce a gas (such as the reaction of silicates with hydrofluoric acid) are retarded by pressure.

The formation of fluosilicic acid (H2SiF6) from evolved gas, silicon tetrafluoride (SiF4), contributes to the overall reaction of the formation, which could explain the increased reaction rate of hydrofluoric acid under pressure.

## Fracture Rates and Penetration.

After the flow channels are enlarged, the materials creating the damage can be removed from the formation with the acid as it is produced back. In treating formation damage such as mud filter cake and scale, care must be taken to treat at less than the fracture pressure of the formation to avoid fracturing past the damaged area.   Table 3 shows the effect of exceeding fracture gradient for various well types.  Note the very poor results when acid is pumped into the formation via a fracture versus through the formation’s pore structure.

Table 3 : Effect of Exceeding Fracture Gradient

| Well Type | Below Frac Press. Jobs % Succ. | Above Frac Press. Jobs % Succ. |
| --- | --- | --- |
| Oil wells | 11 65 | 9 0 |
| Gas wells | 8 88 | 7 14 |
| Injection wells | 6 100 | 0 - |
| Total | 25 80 | 16 6 |

For maximum penetration when matrix acidizing, the acid should have a low viscosity and low surface tension. Gelled and emulsified acids should not be used for matrix acidizing because their viscosity and interfacial tension greatly increase the injection pressures and impede the flow back of the spent acid.

In matrix acidizing, effective stimulation depends upon the presence of skin and the ability of the reservoir to transmit fluids.

# Formation Damage.

(Ref. Understanding Formation Damage, prepared by D. Ross, 1991)

## The Effect of Formation Damage.

Porous media is the more scientific term applied to the common oil field substances known as rocks.  Rocks are materials that are part solid (the matrix) and part void (the pores).  One cubic foot of rock may be made up of 0.75 cubic feet solid and the remainder, or 0.25 cubic feet, is void.  The porosity,  , of the rock is, by definition, the fraction of the rock made up by void space.  In the above case,

Porosity,  = 0.25 of 1 cubic ft. =  25%

Some of the pore spaces are connected to each other, while others are not, much like a bubble in a piece of glass.  The occurrence of connected pore spaces leads to the term permeability.  Permeability, k, is by definition, the ability of a rock to transmit or allow fluid to flow through the rock.  Therefore, if a porous rock has connected voids throughout its length, permeability exists throughout its length.

Most commercial oil and gas reservoirs are limestones, dolomites, and sandstone rocks.  The physical characteristics of these rocks vary.  Sandstones tend to have a uniform permeability and continuous porosity.  This results in a condition of radial flow into the wellbore.  Limestones and dolomites, however, generally have irregular permeability patterns and discontinuous porosity.  The flow of fluids can be primarily through natural fractures.  These fractures can, however, produce high permeable flow channels to form a flow system very much like a radially uniform sand body.  The productivity or injectivity of a well is, therefore, dependent very much on both the permeability and radius of the altered zone immediately surrounding the wellbore.

In order to better understand the effect of formation damage, it is important to review how oil and gas flows through a porous material and how the porous material can become damaged.

### Linear Flow Through Rocks.

Imagine that a piece of rock is cut in the shape of a round cylinder, as shown in Figure 1.  Fluid is forced into the end on the right, flows through toward the left, and out the left end through a cross-sectional end area a.  The force, or energy, that causes the fluid to pass through the core is the difference between the inlet pressure and the outlet pressure (p2 - p1).  Resistance to flow is caused by friction and the viscosity, , of the flowing fluid.

2

Figure 1

The equation that describes this type of fluid flow is known as Darcy's Law, and for incompressible liquid flow is:

v	=	q/a  =  1.127 x 10-3 k (p2 - p1)				(1)

L

Where:

v	=	velocity of flow - ft/sec

q	=	quantity of flow - bbl/day

a	=	cross-sectional area - ft

k	=	permeability - md

	=	viscosity - centipoise

p2	=	inlet pressure - psi

p1	=	outlet pressure - psi

L	=	length of core - feet

This equation may be solved for k/L and is

k / L	=	                q 			(2)

1.127 x 10-3 A (p2 - p1)

Now, if a core has a permeability, k1, throughout its undamaged length, L1, and another permeability, k2, in its damaged length, L2, then:

kavg	=	           L           						 (3)

L1 / k1 + L2 / k2

or

L/ kavg	=	L1 / k1 + L2 / k2							(4)

Where:

L2 / k2 		=	the permeability in the damaged portion.

The significance of this is that if a core of total length, L, is damage-free, fluid will flow through according to Equation 1.  However, if some portion of  its length, L2, is damaged by solids or some other means, then the average permeability, kavg , is altered as shown by Equation 3, and this kavg will be less than the original undamaged k.  Therefore, the flow-rate through the core will be reduced when damage exists over some portion of its length, L2.

Permeability damage will be discussed later in detail, but the preceding section is necessary for an understanding of how it affects the flow of fluids and how one can test for the amount of damage.

### Radial Flow Through Rocks

When a real wellbore penetrates a layer of rock, any flow of fluid from the wellbore to the rock strata, or from the rock strata to the wellbore, is in a radial pattern.  This means that the system resembles an old-fashioned wagon wheel.  Fluid flows out from the hub (wellbore) along lines parallel to the wagon wheel spokes.  The reverse is true if flow is into the wellbore.  Figure 2 shows the radial flow case.

3

Figure 2

If the wellbore has a radius, rw, and the rock radius under consideration is re, and the thickness of the rock is h, then the flow situation is described by the following form of Darcy's Law:

v	= 	q/A  =  q/2rh  =  1.127 x 10-3 k (dp)			(5)									 	    dr

or, if written in oil field units is;

q	=      	7.08 x 10-3 (pe-pw)    					(6)

ln (re / rw)

Where:

a	=	area of flow around well-bore

q	=	flow rate - bbl/day

k	=	permeability - md

h	=	rock thickness - feet

	=	viscosity - centipoise

pe	=	pressure at outer radius - psi

pr	=	pressure at well-bore - psi

re	=	outer radius - feet (reservoir radius)

r1	=	inner radius (damaged zone)

rw	=	well-bore radius - feet

ln	=	natural logarithm

Equation 6 may be solved for (pe - pw) which results in:

(7)

(pe - pw)	=	q   ln (re / rw)

7.08 x 10-3 kh

Now, if a permeability damage zone exists from the wellbore out to a radius r1, as shown in Figure 2, its effect is shown by the pressure drop across both the damaged and undamaged zones.

(pe - pw)	=	(pe - p1) + (p1 - pw)					(8)

Substituting the corresponding quantities from Equation 7 into Equation 8 results in:

q   ln (re / rw)	=	q   ln (re / r1)    +    	q   ln (re / r1)	(9)

7.08 x 10-3 kavg h		7.08 x 10-3 k h		7.08 x 10-3 k1 h

Equation 9 reduces to:

(10)

ln (re / rw)

kavg		=	ln (re / r1)   +	ln (r1 / rw)

k		      k1

Where k1 is the permeability of the damaged or invaded zone and r1 is the damaged radius (equals rw + depth of damage).

Flow capacity (Q/Q0) of a well is the ratio of existing rate versus original rate and can be simplified to the ratio of damaged permeability affect versus original permeability affect.  The ratio is as follows;

Q/Q0	=	kavg ln (re / rw)		=	kavg  / k				(10a)

k ln (re / rw)

Where k is original permeability. This simple formula can be very useful in characterizing damage on a well’s capacity to flow.  The following are examples to illustrate the formula above.

Example-1

What is the flow capacity of a well which has been damaged with mud filtrate to a depth of 2 feet.

k	=	100 md

k1	=	20 md

re	=	330 ft.

rw	=	4.5” (convert to feet), = 0.375

r1	=	rw + 24”, = 4.5 + 24 = 28.5”/12 = 2.375’

Note: Hold formation height, delta P, Volume factor and Viscosity constant.

Solution:

ln (re / rw)

kavg		=	ln (re / r1)   +	ln (r1 / rw)

k		      k1

=	          ln(330/0.375)

ln (330/2.375) + ln (2.375/0.375)

100	             20

=	48 md

Q/Q0		=	kavg / ke

=	48 md / 100 md

=	0.48 or a loss in well capacity of 52%

Skin Damage:

The latter term in equation 10 represents the permeability change in the damaged zone near a wellbore.   One can readily recognize the similarity between Equation 10 and Equation 4.  This is the reason why a permeability damage test on a linear core in the laboratory is related to the actual conditions in a wellbore.  All terms are really flow length divided by permeability terms that represent resistance to fluid flow.

This additional pressure drop from damage zone close to the wellbore is represented by the skin factor S, and is defined by Everdingen as;

pskin		=	      q          	S						(10b)

7.08 x 10-3 kh

Therefore Darcy’s equation for radial flow with the pressence of skin becomes;

q		=	7.08 x 10-3 kh (pe / pw)						(10c)

(ln (re / rw) + S

The effect of formation damage on well productivity is illustrated in Figure 3.  This applies when flow into the well is radial and the well is producing a homogeneous fluid under steady-state condition.  This graph shows that if the region in the immediate vicinity of the wellbore is damaged, a major reduction in productivity occurs.  For example, if the permeability out to four feet is reduced to one-tenth the formation permeability during drilling and completion, well productivity could be reduced to less than one-fourth of the original productivity.

1

Figure 3

## Detection of Formation Damage

### Productivity Index

There are many ways to detect the existence of wellbore damage, one of which is the productivity index of a well.  If the productivity index of a given well is much lower than that of surrounding wells completed in the same zone, it is indicative of formation damage.

Productivity index is defined as the volumetric flow rate divided by the pressure drawdown at the wellbore.  In equation form this is:

(11)

PI	=	q / (pe - pw)

Where:

q	=	flow rate - bbl/day

pe	=	static reservoir fluid pressure out away from the well-bore

pw	=	wellbore bottom hole flowing pressure

If the value of q from Equation 6 is inserted in Equation 11, then:

(12)

PI	=	7.08 x 10-3 kh

[ln (re / rw)]

and this equation may be solved for kh, which is a direct measure of the average flow capacity of a well.

(13)

kh	=	PI    [ln (re / rw)]

7.08 x 10-3

or

(14)

kh	=	q    [ln (re / rw)]

7.08 x 10-3 (pe - pw)

The formation volume factor, ß, is inserted in the last three equations to convert q from stock tank barrels to reservoir barrels per day.

If a PI test is run and the kh is calculated by Equation 14, the value obtained is an estimation of average kh.  ln other words, if damage exists it will be reflected in the calculated flow capacity, kh.  The amount of actual formation damage cannot be determined with Equation 14 and a PI test on a well.

### Pressure Drawdown and Buildup Test

The accepted way to determine the amount of damage in a real well is to run a pressure drawdown and buildup test.  Pressures are measured at the formation while the well is flowed for a given flow time, T.  Pressures are recorded at the same place after the well is shut in for a period of time, .  These pressures are then plotted on semi-log paper with the pressure on the y-axis and  on the x-axis. A straight line normally results if the test was of long enough duration.  The slope of this straight line is labeled “m” and is given by:

m	=	162.6 q  			(15)

kh

Solving Equation 15 for the flow capacity, kh, results in:

kh	=	162.6 q  			(16)

m

This kh is the undamaged flow capacity of the formation out away from the wellbore.  The flow capacity of the formation away from the wellbore should normally be higher than the damaged flow capacity near the wellbore, or the average of both, which is given by Equation 14.  Therefore, if a ratio of these two flow capacities were formed, a measure of the well completion efficiency would result.

### Completion Efficiency

Well Completion Efficiency		=	kh (from eq. 14)

kh (from eq. 16)

(17)

Griffin and Zaki introduced the concept of a damage ratio, which in effect is essentially the reciprocal of the well completion efficiency as defined above.  The simplified form of this is the estimated damage ratio and is:

D.R.	=	1 / F.E.								(18)

Theoretically, one can determine a "skin" damage factor for a well.  However, this value will be some number that cannot be correlated to the amount of actual damage.  Therefore, it is better to calculate a factor given by Equation 17 or 18 to determine if a well has been satisfactorily completed.

A pressure drawdown and buildup test is normally run with a bottom hole pressure bomb in the hole.  Flow times and rates are carefully noted and the pressures are continuously recorded.  Once this information is obtained, the pressures are tabulated opposite the flowing time and shut-in time.  These data points are then plotted as shown in Figure 4.  The slope is the "m", in Equation 17.  With this slope, “m”, and the other data on flow rate, pressures, etc., the well completion efficiency may be calculated.  This is the most direct measure of formation damage.  If the completion efficiency is much less than unity (one), then the well has not been completed in a manner that utilizes all of the natural flow capacity of the formation, ie. it is “damaged”.

4

Figure 4

## Damage Mechanisms

Wells can be damaged throughout their life - during  drilling, completion, production and workover. Depending on the extent and type of damage, acid can be used as a treatment.  HCL:HF acid mixtures have been extensively used to treat sandstone reservoirs, however, their application has been restricted to restoring the natural permeability of the formation. In fact matrix acidizing has been defined as the restoration of original reservoir permeability.  On many occasions the exact type of damage cannot be identified and assumptions must be made.

New wells are frequently damaged by drilling mud solids invasion or drilling mud filtrate invasion.  Mud solids block pore spaces in the very near wellbore of high permeability reservoirs, while filtrate invasion can cause clay swelling, destabilization and migration of clays or scale precipitation.  Filtrate damage can penetrate several feet.  Completion damage may result from use of “dirty” (unfiltered) brines, perforation damage, gravel pack damage etal.  Damage from production mat stem from; emulsions, water block, fines migration, asphaltene/paraffin deposits, scale and wettability changes.  Finally workover damage may result from a combination of the drilling, completion and production mechanisms.  Below is a list of the most common damage mechanisms

- Solids Invasion

- Clay Swelling and Dispersion

- Wettability Changes

- Emulsion Blocks

- Water Blocks

- Bacteria By-products

- Organic Deposition

- Scale Deposition

- Perforation Damage

- Acidizing Damage

- Fines Migration

Table 4 : Types of Damage Associated with Well “Age”

| New Well | Old Well | Injection Well |
| --- | --- | --- |
| Invasion | Scale deposition | Scale deposition |
| a) Mud solids / filtrate | Water / Emulsion block | Clay swelling |
| b) Cement filtrate | Paraffin / Asphaltine |  |
| c) Completion solids / filtrate | Clay Dispersion / migration |  |
| d) Perforation debris |  |  |

## Solids invasion:

One of the primary causes of formation damage is the invasion of solid particles that plug interconnected pore throats, and as a result, substantially reduce the natural permeability of the formation.  The critical size range for solid contamination involves a basic understanding of particle bridging theory.

5

During the 1930's, C.J. Coberly and E. M. Wagner conducted experiments with spherical bodies in order to determine how these objects form stable bridges over openings larger than their own diameters.  The following principles were published in their paper entitled "Some Considerations in the Selection and Installation of a Gravel Pack for Oil Wells".  The predominant form of sand grain packing is hexagonal, and this packing arrangement is the controlling factor in determining the size of openings to be bridged in the sand grains.

D = Diameter of formation sand grain

d = Diameter of inscribed circle (representing  diameter of pore throat)

d = (0.1547)D

D = (6.4641)d

Assuming hexagonal packing, stable bridging of small sand grains over the spaces between large sand grains occurs when the diameter of the circle inscribed in the space between the large grains is approximately two times the diameter of the small grains.

6

Figure 6

Recent research has been done to more clearly define the range of particle sizes that will invade formation sands and plug up the pore spaces.  A. Abrams' SPE paper 5713, "Mud Design to Minimize Rock Impairment Due to Particle Invasion", was presented at the 51st Annual Fall Technical Conference in New Orleans during October, 1976.  Basic conclusions of this work are stated below:

Core permeability studies confirmed that particles with diameter size ranges between 1/3 and 1/7 the size of the pore throat will plug pore channels.  Particles smaller in size than 1/7 the size of the pore throat will migrate freely through the formation matrix.

To understand how such small particles become trapped, it is necessary to realize that oil and gas producing formations are excellent depth filters.  Their many interconnecting pores vary greatly in diameter (which is a function of the particle size distribution of the formation itself) with pore entries and exists usually smaller than the pore spaces themselves.  This structure causes fluids passing through the formation to change direction and velocity frequently while being subjected to many different pressure drops.

The physical characteristics of the matrix and the non-uniform flow or fluid cause the three basic mechanisms of filtration to come into play:

a.	Screening (Pore Openings)

b.	Absorption (High Surface Area)

c.	Sedimentation (Pore Depth)

Particles being transported with fluid flow through the pore spaces may be "screened out" by path restrictions smaller than the particle diameter.  They will form stable bridges if the restrictions are between 2 to 3 times the particle diameter.

Even if particles are small enough to pass through physical pore restrictions, they still have a good chance of becoming trapped by the other filtration mechanisms.  Particles that come in contact with the pore walls may remain there due to ionic forces if the fluid velocity is not high enough to overcome these charges.  If a pore space is deep enough and the fluid velocity slow enough, gravitational forces may be strong enough to cause the particles to "settle" to the bottom of the space.

### Actual Micron Sizes of Critical Particles

If the size of an average pore space opening is known for a specific formation, it is easy to calculate the actual micron sizes of particles that would invade the formation matrix and cause plugging.  For instance, if a formation has an average pore size of 15 microns, the critical size range for contaminants would be 5 to 2.1 microns.

However, since core samples are not always available to determine pore space sizes, a method of estimating pore space size can be helpful.  Harris and Odom's article, "Effective Filtration in Completion and Other Wellbore Operations Can Be Good Investment", in the Sept. 20, 1982 issue of Oil and Gas Journal provides the following rule of thumb for estimating pore space size in the Gulf Coast:

The pore size in microns equals the square root of the permeability in millidarcies.  Example:  For a reservoir where the permeability is 900 md, the pore size would be 30 microns, and the critical plugging size is 10 to 4.2 microns.

Using this rule of thumb, Table 2 shows the different critical plugging sizes associated with various permeabilities.

| PERMEABILITY (MILLIDARCIES) | PORE SIZE (MICRONS) | CRITICAL PLUGGING RANGE - 1/3 to 1/7th (MICRONS) |
| --- | --- | --- |
| 100 250 500 750 1000 1500 2000 | 10.0 15.8 22.4 27.4 31.6 38.7 44.7 | 3.3 to 1.4 5.2 to 2.2 7.4 to 3.2 9.1 to 3.9 10.5 to 4.5 12.9 to 5.5 14.9 to 6.3 |

Table 2

After studying the table, it is apparent that even sands with very high permeabilities are still subject to plugging by low micron-sized contaminants.  This potential for small micron-sized particles to create irreparable formation damage, has forced many companies to establish the following filtration guideline:

Completion, workover and stimulation fluid should be filtered to remove all particles larger than 2 microns in diameter.

### Relationship of Formation Plugging to Well Productivity

Since a direct relationship exists between the pore space and permeability, any reduction in the actual pore space volume will result in a corresponding reduction in permeability.  The effect of particle invasion into a formation matrix is to fill the pore spaces with solid material and, thereby, reduce the total pore volume.  To determine the amount of permeability reduction, it is necessary to consider the effects of mixing large and small size particles together.

### Particle Mixing Theory

Many people erroneously assume that if quantities of two different sized particles are mixed together, the resulting permeability of the mixture would be the weighted average of the two permeabilities.  Derry Sparlin's SPE paper 4772, "Sand and Gravel -- A Study of Their Permeabilities", presents a much different conclusion:

In the actual case of a mixture, the smaller particles tend to fill the void spaces between the larger particles so that the permeability of the mixture is almost always less than the permeability of the smaller particulate matter.

An easy way to illustrate this phenomenon would be to take two identical boxes (A and B) and fill A with golf balls and B with gravel.  The golf balls represent formation sand grains

and the gravel represents contaminant particles 20 to 40 times smaller than the formation sand.

7

The permeability of the box of balls is much greater than the permeability of the box of gravel. When the gravel is mixed with the balls, the pore spaces between the balls become filled.  Now the total resulting pore space in the box is even less than the total pore space in the box of gravel.  Therefore, the permeability of the mixture is even less than the permeability of the box of gravel.

8

## Clay Swelling and Dispersion.

Water sensitive formations are characterized by their reduced permeability when contacted with fluids foreign to the formation. This reduced absolute permeability results from plugging of pore channels by induced or inherent particles.

Sandstone contain many interconnecting pores, with typical pore diameters varying from 10 to 100 microns with up to 3000 pores per square inch of surface. Fluids that move through these pore spaces are subject to changes in direction and flow velocity , and as fluids get nearer the well bore, in a radial flow system, velocities can increase in the smaller pores to where the fluids attain turbulent flow.

In addition to dictating the fluid path and velocity, the pore throats of a sandstone act as a filter for particles carried in a fluid system, and can be highly sensitive to flow rate and pressure differential. Sandstones will rapidly plug when critical flow rates are obtained as particles of clay, feldspars and other minerals are picked up and moved between pore spaces. Where the velocity drops in the next pore space these mobile particles may settle out. However, if several particles meet at a pore restriction having a diameter of between three and seven times the average particle size, the particles are likely to cause a bridge. This bridge may cause partial or complete plugging of the pore and force the fluids to seek an alternative path through the sandstone to the well bore. This causes a reduction in absolute permeability.

Several other factors besides high velocities can be responsible for inducing the mobility of clay particles within a sandstone formation. The disturbance of clay particles can result from changes in the pH of the well fluids and abrupt changes in salinity. These changes can be caused by fluid filtrates, stimulation treatments and water-flood operations. Laboratory techniques such as Scanning Electron Microscopes (SEM), provide a clearer picture of this plugging within the formation pore system by both particles inherent in the formation and damage particles carried into the formation by various fluid filtrates.

## Indigenous Formation Clays.

Most sandstone formations contain a certain percentage of indigenous clays in their mineral composition. These clays can be present as part of the matrix, as coatings on individual sand grains or pore walls, or as individual particles lying in the pores. Clay particles are often found at the junctions of sand grains, and in particular are concentrated near shale lenses. A sandstone containing between 1.0% and 5.0% clay is considered a "clean" sand. A dirty sand would be one containing between 5.0% and 20% clay content.

Carbonate formations are seldom clay bearing, and when clays are present, they are incorporated into the matrix. These clays are not normally affected by invading fluids, nor do they generally interfere with the effective permeability of a carbonate.

As previously mentioned, clays found in sandstones can cause a severe reduction in the permeability of a formation due to their tendency to block or plug interconnecting pore throats. In order to better understand the causes of clay damage, it is important to know what clays are.

Clay minerals originate from the weathering and degradation of igneous rocks. The parent minerals consist of micas, feldspars ((CaO,K20)Al2O3.6SiO2) and ferromagnesium minerals such as hornblende ((Ca,Na2)2 (Mg,Fe,Al)5 (Al,Si)3O22 (OH,F)2).

### Clay Definition.

A clay mineral can be defined as, any number of hydrous alumino-silicate minerals with sheet-like crystals structures, formed by weathering or hydration of other silicates; also, any mineral fragments smaller than 1/256 mm.

There are two main categories that clays can be classified under:

Detrital Clays.

Those clays that are sedimented by a physical process at the earth’s surface by weathering and transportation to a depositional basin.

Authigenic (

Diagenetic) Clays.

Clay sediments that undergo physical and chemical changes (diagenesis) during lithofication and compaction in subsurface formations. Authigenic

clays usually occur as a deposit of clay platelets on the pore walls oriented normally to the grain surfaces. They are also present in thin layers, or partings, in the sand beds.

### Clay Structures.

Clays are composed of thin platelets which, under normal conditions, are orientated in stacks or as discrete packets which are not necessarily ordered in structure and often overlap. The clay platelets consist of various combinations of building units, the silica tetrahedral sheet (Figure 9) and the alumina octahedral sheet (Figure 10). These crystal structures extend indefinitely in two directions, but have a definite thickness range of between 7.0 Å and 20 Å.

Figure 9 : Single Silica Tetrahedron and Sheet Structure.

Figure 10 : Single Octahedral Unit and Sheet Structure

### Common Clays.

The clays most commonly found in sandstone producing formations are:

Montmorillonite (Bentonite).		(Al2-X, MgX)Si4O10(OH)2 Na. H2O

Illite.					K Al2(Al Si3O10)(OH)2

Kaolinite.				Al Si4O10(OH)8

Chlorite.				(Fe, Mg)5 Al(Al Si3O10)(OH)8

Mixed layer clays 			Combinations of above.

(Primarily montmorillonite-illite).

#### Smectite (Montmorillonite, Bentonite).

Smectite is the name for a 2:1 clay mineral group which includes montmorillonite and others. Montmorillonite is an abundant clay found in tertiary rocks though it is less common in deeper mesozoic rocks and is rare below that. Montmorillonite is formed by weathering in warm and semi-arid climates. It has a structure and cation composition that gives it the ability to soak up large quantities of water, which spreads its sheet like layers apart. This tendency is the main reason montmorillonite can be so damaging to formation permeability when it is exposed to aqueous filtrates. Wyoming bentonite consists of about 85% montmorillonite.

Montmorillonite is a three layer lattice structure composed of a central alumina octahedral sheet with a silica tetrahedral sheet above and below. The tips of the tetrahedra are orientated inwardly to contact the octahedra producing a weak chemical bond between the adjacent oxygen molecules. Substitution of metal ions such as iron (Fe2+), or magnesium (Mg2+) for the aluminium (Al3+) in the octahedral sheet creates a charge imbalance of minus two (-2) for three crystal units. This imbalance can be neutralized by the adsorption of calcium (Ca2+), magnesium (Mg2+), hydrogen (H+), potassium (K+) or sodium ions (Na+).

Figure 11 : Montmorillonite Platelet Structure.

The weak bond between the crystals permits water molecules to enter and expand the distance between the clay crystals. In general fresh water and sodium ions tend to swell these clays, but potassium and calcium ions tend to shrink them. The permeability of formations, that contain these clays, will be reduced to nearly zero when contacted by fresh water.

Montmorillonite is the only clay that will swell by absorbing water. Kaolinite, chlorite and illite may be classed as non swelling clays. Generally fresh water tends to swell "hydratable clays" and a higher salinity brine tends to shrink them.

Figure 12 : Clay Structural Expansion.

#### Illite.

Illite and chlorite are the most abundant clay minerals and are generally found in rocks of all ages.  Illite is a diagenetic clay that may be formed from the diagenesis of montmorillonite which was transported to the basin of deposition. Authigenic illite appears as hair like structures or platelets lining pore walls (Figure 13).  Permeability reduction caused by dispersed illite is primarily due to the resultant increase in tortuosity.

Illite has a similar structure to that of montmorillonite, with an aluminium (Al3+) atom being substituted for the silicon (Si3+) atom in the tetrahedral sheet and iron (Fe2+ or Fe3+) and magnesium (Mg2+) for the aluminium (Al3+) in the octahedral sheet. This results in a charge deficit of minus one (-1) per unit crystal and occurs mainly in the outer tetrahedral sheets.

This imbalance is compensated by the absorption of potassium ions which tightly bind the layers of illite crystals together preventing the expansion due to water. When conditions prevail, due to outside influences, that the potassium ions are leached out from the crystal structure, illite can change to a clay that will expand on contact with water.

#### Kaolinite.

On the earth’s surface kaolinite is formed by the weathering of feldspars in typically non-arid environments. In the subsurface, authigenic kaolinite may form by the dissolution of feldspars and reprecipitation of the clay in the pore system. Alternately, if fluid compositions are correct, entire feldspar grains may be diagenetically altered to kaolinite. Water is essential to the chemical reaction that changes feldspar to kaolinite.  Kaolinite is typically present in both young and old rocks in small amounts. In its raw form, kaolinite is commonly used for making pottery.  The main permeability damage caused by kaolinite found in sandstone is due to its tendency to bridge off in pore throats once it has been dispersed and deflocculated.Deflocculation:kaolinite; 	Kaolinite has a two layer structure with one alumina octahedral sheet and one silica tetrahedral sheet. These units are held together by hydrogen bonds which prevent swelling by the absorption of water into the crystal lattice. Kaolinite particles tend to form discrete units (Figure 14).

Figure 13 : Pore Bridging - Illite.

Figure 14 : Discrete Particles - Kaolinite.

Figure 15 : Pore Lining - Chlorite.

#### Chlorite.

Chlorite is also considered a diagenic clay similar to illite; however, the chemical change (metamorphism) that created chlorite is a more severe extension of the typical diagenesis normally associated with sediments after burial. Chlorite tends to found as a coating that lines the inside of pore throats (Figure 15). The dissolution by acid of chlorite, being an iron-bearing mineral, could create the potential for the formation of pore plugging iron hydroxide precipitates.

#### Mixed Layer Clays.

These are composed of layers of different clays. Some mixed layer clays such as chlorite do not swell upon contact with water unless they are degraded. Irregular mixed clays usually contain montmorillonite and illite and thus show marked swelling tendencies. The montmorillonite crystal is relatively weak and may allow the mixed clay to break up and cause severe plugging problems. Some tests show that permeability reduction is the greatest when montmorillonite and mixed layer clays are present.  Reduction is less with illite least with kaolinite and chlorite.

## Damage Mechanisms Associated with Clays

The main causes of permeability reduction associated with clays are due to the following:

- Clay Hydration (Swelling).

- Dispersion.

- Migration.

### Hydration of Clays. (Swelling)

The swelling of clays with water is due to the hydration of the cations attached to the clay. The amount of swelling depends on two factors:

- The cation adsorbed on the clay.

- The amount of salts in the fluid contacting the clay.

There are two types of swelling mechanisms that can occur due to the interaction of clays and filtrate invasion:

- Crystalline swelling.

- Osmotic swelling.

Crystalline swelling (surface hydration) occurs due to the adsorption of layers of water on the basal crystal surfaces of the clay particles, and can cause the clay volume to more than double. If present on pore walls, swelling has little effect on permeability. However, deflocculated and dispersed clay particles are usually trapped in the tortuous flow paths, and there is a sharp drop in permeability.

Osmotic swelling is caused because the concentration of cations is greater between the clay layers than in the bulk solution. Therefore, water is drawn between the layers, thereby increasing the lattice spacing. Osmotic swelling can cause the clay volume to increase by a factor of 20, thus completely blocking all permeability.

Montmorillonite and mineral layer swelling clays containing smectite are the only clays that will swell by absorbing water. Kaolinite, chlorite and illite may be classed as non swelling clays. Generally fresh water tends to swell "hydratable clays" and a higher salinity brine tends to shrink them.

### Dispersion.

At less than 20 g/l of NaCl or equivalent in the interstitial fluid, the lattice spacing of a clay particle expands and becomes "unstable" or dispersed. At salinities below about 1.0 g/l clay particles are deflocculated, as well as dispersed, and are much more mobile than when flocculated and dispersed. In order to understand the relationship between salinity and the damage mechanisms, an explanation of the base ion exchange reactions that take place is required.

Figure 16 : Clay Dispersion (Kaolinite Platelets).

#### Base Cation Exchange Reactions.

All clays have an overall negative charge. In the case of montmorillonite, this negative charge is predominant at the faces of the crystal, whilst a smaller positive charge can be found at the edges.
The density of the negative charge on the faces of the crystals determines the cation exchange capacity of a clay and is greatest for montmorillonite (Figure 17).

Figure 17 : Cation Exchange.

When montmorillonite comes into contact with brine having different salts to the formation water, a cation exchange may take place. When the aluminium atoms in the platelet structure are replaced by other metal atoms, a new negative charge is built up on the faces of the clay mineral surfaces.

Cations in the brine are attracted to the crystal faces by this negative charge. Calcium ions are particularly efficient at replacing sodium ions, and potassium is more effective than sodium for replacing calcium ions (Figure 17). ;;;This exchange of ions may cause the clay to swell due to the size and number of ions exchanged. For each cation that is attached to the clay mineral, there can be associated with it a number of water molecules.

This number, called the hydration number, determines to some extent the effectiveness of the brine. The hydration number for potassium is between 1.0 and 4.0, for sodium is between 5.0 and 11, and for calcium 10 to 20.

The amount of water absorbed by the clay can be controlled by attaching cations with low hydration numbers, such as potassium. This, in general, makes potassium chloride brines more effective for controlling clays.

Where calcium is the exchange cation, the clay will only adsorb a water layer a few molecules thick. When sodium is the exchange cation the clay will adsorb a further layer of water onto the already existing water layer, making it thicker.

When more than one species of cation is present in a clay-brine system, the behavior of the clay is determined by the base exchange reactions. The base exchange equilibrium constants for different cations, favor the adsorption of polyvalent cations over monovalent cations. The following series shows the adsorption preference hierarchy for mono and divalent ions :

H+ = Nh4+ > k+ > Ba2+ > Sr2+ > Ca2+ > Cs+ > Rb+ > Na+ > Li+

Further, when polyvalent cations are in the base exchange positions, the clay aggregates do not disperse, even in distilled water. Consequently, when the base exchange ions on the clays in water-sensitive rocks are predominately polyvalent, permeability impairment by fresh water filtrates is avoided.

For example, the base exchange ions on the clays in berea sandstone are predominately calcium, so that when a dry core is flooded with distilled water, no impairment is observed. If, however, the dry core is first flooded with a sodium chloride brine, the clays are converted to the sodium form, and a subsequent flood with distilled water will plug the core.  But when the sodium chloride brine contains polyvalent cations in a proportion sufficient to keep the clay in the polyvalent form, dispersion is inhibited.  A ratio of one part of calcium chloride or magnesium chloride to ten parts sodium chloride prevented clay blocking in berea sandstone, provided that the salinity was reduced gradually.

Although potassium is monovalent, it is closely held in the clay lattice and thus does not give rise to the high repulsive forces that cause dispersion. Consequently, potassium salts act like calcium salts and inhibit clay blocking. Table 2 lists the minimum salinities required to prevent clay blocking in water sensitive formations containing the four major clay groups.

Table 2 : Minimum Salinities of Brines Required to Prevent Clay Blocking in Water Sensitive Formations.

| Species of Clay Mineral in the Formation | Sodium Chloride Brine (ppm NaCl) | Calcium Chloride Brine (ppm CaCl2) | Potassium Chloride Brine (ppm KCl) |
| --- | --- | --- | --- |
| Montmorillonite | 30,000 | 10,000 | 10,000 |
| Illite, Kaolinite, Chlorite | 10,000 | 1000 | 1000 |

The cations in subsurface formations are in equilibrium with those in the interstitial water. The major constituent of formation brines is normally sodium chloride in concentrations above 20 g/l (20,000 PPM), plus significant amounts of calcium and magnesium salts so that the clays are stabilized on the pore walls. The invasion of filtrate upsets the existing equilibrium and may or may not cause clay blocking, depending on the formulation of the invading fluid.  Table 3 shows how the rate of reduction of salinity affects clay blocking and  how there is less blocking when the salinity of the invading fluid is reduced gradually.

A calcium montmorillonite may swell (deflocculate) when in contact with brine with a low sodium ion concentration. Conversely a sodium clay may shrink (flocculate) when contacted by a brine with high calcium ion content. Lower calcium ion concentration is required to cause shrinkage (flocculation) than with sodium or potassium ions due to its divalent ionic nature.

Potassium montmorillonite will show little swelling with a low concentration potassium chloride brine. Sodium montmorillonite requires a higher concentration of salt with sodium chloride brine to minimize its size than with either potassium chloride brine or calcium chloride brine.

Table 3 : Permeability Reduction of Berea Cores with Continuous Salinity Decrease*.

| Time** | Salinity ppm | Permeability md | Salinity ppm | Permeability md |
| --- | --- | --- | --- | --- |
| 0 | CO = 30,000 | 190 | CO = 30,000 | 190 |
| 1 | 18,200 | 180 | 28,500 | 187 |
| 2 | 11,200 | 175 | 27,100 | 187 |
| 4 | 4,050 | 170 | 24,500 | 188 |
| 8 | 550 | 100 | 20,100 | 188 |
| 10 | 200 | 50 | 18,200 | 187 |
| 20 | 1 | 25 | 11,000 | 186 |
| 40 |  |  | 4,050 | 183 |
| 60 |  |  | 1,500 | 180 |
| 80 |  |  | 550 | 180 |
| 100 |  |  | 200 | 179 |
| 150 |  |  | 17 | 178 |
| 210 |  |  | 1 | 177 |

*From Maungan.  Copyright 1965 by SPE-AIME	**Injection Rate = 1 Pore Volume/HR = 120 cc/Hr

#### Effect of pH.

Clay dispersion is influenced by pH because it affects the base exchange equilibrium, but its effect on a particular system depends on the electro-chemical conditions in that system. However, generally clay dispersion is detrimentally affected by alkaline waters with a pH of greater than 7.0 making the clays more mobile. At pH of 4.0 no disturbance is seen.

The pH of the filtrate may be the cause of impairment by another mechanism if the matrix cement is amorphous silica. Filtrates with a very high pH dissolve the silica, releasing fine particles, which may then block pores. Once a clay is dispersed its particles become free to move and may cause plugging of the pore throats.

#### Electrostatic Double Layer.

Particles in colloidal suspension carry a surface charge. This charge attracts ions of the opposite sign, which are called counter ions. This combination is called the electrostatic double layer. The distribution of ions in the double layer results in a potential grading from a maximum at the particle surface to zero in the bulk solution. The zeta potential is maximum, and the mobile layer is most diffused when the bulk solution is pure water. Electrolytes in a solution compress the diffuse layer and reduce the zeta potential. If low valence ions are replaced with higher valence ones through base exchange, the zeta potential is greatly decreased.  (Ratio 1 to 10 to 500 for more valent, divalent and trivalent cations, respectively). The zeta potential is also reduced by the adsorption of certain long-chain organic cations. It is possible to neutralize and reverse the zeta potential. The potential difference between the surface of a particle and the bulk solution is known as the Nernst potential.  This potential is independent of the electrolytes in solution.

### Clay Migration.

All clay types are capable of migrating when contacted by waters which upset the ionic balance within the formation. Montmorillonite and mixed layer clays have increased probability of migrating due to swelling and water retention. Figure 18 illustrates clay particles in a balanced system, where the clays are in a stable unexpanded (flocculated) condition with formation water. Figure 19 illustrates clay particles in a fresh water system where they have an unstable, expanded (deflocculated) condition. It should be remembered however that high flow rates alone can be sufficient to cause particle migration.

The effect of aqueous fluids on clays depends primarily on the following factors:

- The chemical structure of the clays.

- The difference between the composition of the native formation fluid and injected fluid.

- The arrangement of the clays on the matrix or in the pores.

- The way in which the clays are cemented to the matrix.

- The abundance of clays that are present.

Figure 18 : Flocculated and Unexpanded Clays.

Figure 19 : Deflocculated and Expanded Clays.

The movement of particles within a pore system is affected by the wettability of the formation, by the fluid phases present in the pore spaces and the flow rate through the pore spaces. Under normal circumstances an oil bearing zone contains both oil and water within the pore spaces. Where the formation is water-wet, water is in contact with the mineral surfaces, and oil flows through the center of the pore space. (Figure 20).

Figure 20 : Oil Flow Through Sandstones.

Where clays and other fines are water-wet these particles are attracted to and immersed in the envelope of water surrounding the sandstone particles (Figure 21). In this case the clay particles will only move with the flow of water, and where the water saturation is low these particles are unlikely to cause problems with being mobile.

Figure 21 : Water-Wet Clay Particles.

If the clay particles become oil-wet or partially oil-wet, due to some outside influence, the fines and clay particles are attracted to and immersed in the oil phase. The particles then tend to move with the oil and the resultant plugging of pore throats can be quite severe. (Figure 22).

Figure 22 : Pore Blocking by Oil-Wet Clay Particles.

Following work-over and completions operations, the near well bore will often be saturated with a water or brine based filtrate. As this filtrate is produced back, essentially single phase flow will occur in the pore spaces near the well bore, and particles that under normal conditions might not be free to move, may be carried back towards the well bore. Also present will be a high concentration of particles that entered the formation along with the filtrate. If the return production rate is high the risk of plugging the near well bore pore spaces with these particles is increased.

Work by Krueger et al, has shown that permeability reduction due to particle bridging at pore throats is dependent upon flow rate. Their work showed that at high flow rates, particles that are normally random in their dispersion, tend to interfere with one another as they approach pore constrictions and form bridges. Once this bridge has formed it is very difficult to remove even where back flow is attempted.

At low flow rates the particles align themselves so that they can pass one by one through the restriction without bridging, or are displaced into the sandstone grain water envelope out of the main flow stream (Figure 21).

Thus it can be seen that when clays or other fine particles are present, formation damage to permeability can occur. The degree of damage caused will be dependent upon the type and concentration of clays or other particles present in the pore spaces, their relative position in the pore spaces, the degree of change in the ionic environment, and the flow rate through the pore spaces.

## Prevention of Clay Damage.

Once clay particles have become mobile and disturbed, the resulting plugging and loss of permeability is almost impossible to correct. In this respect it is better to prevent damage due to clay than to try to remove it by remedial action. To reduce the effect of plugging by particles in the formation the following points should be observed.

- Minimize the entry of fluids foreign particles to the formation during drilling completions and work-overs.

- Maintain hydrostatic of well bore fluid as near balanced as possible or underbalanced where feasible.

- Ensure all surface equipment and tubulars are cleaned thoroughly.

- Adjust the cation content of the fluids to prevent the disturbance of formation clays.

- Filter all fluids through two micron filters at surface.

- Use sized carbonate or sized salt fluid loss additives for ease of removal.

- Leave fines and formation surfaces water-wet by the use of surfactants.

- Initiate production slowly.

X-ray diffraction tests can indicate the amount, type and position of clays present in a given sandstone. From these tests, indications of what measures need to be taken to avoid formation damage can be surmised. The use of an SEM test can also help determine the effect of certain actions upon the clays.

In a virgin formation the clay particles are usually at equilibrium with the type and concentration of salts in the formation water. Thus where possible it is beneficial to use clean filtered formation brine as the work-over fluid. If clean formation brine is not available, it is necessary to use an economic synthesized brine as a substitute. In many cases a 2.0% KCl (potassium chloride) brine is capable of preventing clay swelling and disturbance whilst being economic.

Clays can also be "cemented" in place by the use of polymers. The migration process can be effectively controlled by the use of the hydroxy aluminium ion. This large cation replaces all other cations at the surface of the clay and forms a protective polymer coating. This inorganic coating then binds the clay particles together. This polymer is stable in brines but will be removed by contact with acids.

Multi-nuclei organic polymers behave in a similar way to the hydroxy aluminium ion clay inhibitors, with the additional advantage that they are not sensitive to contact with HCl acid, and are stable for at least an hour in contact with standard 12:3% HCl:HF acid. These polymers can be used in both aqueous and alcohol based fluids, hydrochloric acid, sea water and brines. These additives have the benefit that they leave the sand and clay particles water wet.

### Clay Stabilizers.

In acid treatments the clay particles may be released when the treating solution dissolves the surrounding matrix. In water-flooding the salinity of the injection water may differ sharply from that of the formation brine, causing the clay to flocculate, or coalesce into small clusters.

The same is true for completion and work-over fluids. In each case the clay particles become entrained in the moving fluids and plug the flow channels of the rock, reducing its permeability.

Liquid organic polymers (Clay Master-5C) are designed for treating water sensitive formations to control production problems caused by the swelling, dispersion and migration of clay minerals. The additive strongly adsorbs on to the clay minerals surfaces to form a shield on the clay surfaces, thus preventing the break-up of the clay particles.

BJ Services Clatrol (Clay Master) additives can be used for control of these clays. For example Clay Master-5C is a liquid additive soluble in both water and acid, that controls the dispersion of clay particles. Results from laboratory tests have shown that fluids containing Clatrol additives can be used with only minor changes in permeability even where there is abruptly increasing salinity.

Clatrol additives are also beneficial in preventing the formation of emulsions involving crude oil, brines and acid. They can also be used as demulsifiers, but are generally not as effective as normal NE-additives (surfactants) and may require the use additional additives.

Clatrol additives can be used in gas and water producing, or injection wells to prevent clay dispersion. In acid treatments, the Clatrol additive should be used be used in a brine solution ahead or behind the acid treatment. Normally the "Clatrol" flush should be at least half of the volume of the acid treatment. Clatrol additives can also be used in the acid treatment itself, but the benefits will not be as good as when it is used in the preflush or post-flush.

Clatrol additives can also be used in fracturing treatments, where they will minimize formation damage caused by clay dispersion and will lower surface tension, allowing the fracturing fluid to water wet the formation.

Normal concentrations of Clatrol additives would be between 1.0 and 6.0 gallons per 1000 gallons of fluid. Concentrations will depend greatly on the application, type of well completion, formation properties and mineralogy, and treating fluids.

#### Potassium chloride.

Ideally the most effective fluid for treating a water sensitive formation is one which has established equilibrium with the formation clays in an unswollen state. The formations interstitial brine is such a fluid. However formation brine is not always available in large enough quantities, and to synthesize it usually prove uneconomical. Therefore an artificial brine must be prepared to meet the following criteria:

- Will prevent the hydration and swelling of formation clays.

- Will be economical to prepare and apply.

- Can be used for treating most formations.

- Will be compatible with chemicals normally used in other well fluids.

In tests conducted, a 2.0% solution of potassium chloride (KCl) could be passed through formation samples containing clays, at a higher rate than other solutions containing up to 5.0% calcium chloride, or up to 10% sodium chloride.

An advantage to using potassium chloride brines is their compatibility with other fluids. Potassium Chloride brines will not form harmful scales when in contact with formation brines, where as calcium chloride brine  will form calcite in contact with bicarbonates and gypsum with sulphate ions. Another advantage is that a smaller percentage of potassium chloride is required, thus reducing the mixing time to produce a true solution and normally reducing the cost in terms of chemical requirements

Potassium chloride finds its greatest use in the preparation of fluids for fracturing water sensitive zones. Good final permeability around the fracture is vital for obtaining the desired productivity increases; and potassium chloride is used to protect the formation adjacent to the fractures from fluid that leaks off during the treatment and flow-back operations.

Potassium chloride is also used as the carrier fluid in the make up of solutions for breaking emulsions or water blocks, for combating bacteria and for protecting production zones during work-over operations.

#### Liquid KCl (Potassium Chloride Substitute).

The primary application of liquid KCl (Clay Treat 3C) is a substitute for KCl in aqueous fracturing treatments. In recent years, the development of continuous mix fracturing systems has resulted in the need for a liquid KCl substitute that can be injected on the fly at the mixing unit (hydration unit or blender) during the treatment. This eliminates the time consuming operation required for premixing and the storage requirements. Since the remaining water on the job site is fresh water it can be disposed of more easily than salt water. It should be noted that liquid KCl is a temporary clay stabilization additive for stimulation treatments and should not be used as a permanent protection additive.

The normal recommended concentration usually ranges from 1.0  to 4.0 gallons per thousand gallons depending on the formation characteristics and properties. The usage range is sufficient for most formations with less than 5.0 percent water sensitive clays. In the case of formations with 5.0 percent or more water sensitive clays, the concentration of liquid KCl should be determined through laboratory testing or a Clatrol additive should be added.

#### Ammonium Chloride.

Ammonium chloride is occasionally used as a clay stabilizing solution where it has shown to be more effective than potassium chloride. This product is often used in conjunction with acid treatments as a preflush or post-flush, particularly with hydrofluoric acid where it is important to maintain the reaction by-products in solution.

#### Potassium Hydroxide Treatments.

Treatment of clays using potassium hydroxide has been described by Marathon in the August 1984 issue of the JPT to render clays insensitive to fresh water. This treatment entails injecting a 15% to 20% solution of potassium hydroxide into the clay bearing formation, where it reacts chemically with the clays to change their structure. This process cannot be used where calcium chloride is present either in the work-over fluid or the formation brine as calcium hydroxide would be precipitated which could cause severe damage to the formation.

### Fines Stabilization.

Migration of non-swelling (kaolinite, fibrous illite) clay and non clay siliceous fines can be controlled by the use of an organosilane compound (FSA-1). The organosilane reacts with fines in the formation and then bonds them to the formation face. The compound is most effective with HCl-HF acid mixtures, where potential for mobile siliceous fines are the greatest because of the potentially damaging effects excessive mineral dissolution caused by HF.

This compound can be added directly to acid mixtures (HCl, HCl-HF, Sandstone Acid™), and any preflush or displacement fluids. Normal concentrations range from  one to ten (1.0 to 10) gallons per thousand gallons of treatment fluid. The best results obtained are with concentrations in the range five to ten (5.0 to 10) gallons per thousand gallons range.

A spacer of KCl (not in conjunction with HCl-HF) or NH4Cl brine should be used to separate the treatment fluid from xylene or other solvents used for hydrocarbon dispersion during a clean up of the reservoir.

The organosilane material does not protect against clay swelling, and a stabilizer should be used in conjunction with this material to prevent swelling of clays.

Table 4 : Clay Stabilizing Agents.

| Stabilizing Agent | Product Type | Normal Usage |
| --- | --- | --- |
| Clatrol-3 (^) | Quaternary Alkanol Amines | 0.1% - 1.0% |
| Clay Master-FSC (^) | Full Quaternary Amine | 0.05% - 0.50% |
| Clay Master-5C (^) | Full Quaternary Amine | 0.1% - 0.5% |
| Clay Treat-3C | Mono Amine Salt | 0.1% - 0.4% |
| FSA-1 (^) | Organosilane | 0.1% - 1.0% |
| Potassium Chloride | Inorganic Salt | 1.0% - 3.0% |
| Ammonium Chloride | Inorganic Salt | 2.0% - 3.0% |
| Potassium Hydroxide * | Inorganic Base | 15% - 20% |

^ Permanent stabilizer, others are temporary.

* Not for use in acid solutions, only on its own.

### Removal of Particle Blocks.

As a rule it is desirable to maintain formation clays in equilibrium in the reservoir. In general, invasion of particles from drilling mud, dispersion, flocculation, breaking loose and movement of clays within the formation cause more damage to wells than the swelling of the in-situ clays.

Careful analysis of the problem should be made to ensure the correct treatment is applied.

### Dispersion of Clays.

Dispersion of clays is a frequent cause of formation damage:

- Anionic surfactants will disperse clays in acid solutions.

- High pH fluids tend to disperse clays.

If surfactants are used to disperse clays in mud plugged perforations, then high concentrations of the surfactants may also disperse clays in the formation.

### Flocculation of Clays.

Clay flocculation may reduce or increase the formation damage:

- Specific nonionic surfactants may be used to flocculate clays.

- Acid and other low pH fluids tend to flocculate clays.

In the case where blocks are caused by clay dispersion a non-ionic surfactant would be used to flocculate the clays and reduce the block.

Clays and other fine particles, when mobile, help to stabilize emulsions formed during work-overs and stimulation treatments. Anionic surfactants (demulsifiers and non emulsifiers) are used to break or prevent the formation of these emulsions by water-wetting the surfaces of the particles and removing them from the oil-water interface of the emulsion.

Oil-wetting clays with cationic surfactants greatly increases the size of the clay particles and increases the blocking caused by clays. These surfactants are extremely difficult to remove from sandstones and clays, and their use in injection or circulation fluids should be avoided in sandstone wells. Mutual Solvents used as a preflush with treatment fluids will usually reduce cationic adsorption on clays and sands.

### Hydrochloric Acid on Clays.

Hydrated sodium clays may be reduced in size by an HCl acid treatment. When these clays react with acid, the sodium ions on the surface of the clay platelets are replaced with hydrogen ions from the acid by ion exchange. The resulting hydrated hydrogen clay is smaller than the original hydrated sodium clay, thus this tends to increase formation permeability. However HCl may cause the flocculation of the clay, thus causing damage by the release of fines. In general anionic surfactants are used when acidizing sandstones with HCl to prevent clay flocculation.

### Hydrofluoric Acid on Clays.

Hydrofluoric acid dissolves feldspars, naturally occurring clays and clays that have invaded from drilling mud. It also reacts slowly with formation sands. Acid treatments may release some fines such as undissolved clays. These fines are usually in the order of two microns in diameter which under normal circumstances will be removed from the formation with the treating fluid particularly where a fines suspending surfactant is used. However, any fines created can plate out on clay bridges already formed and further reduce the permeability of the formation. Another problem  with fines is their ability  to stabilize emulsions, particularly when they are oil wet, this needs to be controlled by the use of surfactants. Removal of all the cementaceous material (carbonates and silica) between the sand grains will result in the sand becoming unconsolidated.

In general hydrofluoric acid treatments should be pumped using mutual solvents such as EGMBE (US-2, US-40) to reduce surface tension and improve the effectiveness of surfactants and corrosion inhibitors.

- Mutual solvents effectively prevent the adsorption of cationic surfactants and corrosion inhibitors on sand and clay.

- Mutual solvents should be used with acid treatments where cationic emulsion breakers are used.

- Some anionic emulsion breakers do not adsorb on sand and clay, therefore when acids contain these surfactants mutual solvents are not always required.

- With hydrofluoric acid treatments using cationic surfactants mutual solvents should be used in both the preflush and the main acid treatment.

Table 5 : Treatment Options for Damage Caused by Indigenous Minerals.

| Mineral | Potential Problems | What to Avoid | General Treatment Options |
| --- | --- | --- | --- |
| Kaolinite | Mobile Fines | High flow rates. | - Rate Control - <5% - Acidize - standard HCl/HCl:HF - >5% - Evaluate for stress pack. |
| Illite | Mushing | Fresh water systems. | - Permeability >120 md Acidize - standard HCl/HCl:HF - Permeability <120 md Acidize - standard HCl/HCl:HF |
| Smectite (Montmorill-onite) | Swelling | Fresh Water Systems. | - Acidize - standard HCl/HCl:HF - Perform immersion tests to see if alcohol/acid blends or other additives are necessary to prevent swelling. |
| Chlorite | Iron Hydroxide precipitate, Hydrous Silicate, Amorphous alumino silicate scale. | Oxygen rich systems; pH > 2.8. Inadequate sequestrant.. Buffered acid Systems. | - Preflush - 25 gal/ft 10% Acetic - Acidize - 50 gal/ft 7.5% HCl + 100 ppt NTA (FE-810)+ 20 ppt Erythorbate (FE-210)+1.0 ppt X-14. Mud acid - 100 gal/ft 7.5:1.5 HCl:HF +25 ppt NTA - Overflush as required. |
| Mixed layer Illite/ Smectite | Swelling | Fresh water systems. | - Acidize - standard HCl/HCl:HF |
| Feldspars | Silica precipitation. | High % of Hydrofluoric Acid. | - >20% - 7.5% HCl, 7.5:1.5% HCl:HF - <20% - Standard HCl/HCl:HF |
| Chamosite | Iron hydroxide precipitate. | Oxygen rich systems; pH > 2.8 | - Preflush - 25 gal/ft 10% Acetic - Acidize - standard HCl/HCl:HF - Chelate as required. |
| Formation Fines (Quartz, Feldspar Chert etc.) | Mobile Fines | High flow rates | - Preflush - 7.5% HCl - Acidize - 7.5:1.5% HCl:HF . Volume as required. - Overflush as required |
| Calcite, Dolomite | Calcium fluoride precipitate. | Contact of Calcium ion with HF Acid. | - >18% - HCl only. - <18% - Standard HCl/HCl:HF |
| Siderite, Hematite | Iron hydroxide precipitate. | Oxygen rich systems. | - Acidize - standard HCl/HCl:HF Chelate as required. |
| Pyrite | Iron Hydroxide precipitate. Asphaltene sludging. | Oxygen rich systems. | - Preflush - 25 gal/ft Xylene + 10% Acetic - Acidize - Standard HCl/HCl:HF Chelate as required Anti-sludge agent should be added to HCl preflush. |

## Wettability Changes

23

When two immiscible fluids such as oil and water are together in contact with a rock surface, one of the fluids will preferentially adhere to the rock surface more than the other.  The term wettability refers to a measure of which fluid preferentially adheres to the surface.  Most producing reservoirs generally exist in a water-wet state, that is to say that connate water preferentially adheres to the rock surfaces.  The figure below illustrates the condition that exists on a rock surface.  The angle  measured through the water is called the contact angle.  Wettability is described by the contact angle.  If the contact angle  is < 90, then the rock surface is said to be water wet.  On the other hand, if  is > 90, the rock surface is said to be oil-wet.

## Water Blocks

(Ref. Workover Fluids by Bob Hall, World Oil Sept. 1986)

Water blocks and/or condensate blocks are frequently encountered in gas and retrograde gas wells.  Thes blocks can be caused by a number of operations including;

- Pressure drawdown in a retrograde well

- Clean filtered brine kill fluids

- Stimulation fluids

- Shut-in where fluid is allowed to fall back into perfs.

- Normal reservoir pressure decline

- Crossflow

- Water conning

A typical scenario for this damage may be that, after a gas well has produced for a period of time, the formation around the perforations becomes very dehydrated.  This can result from the rapid expansion of gas and the flash distillation of liquids in this area of maximum pressure drop.  This mechanism can also result in the formation of salt deposits around the perforations.  When this dehydrated formation is contacted by fluid, formation minerals, especially clays and salts, act like a sponge to strongly imbibe water.  A block can then result when the maximum differential pressure that can be achieved across the perforations is not sufficient to overcome capillary forces in the pore channels.  As surface tension of a fluid is increased, contact angle is decreased or capillary radius is decreased, the differential pressure required for fluid to flow down a pore channel will increase.  To minimize fluid blocks, surface tension should be minimized and contact angle maximized for that fluid in the formation.  If a fluid block is present in a well, several techniques can be attempted to remedy the problem.  Alcohols often have been used to remove fluid blocks.  When IPA and methyl alcohols are diluted with water, surface tension of the mixture increases significantly.  This increase can increase the differential pressure required to remove the fluid block.  Thus, preferred alcohol systems also include a surfactant to maintain low surface tension.  Surfactant concentrations as low as 0.1% by volume can reduce surface tension of water to about 30 dynes/cm.  The selection of surfactant should be goverened by formation type, and reservoir fluids.  In a sandstone, anionic or nonionic surfactans should be used.

For fluid block removal in a matrix application, the preferred treating procedure is to:

- Clean tubulars of scales, parrafins, asphaltenes and other solids.

- Establish injection with a gas.  eg. N2.

- Inject 25 gpf either methanol or IPA containing 1% surfactant.

- Displace tubulars with gas.

- Return well to production slowly.

For intervals over 25 feet, divert with foamed alcohol or foamed ammonium chloride solution.

## Emulsion Blocks

Emulsions can occur within a formation between two immiscible fluids such as oil and water.  In order to understand the damaging effects that emulsions can create, it is important to understand what emulsions are.

An emulsion is defined as a system that consists of a liquid dispersed in another immiscible liquid usually in droplets of larger than colloidal size.

Energy is required to make an emulsion, and stabilizers must collect at the interface between the fluids to keep the emulsion from breaking.  The most significant stabilizers are: fine particles of clay or other materials, asphaltenes, and surfactants.  Depending on the effects which occur at the interface between the two fluids, some emulsions can become very stable and devastating to productivity.  In extreme cases, emulsions can in fact kill a well.  It should be noted that it is very rare for indigenous oil and formation water to create an emulsion block.

24

When discussing emulsions, a water-in-oil emulsion, sometimes called an oil external emulsion is the type that will be dealt with 90% of the time.  The water-in-oil emulsion has a continuous oil phase and a dispersed aqueous phase.  The water phase consists of many small droplets which vary in size depending on the tightness of the emulsion.  Some droplets have a solid nucleus and others do not.  Figure 26 (above) illustrates the two types of emulsions that can occur.

Field evidence of an emulsion is; low PI on initial completion, especially after using oil based mud; low PI after using a surfactant laden workover fluid.  Diagnosis is often difficult, as emulsions at surface do not necessarily mean that they are occuring downhole.  Emulsions can be treated with a diesel or brine containing a properly tested and designed surfactant or 25 to 50 gpf of EGMBE at 10% concentration.

## Acidizing Damage.

Formation damage will almost always occur during an acid job, therefore, it is necessary to make certain that the stimulation benefits outweigh the negative effects of the created or existing damage, for the job to be successful. When designing an acid job, the engineer must be aware the various types of damage that can occur and take the necessary steps to prevent them. The most common types of formation damage caused during acidizing are as follows :

- Formation de-consolidation.

- Fines mobilisation.

- Reaction by-products.

- Chemical incompatibilities.

- Precipitation of iron compounds.

- Emulsions and sludges.

### Formation De-Consolidation.

The problems and damage resulting from formation de-consolidation can be very severe. Reduced permeability due to the mobilisation of the formation material can shut off production itself. Formation sand flowing into the wellbore causes numerous problems with the production equipment that can be extremely costly.

The potential for damage due to de-consolidation depends upon the formation geomorphology, acid strength and acid volume pumped. For example, if the formation consists of 10.0 % carbonate material and this material is cementing the sand grains together, it is undesirable to dissolve all of this material with acid. Reduced HCl volumes and strengths should be used in this situation when pre-flushing ahead of mud acids.

In the case where sand grains are cemented together with clay material, reduced mud acid strengths and volumes should be considered. Full knowledge of the formation mineralogy and geomorphology is essential.

### Fines Mobilisation.

Release and mobilisation of clay and other silicate materials can severely damage a well’s productivity. Since a given volume of strong acid dissolves more formation than the same volume of weaker acid, a greater volume of insoluble fines may be released by the reaction. As the well is put back on production, the fines that have been released can migrate and bridge in the near wellbore area. Released fines can also act to stabilise emulsions. The use of non-ionic, anionic surfactants, or mutual solvents such as INFLO-40 (EGMBE) will help to water-wet these fines, thus preventing emulsion stabilisation. More importantly, if the fines become oil-wet, the ability for these fines to migrate is increased. Consideration should also be given to the use of suspending agents (MMR Acid), fines stabilising agents (FSA-1), or BJ Sandstone Acid.

### Reaction By-Products.

Matrix acidizing is sometimes defined as; “Pumping an acid into a formation in hopes of creating more stimulation than damage”.   This is and indication of the scope of damaging precipitation caused by acidizing sandstones, particulary with HF.

Because fluorine is a very reactive element and because the composition of sandstone is varied, many reaction products are formed when sandstone formations are stimulated with hydrofluoric acid. For example calcium bifluoride (CaF2) is formed when hydrofluoric acid reacts with calcium carbonate (CaCO3).

Sodium and potassium ions that may be present in the formation water can react with spent hydrofluoric acid to form insoluble precipitates, such as sodium or potassium hexafluosilicate (Na2SiF6 or K2SiF6).

There are many reaction by-products that can form when certain minerals are dissolved by hydrofluoric acid. The amount of precipitates actually formed is highly dependent upon the bottom hole static temperature and the acid contact time in the formation.

The most well known precipitate is calcium bifluoride which is a by-product formed by the reaction of live hydrofluoric acid with calcium carbonate material found in the sandstone matrix.

2HF + CaCO3  CaF2 + CO2  + H2O

Hydrofluoric acid reacting with clay materials, can form by-products such as fluosilicic acid (H2SiF6) and fluoaluminic acid (H3AlF6). These acids will react further with carbonate materials to form hydrated silica and aluminium respectively. These hydrated compounds will precipitate out of solution in volumes that are much greater than the volume of the of the original formation material dissolved, thus significantly increasing the damage potential.

HF + Clay 		 	H2SiF6 + H3AlF6

H2SiF2 + CaCO3 	 	Calcium difluoride

H3AlF6 + CaCO3 	 	Hydrated Aluminium

The Feldspar content of a sandstone is often overlooked. Hydrofluoric acid reacting with potassium feldspar will form a precipitating by-product of potassium hexafluosilicate. Sodium hexafluosilicate will also precipitate where a high concentration of hydrofluoric acid is reacting with sodium feldspar.

6HF + K-Feldspar	 	K2SiF6

6HF + Na-Feldspar	 	Na2SiF6

Amorphous hydrous silica will always be a by-product from the reaction of  hydrofluoric acid with sandstone formations. This cannot be prevented, but the damage caused by it and other precipitates can be minimised with a well designed and executed acid job.

4HF + SiO2 		 	SiF4 + 2H2O

### Reaction By-Products Prevention.

A preflush of hydrochloric acid should always be used ahead of the hydrofluoric acid treating solution to displace the formation water. Brines and sea water should never be used to prepare this treating fluid, as the metal ions present in solution, can react to form precipitates with hydrofluoric acid.

The use of HCl as a preflush serves three purposes:

- It removes calcium and magnesium carbonates.

- It minimises the loss of the hydrofluoric acid used in the second phase of a treatment.

- It serves as a spacer between the HCl:HF acid and the formation brine.

Other preflushes and  which can be used to provide a barrier include:

- Ammonium Chloride (2.0% to 4.0%). Also acts as a chemical buffer to prevent formation of precipitates.

- Diesel oil .

- Clean Lease oil.

- CO2

A post-flush of 3.0% to 7.5% HCl should be used as a spacer to separate the HCl:HF acid from the displacement fluid. This post-flush will prevent the formation of precipitates, by preventing brine from mixing with the HCl:HF acid during displacement, and act to maintain a low pH as the spent acid is produced back. Sufficient volume should be used to displace the HCl:HF acid out into the formation and reduce the risk of damage from precipitation in the near wellbore area. Other post-flushes include:

- Ammonium Chloride (2.0% to 4.0%). Also acts as a chemical buffer to prevent formation of precipitates.

- Diesel oil.

- Clean Lease oil.

Shut in times should be kept to a minimum to reduce the possibility of precipitation of reaction by-products that might reduce the effectiveness of the stimulation treatment.

When calcium sulphate in the form of anhydrite (CaSO4) or gypsum (CaSO4.2H2O) is present in the formation, the problem of re-precipitation may arise. This occurs because this sulphate is less soluble in spent acid than in live acid.  Since calcium sulphate has a maximum solubility in hydrochloric acid within the range of 8.0 % to 12.0 %, its re-precipitation can be minimised by the use of high-strength treating solutions.

Strong acids, when spent, will have a higher concentration of dissolved reaction by-products than a weaker acid. The solubility of other salts is usually lowered in strong spent acid. If formation water with a high sodium chloride content is encountered, precipitation of the salt may occur.

The following are general guidelines that can be used to minimise the damage associated with precipitates formed as reaction by-products with hydrofluoric acid :

- Use chelating and sequestering agents based on the mineralogical properties of the formation.

- Determine the hydrochloric acid (HCl) preflush and Hydrofluoric Acid (HCl:HF) volumes from formation solubility analysis, permeability and porosity.

- Overflush the acid treatment to four or five feet away from the wellbore.

- Minimise acid contact times to four  hours.

- Acid strengths should be determined from the bottom hole static temperature.

### Additive Incompatibility / Solubility / Dispersibility.

When choosing the additives to be included in an acid treatment, one must always consider the compatibility of the additives with each other and with the acid. Solubility, dispersibility and chemical compatibility should be checked in the laboratory prior to pumping the acid system into the well.

Some additives are soluble in acid at low concentrations only. At high concentrations, these additives will flocculate out. If the additive is oil soluble, it will float to the top.

If an oil soluble additive is to be used, a dispersant must be added to the acid system. A good example of this is the use of xylene in acid. Without a dispersant, the xylene would simply float to the top of the acid. The dispersant must be strong enough to keep the xylene dispersed for a sufficient time for the acid to be pumped into the formation.

### Chemical Compatibility.

The most common problem encountered here is the mixing of an anionic additive with a cationic additive. This can sometimes cause a precipitate to form. At times it is necessary to mix cationics with anionics, for example, retarded acids often use anionic surfactants to emulsify the acid, whilst most corrosion inhibitors are cationic. The same applies to anti-sludging agents. In these cases consideration should be given to the use of pre-flushes containing diesel or solvent to contain one of the additives.

### Iron Compounds.

Precipitation of iron hydroxides can severely damage a well. There are two forms of iron hydroxide that need to be considered, ferrous (Fe2+) and ferric (Fe3+).

Ferrous hydroxide will precipitate out at a pH of 5.0 or more, whereas ferric hydroxide will precipitate out at a pH of 2.2 or more, therefore ferric hydroxide is the main one to be concerned with. Some iron control agents are designed to reduce ferric iron (Fe3+) to ferrous iron (Fe2+), whilst others are designed to maintain a low pH (Acetic Acid).

The most common source of iron problems in a well are the tubulars. New tubing is covered with mill scale which is in the form of ferrous oxide (FeO) and ferric oxide (Fe2O3). Acid will dissolve and dislodge this scale from the tubing as it is pumped into the well and carry it down into the formation.

Currently used iron control agents can only effectively control up to 10,000 ppm iron. Case studies have shown that as much as 100,000 ppm of dissolved iron can be picked up by the acid in less than 9000 ft of tubing. For this reason tubing strings should always be cleaned of iron scales with a "Pickling" treatment prior to pumping acid through them and into the formation.

### Emulsions and Sludge.

#### Emulsions.

Most crude oils contain natural chemicals which frequently act to stabilise emulsions formed with acid or with spent acid during a treatment. In general the tendency to form emulsions increases with the concentration of the acid. The viscosity of an emulsion, being higher  than that of either its components, means that its flow from the formation into the wellbore is impeded. An added expense is the disposal of the emulsions aqueous phase once it has been produced. When designing treatments the following should be considered:

- Emulsions can be prevented if tested.

- Crude oil from offset wells can have different emulsifying tendencies.

- Over-treating with surfactants can cause an emulsion.

Production is severely hindered due to the high viscosities inherent with emulsions. Emulsions are usually very easy to prevent by selection of the correct surface acting agents and can be easily determined by simple laboratory tests. Whenever possible, emulsion tests should be carried out using crude oil samples from the well to be treated and the proposed acid treating solutions. It is often assumed that if an acid system does not form an emulsion with crude from one well in the field that it will not form one with others. Unfortunately this does not always hold true. Crude oils from offset wells can have very different emulsion forming tendencies.

#### Sludge.

Some crude oils react chemically with hydrochloric acid during stimulation treatments to form solid or semi-solid particles called sludge, or asphaltene sludge. This can restrict or completely plug the flow channels in the producing formation reducing the effectiveness of the acid treatment. The following can be said of acid sludges:

- Form in wells that produce asphaltic crude oils.

- Can be prevented if tested.

- Low pH destabilises the asphaltic colloidal dispersion.

- High aromatic solvents are required for removal. (Xylene or Toluene).

- Form more readily with stronger acids.

The formation of sludge when acidizing can occur in any crude oil that contains asphaltenes.  In general it can be said that, if a crude oil has a tendency to sludge, it is more likely to form with stronger acids and in greater volumes. Crude oils with this tendency can be identified by simple laboratory tests and treatments can then be designed to prevent their formation.

Sludges form when asphaltenes are precipitated out of the crude oil. Asphaltic material exists in the formation as a colloidal dispersion of minute asphaltene particles permeated by adsorbed maltenes. These  maltene-asphaltene micelles are stabilised by a double electrolytic bond. The low pH environment created by acids disrupts this bond and causes destabilisation of the asphaltenes, thus allowing them to aggregate and to precipitate out from the crude oil.

Unfortunately, sludge is extremely difficult to remove because it is insoluble in most treating solutions, so most effort is directed towards prevention rather than correction. Acid concentrations as low as 1.0% have produced sludge with susceptible crudes, sludge prevention generally becomes more difficult as the concentration of the acid increases.

Sludges are very viscous and usually require the use of a high aromatic solvent (xylene or toluene) for removal. Sludge prevention procedures should be used when acidizing wells that produce asphaltenic crudes.  An aromatic solvent preflush ahead of an acid job will greatly help in preventing sludge from occuring, however, remember to divert the solvent stage, as you would with acid - so that it contacts the zones with the highest tendency for sludging.

# Acid Types.

Although many acid compounds are available to the oil industry, only the following types have been proven economically effective in oil well stimulation:

Inorganic Acids (Strong).

- Hydrochloric Acid (HCl).

- Hydrofluoric Acid (HF), used only in a blend.

Organic Acids (Weak).

- Acetic Acid and Glacial Acetic Acid.

- Acetic Anhydride.

- Citric Acid.

- Formic Acid.

## Inorganic Acids.

### Hydrochloric Acid (HCl).

Hydrochloric acid is an inorganic acid and is the most commonly used acid in oil well stimulation. Hydrochloric acid has many advantages in its application as follows:

- Low cost and availability.

- Easily inhibited to prevent attack on oil-field tubulars.

- Can be emulsified for slower reaction rate.

- Exhibit de-emulsification properties for rapid clean up.

- Most reaction products are water soluble and easily removed.

It has long been recognised that hydrochloric acid is the best field acid for most applications. It is however, not without limitations. Hydrochloric acid is quite reactive; therefore, it will spend quite rapidly on some formations. It is essential with hydrochloric acid to size acid treatments and pump rates to optimise this property.

The reaction rate also dictates the selection of additives that will perform their functions during the relatively short spending time. These same additives must survive the spending process and function in the spent acid. Certain materials are soluble in hydrochloric acid but not necessarily in the spent acid water.

For example, calcium sulphate can be partially solubilised by hydrochloric acid, but will crystallise out as scale when the  acid spends. Iron oxide will dissolve in hydrochloric acid but will re-precipitate, as the acid spends, at about a pH of 2.0. These properties require the selection of additives that will circumvent these problems.

Hydrochloric acid is normally pumped in concentrations ranging from 3.0% to 28%. The low concentration acids are used for the removal of salt plugs and emulsions. The high concentration acids are selected to achieve longer reaction times and to create larger flow channels. By far the most frequently used strength is 15% (15 lbs of Hcl to every 85 lbs of water. The general uses for hydrochloric acid are as follows :

- Carbonate acidizing - Fracture and Matrix.

- Sandstone acidizing - Matrix only.

- Preflush for HCl:HF mixtures.

- Post-flush for HCl:HF mixtures.

- Acidizing sandstones with > 20% carbonate content.

- Clean-up of acid-soluble scales.

- Perforation washes.

Pure hydrochloric acid (muriatic acid) is a colourless liquid, but takes on a yellowish hue when contaminated by iron, chlorine, or organic substances. It is available commercially in strengths up to 23.5° Bé (Baumé scale) or 38.7% percent by weight of solution.

Some processes dictate that hydrochloric acid is not the most suitable acid to use. In these cases, alternatives, such as organic acids (acetic and formic) may be used. These acids are used because of their inherently retarded nature, their ability to be used at higher temperatures and their solvation ability in "dirty" formations. The primary objection to the use of organic acids is their cost and their lack of effectiveness in removing limestone (Table 6).

Table 6: Reaction of Acids on Limestone at Various Concentrations.

| Acid | Concentration % | Calcium Carbonate Dissolved Pounds Per Gallon. | Carbon Dioxide Formed Cu.ft. Per Gallon. | Calcium Salt Formed Pounds Per Gallon. |
| --- | --- | --- | --- | --- |
| Hydrochloric * | 15* 20 25 | 1.84 2.50 3.22 | 6.99 9.47 12.20 | 2.04 2.75 3.57 |
| Acetic | 15* 20 25 | 1.08 1.43 1.80 | 4.09 5.41 6.82 | 1.71 2.25 2.84 |
| Formic | 15* 20 25 | 1.42 1.90 2.40 | 5.38 7.20 9.09 | 1.84 2.47 3.12 |

*	The most commonly used strength for hydrochloric acid is 15%, and 10% for formic and acetic acid. In this table however, HCl reaction with limestone is compared with equivalent strength formic and acetic acids, rather than to the latters commonly used strengths.

Other acids are also used in limited quantities. An example is citric acid, which can be used both alone, or as a component of an acid blend, or for use as a stabiliser, buffer and iron control agent. Also sulfamic acid has been used in the oil industry  on a "do it yourself" basis. Its usage is recommended because of its low corrosivity, although it is limited by its ability to strip chrome from chrome pumps and by its relatively high cost.

### Hydrofluoric Acid (HF).

Hydrofluoric acid, another inorganic acid, is used with hydrochloric acid to intensify the reaction rate of the total system and to solubilise formations, in particular sandstones. In general hydrofluoric acid is used as follows :

- It is always pumped as an HCl:HF mixture.

- Ensure that salt ion contact is prevented.

- Sandstone matrix acidizing.

- Removal of HCl insoluble fines.

- Normal concentrations 1.5% to 6.0%.

Hydrofluoric occurs as a liquid either in the anhydrous form, or in an aqueous solution (as used in well stimulation). Hydrofluoric acid attacks silica and silicates; as well as glass and cement. One gallon of 12:3 HCl:HF will dissolve 0.217 pounds of sand.  It will also attack natural rubber, certain metals such a cast iron and many organic materials.

In well stimulation, hydrofluoric acid is normally used in combination with hydrochloric acid. Mixtures of the two acids may be prepared by diluting mixtures of the concentrated acids with water, or by adding fluoride salts (e.g. ammonium bifluoride, ammonium fluoride solution) to the hydrochloric acid. The fluoride salts release hydrofluoric acid when dissolved in hydrochloric acid. Ammonium bifluoride  salt is nearly always used to make up HCl:HF mixtures. It is preferred, to the use of liquid hydrofluoric acid for safety reasons.

### HCL: HF Mixtures

Since hydrofluoric acid reacts with silicates and most drilling muds, it has been found to be effective in stimulating, and removing damage from sandstone reservoirs. As used in the oil field, mud acid is a dilution of HF acid in HCl. Usually prepared by dissolving ammonium bi-fluoride in HCl, a mixture of 1% HCl and 123 pounds of NH4HF2 will generate 1% HF acid solution. Regular mud acid (12:3 HCl:HF) is hence made from 15% HCl, where 3% HCl is used to hydrolyze the fluoride salt. Excess HCl is added with the sole purpose to increase the solubility of byproducts of the reaction with formation minerals. The 4:1 HCl:HF ratio is based on studying primary reaction of HF on alumino-silicates. Recent studies however, which investigated secondary reactions with fluoroaluminates recommended that this ratio need to be increased to at least 6:1 or to an optimum of 9:1 (Gdanski), so that potential precipitation of fluoride salts is reduced. The best ratio or concentration of HCl:HF strength should be determined by laboratory core tests.  The hydrochloric acid in these formulations has three purposes:

- It acts as a converter to produce HF acid from the ammonium bifluoride salt.

- It dissolves the hydrochloric acid-soluble material and thus prevents the hydrofluoric acid from spending too rapidly.

- It minimizes precipitation of Ca++, Na+, K+ floride salts by keeping pH low.

## Organic Acids.

These acids are used in well stimulation basically because they have a lower corrosion rate and are easier to inhibit at high temperatures than hydrochloric acid. Although mixtures of organic acids are considered corrosive to most metals, the corrosion rate is far lower than that of hydrochloric or hydrofluoric acid, therefore,  organic acids are used when long acid-pipe contact time is required. An example of this is when organic acid is used as a displacing fluid for a cement job. The organic acids is left in the production string. and is subsequently used as the perforating fluid. Organic acids are also used when metal surfaces of aluminium, magnesium, and chrome are to be contacted, such as in trying to remove acid-soluble scales in wells with downhole pumps in place. They can also be used as iron control agents for other acid systems. Many organic acids are available, but the four most commonly used are :

- Acetic Acid / Acetic Anhydride.

- Citric Acid.

- Formic Acid.

### Acetic Acid (CH3COOH).

Acetic acid is a colourless organic acid soluble in water in any proportion and in most organic solvents. Although mixtures of acetic acid with water are considered corrosive to most metals, the corrosion rate is far lower than that of hydrochloric and hydrofluoric acids. Acetic acid is easy to inhibit against corrosion and is used frequently as a perforating fluid where prolonged contact times are required. With this ability, the acid is sometimes used as a displacing fluid on a well cementing job, where the contact time may be hours or days before perforating takes place. This ability is beneficial in three ways:

- Reduces formation damage. The first fluid two enter the formation will be an acid or low pH fluid which will react with carbonate or the calcareous materials of a sandstone formation.

- Reduces clay swelling.

- Can be used where aluminium, magnesium or chrome surfaces must be protected.

The relation of dissolving power of one gallon of a 15% concentration of acetic acid compared to that of hydrochloric acid and formic acid at the same volume is listed in Table 6. The cost of acetic acid per unit, based on dissolving power, is more expensive than either hydrochloric acid or formic acid.

Normally, acetic acid is used in small quantities or with hydrochloric acid, as a retarded acid. The general uses and properties of acetic acid are as follows:

- Available as “Glacial” Acetic - a 99% pure solution.

- Acetic acid is relatively weak and reacts to less extent than Hcl.

- Easier to redard than Hcl, thus good for high temp. applications.

- Normal concentrations of 7.5% to 10% when used alone.

- Used as an iron control additive.

- Perforating fluid.

Commercially available acetic acid is approximately 99% "pure". It is called glacial acetic acid because, ice-like crystals will form in it at temperatures of approximately 60° F (16° C) and will solidify at approximately 48° F (9° C). When glacial acetic acid is mixed with water, a contraction occurs. For this reason, the amount of acetic acid and the amount of water normally total more than the required volume.

### Formic Acid (HCOOH).

Formic acid is the simplest of the organic acids and is completely miscible (capable of being mixed) with water. Formic acid is stronger than acetic acid yet weaker than hydrochloric acid. Formic acid is used in well stimulation, most frequently in combination with hydrochloric acid as a retarded acid system for high-temperature wells. The percentage of formic acid used in such applications is commonly between 8.0% and 10%. Formic acid can be easily inhibited, but not as effectively as with acetic acid at high temperatures and long contact times. The properties and uses of formic acid parallel those of acetic acid as stated below:

- Formic acid is relatively weak.

- Seldom used alone.

- Mainly used in hydrochloric acid mixtures.

- Corrosion inhibitor aid.

- Hot wells.

Acetic acid, acetic anhydride and formic acid are used when exceptionally retarded acid is needed because of extreme temperature or very low injection rates. At high temperatures, blends of organic and hydrochloric acid are much more successfully inhibited by organic inhibitors, than when hydrochloric acid is used alone. This property minimises the danger of hydrogen embrittlement of steel associated with hydrochloric acid treatments in high-temperature wells. Organic acid concentrations of up to 25% by weight are required, making acid treatment costs increase. Organic acids do not give as much reacting capability as hydrochloric acid treatments (see Table 6, page 67).

# Systematic Approach to Matrix Acid Treatment Design.

Successful acidizing begins with recognizing a viable candidate.     Geographic Probability;

Wells that have skin damage are good candidates for well stimulation treatments. Major increases in productivity or injectivity can result. The well and the treatment, however, should be selected with care, and reservoir conditions should be adequate to assure economic pay-out. Misapplied stimulation treatments are costly and ineffective, often creating more problems than they solve. Simplified to a series of questions, the following is the process for recognizing a viable acidizing candidate.

- Is the well performing poorly, or is it simply a poor reservoir?

- Is formation damage the cause?

- Which, if any, acid system will remove the damage?

- How will the formation respond to this acid?

Selecting the correct treatment is often not a simple matter. With an engineering approach to any well problem however, the chance of success is generally increased. The following information should be considered in the selection of a well treatment:

## Primary Considerations.

- If formation damage does not exist, then a matrix acid treatment will probably not be economically viable.

- The production potential must be determined. Even if damage exists, the treatment may not be economical if the production icrease potential is small.

- Production systems must be evaluated to be sure that these are not hindering production. (Size of tubing, perfs, flow-lines etc.). The BJ Services “Production Optimisation Program“ (POP) can be used for Nodal Analysis.

- Obtain information about the formation's physical characteristics and chemical properties. (Acid solubility, Emulsion tendencies, Permeability, Porosity, Mineralogy, Flow Tests, etc.).

- Obtain information about the properties of the formation fluids. (Oil analysis, ie. PVT & BSW, Water analysis and Scaling tendency). Treating fluid compatibility with contaminants present and reservoir fluids.

- Determine the cause, type and extent of formation damage.

- Design the acid treatment to clean-up the damage and to prevent acidizing damage.  Start design with bottom hole temperature in mind.

- The treatment must be placed properly.  Diversion is normaly used in intervals larger than 20 ft.

- Determine if any physical limitations form well hardware will affect the job. eg. Wellhead connection, Down hole configuration, Corrosion of casing should be considered.

### Drilling, Completion and Work-over Design Considerations.

#### Drilling Considerations.

1.	Was the correct type of drilling mud used.

2.	What was the fluid loss of the mud whilst drilling through the zone of interest.

3.	Did lost circulation occur. How many barrels of mud were lost to the formation.

4.	Was lost circulation material used. If so, how much and what kind.

5.	Are there extreme washouts in the production zone.

#### Drilling Formation Damage Mechanisms.

1.	Invasion of mud particles into the formation which includes clays, barite, other weighting agents, lost circulation materials and cuttings.

2.	Drilling and cement filtrate damage may result in polymer plugging, scale formation, surfactant altered wettability damage, clay swelling, clay dispersion, asphaltene deposition, and oil-mud sludging.

Table 7: Drilling Damage Treatment Options.

| Situation | Treatment |
| --- | --- |
| Mud damage, Clay swelling, | Standard HCl:HF mud acid |
| Clay dispersion, Polymer | Silt and sand suspending |
| Damage and Scales. | agents are useful. |
| Wettability damage. | Mutual Solvents and or |
|  | Surfactants. |
| Sludging and Asphaltene | Paravan/Solvent Soaks. |
| Damage. |  |

### Drilling Mud Damage.

Shallow formation damage, commonly called skin damage, results when incompatible fluids and solids invade the formation. On new wells, mud solids and filtrate from the drilling fluid cause most of this contamination.  The mud filter cake deposited on the exposed formation face during drilling operations consists of solid drilling mud particles and some drill cuttings. This filter cake forms a cylindrical barrier of reduced permeability around the wellbore. Generally, the penetration into the formation of drilling solids seldom exceeds a very few inches in non vugular or unfractured formations.

Laboratory tests show that acid treating solutions are most effective for removing this type of shallow formation damage. Although the acid usually dissolves only part of the mud solids, it effectively penetrates and disperses the remaining insoluble solids by reacting with the formation to which they are attached.

In removing skin damage at the wellbore, small treatment volumes at low pressures and injection rates generally are employed. Low rates allow the treating fluid to move radially and uniformly into the damaged zone for more effective clean-up.

In designing a mud removal treatment during initial well completion several factors should be considered.

- Type of mud.

- Acid Solubility of the formation.

- Type of formation.

- Length of perforated or open hole interval.

Acid solutions recommended for treating wellbore damage caused by various types of drilling muds are presented in (Table 8). Whilst this table can be used as a guide, laboratory flow tests on representative cores and fluids at corresponding temperatures should be conducted whenever possible. For example, emulsion testing will determine the correct concentration of surface acting chemicals to be used in the treating solution.

Table 8: Recommended Treatment for Damage Removal Based on Mud Type and Formation Type.

| Mud Type | Limestone - Dolomite Formations (7-1/2% - 28%)* | Sandstone Formations (5% -15%) * |
| --- | --- | --- |
| Water Clay | Clean-up Acid Regular HCl + 1.0% NE-13 MMR Suspending Acid NE-Type Acid ** | Mud Sol; Mud Sol 15:3 Regular HCl + 1.0% NE-940, NE-118 Clean-up Acid HCl:HF Acid Mixes Alcohol-Acid |
| Oil Emulsion | Clean-up Acid Regular HCl + 1.0% NE-13 MMR Suspending Acid NE-Type Acid ** One Shot Acid "Plus" (HCl) | Mud Sol; Mud Sol 15:3 Clean-up Acid HCl:HF Acid Mixes Alcohol-Acid NE-Type Acid ** One Shot Acid "Plus" (HCl:HF) |
| Inverted Oil Emulsion | Regular HCl + 1.0% NE-13 NE-Type Acid ** One Shot Acid "Plus" (HCl) | Mud Sol Regular HCl + 1.0% NE-940, 118 NE-Type Acid ** One Shot Acid "Plus" (HCl:HF) |
| Oil Base | Kerosene, Diesel, Crude Oil Containing NE-110 and/or Acetic Acid One Shot Acid "Plus" (HCl) | Kerosene, Diesel, Crude Oil Containing NE-110 and/or Acetic Acid One Shot Acid "Plus" (HCl:HF) |

*	Range of concentration which applies to all acid groups.

**	Acid containing NE-additive, as determined by emulsion tests.

### Drilling Fluid Damage in Horizontal Wells.

Damage around a horizontal wellbore is neither radial nor distributed evenly along its length. Variations in permeability (anisotropy), create an elliptical damage profile normal to the wellbore (Figure 25). The exposure time to fluids during drilling and completion operations will result in a truncated elliptical cone of damage along the length of the well (Figure 26), with the base of the cone nearest the vertical section of the wellbore.

Figure 25: Schematic of Typical  Damage Profiles in Horizontal Wells.

Iani 	=	Permeability anisotropy variable.

Figure 26: Schematic of Typical Horizontal Well Damage Cone.

Diversion is often difficult to achieve in horizontal section (acid tends to take the path of least resistance), even when using mechanical diverting aids or coiled tubing. Taking the large quantities of acid required for matrix stimulation of horizontal sections into account, it is often desirable to consider only partial damage removal. This will result in the creation of a stimulated zone with improved permeability surrounded by a collar of damage (Figure 27). However, distribution of the stimulation fluid along the length of the well is still important, particularly when considering the nature of the damage distribution.

Figure 27: Partial Stimulation With Damage Collar.

rw	Wellbore radius.

ri	Radius of improved permeability (Stimulated).

rs	Damage radius.

re	Drainage radius (Boundary).

Pe	Outer pressure boundary.

Pw	Wellbore Pressure.

When stimulating with coiled tubing, the rate of tubing withdrawal and the volume of fluid pumped into each section of the hole should take into account the identified shape of the damage cone present, and the type of formation to be stimulated. For sandstones, the stimulation fluid injection should mimic the shape of the damage cone, and a truncated cone of improved permeability should result with acid volumes and tubing withdrawal rates being closely related to porosity. For carbonate reservoirs, the stimulation profile under matrix conditions will be dendritic, with a wormhole network extending radially from the wellbore. The volume of  acid and withdrawal rate of the coiled tubing will be related to the expected radial wormhole porosity created.

Figure 28: Schematic of Stimulation Profiles for Sandstones and Carbonates.

(Looking down the horizontal lateral section.)

### Matrix Acidizing Horizontal Wells.

Horizontal wells are normally targeted for thin formations with good vertical permeability and reservoirs that suffer from coning problems. Matrix stimulation of long horizontal sections have been shown to be successful compared to fracture treatments in reservoirs with relatively low permeabilities (0.5 to 1.5 md) and small vertical height (less than 50 ft). Outside of this narrow range, reservoirs where vertical wells are normally considered candidates for fracture treatments, will also require hydraulic fracturing when drilled horizontally.

When matrix acidizing horizontal wells, all aspects of job design that apply to vertical wells should be considered. Placement of fluids along the length of the horizontal section is critical to the success of any stimulation. “Bull-heading” acid into horizontal wells has generally proven to be unsuccessful. In  most cases the acid has a tendency to enter the formation in a zone close to the vertical section thus, obtaining only partial stimulation.

To overcome this problem, various methods have been applied to improve the distribution of stimulation fluids along the well. These methods include chemical diverters, mechanical isolation devices, partial perforation techniques (leaving blank portions in the casing, and coiled tubing.

In general coiled tubing has proven to be the most effective method for acid washes and matrix stimulation and can be used to obtain adequate coverage. With this technique, the coiled tubing is placed at the end of the horizontal section and withdrawn towards the vertical section whilst pumping acid and diverter stages. At the same time an inert fluid is pumped down the to aid placement and reaction of the acid at the point of injection. However, the stimulation fluid is likely to follow the path of least resistance making diversion and zonal isolation essential. Drawbacks to this method of stimulation include long acid to tubing exposure times (due to limitations on pump rate through coiled tubing. The section to be treated in a horizontal well can be several thousand feet long, thus treatment volumes will be large and particular attention should be paid to economics.

## Cementing Considerations.

1.	Did lost circulation take place during cementing. If so, how much was lost and was it lost to the production zone.

2.	Was KCl or salt used in the cement slurry. (Clays, shales and formation water compatibility).

3.	What was the fluid loss of the cement slurry.

4.	Is there a good bond (to pipe and formation) across, above and below the productive zone (CBL).

5.	Are there any channels in the cement (CBL, CET).

6.	What type of cementing preflush was used.

## Lost Circulation Materials (LCM).

1.	Determine if HCl Soluble.

- CaCO3 dissolve in 15% HCl (1.84 lbs per gallon of acid)

- FeCO3 dissolve in 15% HCl (2.12 lbs per gallon of acid)

2.	Determine volume of LCM pumped.

3.	Calculate additional volume of HCl required to completely remove 	the material, this volume should be added to the required volume calculated to achieve a 2.0 ft radial penetration.

4.	Acid soaks are often helpful.

5.	Auxiliary nitrogen or carbon dioxide is usually required to ensure even distribution of the acid.

## Perforating Considerations.

1.	Was the casing and tubing cleaned (pickled) prior to perforating. (Mill varnish, iron oxide scale).

2.	Was too much pipe dope used.

3.	Was the perforating fluid filtered. If not what was the solids content.

4.	Was the well perforated overbalanced. If so, with how much differential pressure.

5.	Was the well perforated under-balanced. If so, with how much differential pressure.

6.	What is the theoretical depth of penetration of the charge used.

7.	What is the shot density. Did all the shots fire.

8.	Were charge size and shot density the correct choice for the lithological characteristics.

## New Completion Considerations.

1.	Was the well killed. If so, what was the kill fluid.

2.	How much volume of kill fluid was used. Was this volume completely produced back.

3.	Was the kill fluid filtered. If not, what was the solids content.

4.	Was the kill fluid tested for compatibility with the crude oil or with crude oil from an offset well.

5.	Was the well circulated clean, any surfactants added for surface tension reduction.

The most common cause of completion fluid damage is dirty fluids resulting from poor filtration practices and fluid incompatibility.

Table 9: Completion Fluid Damage Treatment Options.

| Damage Mechanism | Treatment Options |
| --- | --- |
| Solids invasion | HCl:Mud acid, Hcl acid |
| Clay swelling | HCl:Mud acid |
| Clay mobilization | HCl:Mud acid |
| Water block | Surface tension reduction |
| Emulsion block | Dependent on emulsion external phase. |
| Polymer Damage | MudZyme Treatments and/or Acid/Oxiders |
| Wettability damage | Mutual solvents and/or surfactants |
| Precipitation of solids | Solids dependent |

## Work-over Considerations.

1.	Refer to completions questions.

2.	Has scale inhibitor been used. If so, is it still effective.

3.	Have acid treatments been performed in the past. If so, what did they consist of and how much volume was pumped.

4.	How were the previous jobs pumped.

5.	Has diverting material ever been used. If so, how much and what kind.

6.	Were previous treatments successful. If so, how successful in terms of folds of increase.

7.	Have the production increases from previous acid treatments been long term.

8.	Has the zone been previously fracture stimulated. If so, what volume and type of fluid were pumped .What type and volume of proppant were used. What were the treatment results.  What was the obtained fracture height.

## Production Considerations.

1.	Is the well's production decline curve typical for the field.

2.	What is the current gas/oil ratio (GOR). Has this changed with time.

3.	What is the current water/oil ratio . Has this changed with time.

4.	Has the water/oil ratio changed with total produced fluid remaining the same.

5.	Has the API gravity of the crude oil changed.

6.	What is the current asphaltene content of the crude oil. Has this changed with time.

7.	What is the current paraffin content of the crude oil. Has this changed with time.

8.	Have paraffins or asphaltenes been known to accumulate at any time in the flow lines, surface equipment, production string, or perforations.

9.	Have scale deposits been found. If so, where and what type.

10.	Has formation sand been produced.

## Production Curves.

As water increases:

- Formation fines plugging occurs.

- Chloride content increases, resulting in the nucleation and deposition of acid-insoluble organic deposits.

- Maltenes are stripped due to their lower melting point than that of the host asphaltene molecule, resulting in asphaltene deposition.

- Higher probability of the formation of mineral salts.

Sudden Decline:

- Mechanical Failure.

- Fines Migration

Water increases, oil decreases, but total fluids remain the same:

- Chemically altered wettability1.

- Acid insoluble organic deposition1.

- Depletion.

1 This damage should be removed with low cost non-acid systems.

Increasing  GOR with associated drop in API Gravity:

- Acid insoluble organic deposition.

## Treating Pressure Design Considerations.

1.	When matrix acidizing, the bottom hole fracture pressure (BHFP) must not be exceeded. (The fracture pressure decreases with reservoir pressure). BHFP and surface treating pressure (STP) should be determined. All personnel involved, should be informed of the maximum STP and instructed not to exceed it.

2.	Injection rate control should be maintained for low reservoir pressure by using nitrogen or a control valve.

3.	If the treatment is done over balanced, solid diverting materials such as Oil Soluble Resins (OSR) or Benzoic Acid should be used. Uniform zonal coverage is difficult to achieve when overbalanced. Overbalance causes the highest incidence of unplanned fracturing.

4.	If the treatment is performed under-balanced, non particulate diverting material can be effective. Utilisation of Nitrogen in conjunction with foam or solids is required. 100 % equipment back-up should be utilised.

Treatment guidelines for sandstone reservoirs with Nitrogen and Carbon Dioxide:

Table 10: Carbon Dioxide (CO2) and Nitrogen (N2) Guidelines.

| Reservoir Type | Nitrogen | Carbon Dioxide |
| --- | --- | --- |
| Gas. | Yes | Yes (1) |
| Low Pressure Gas. | Yes | (2) |
| Oil, (Above the Bubble Point). | (3, 4) | Yes |
| Oil, (Below the Bubble Point). | Yes | Yes |
| Water Injection or Disposal Well. | (3, 4) | (3, 4) |

1) Larger volumes are normally required to create a free gas phase.

2) Due to higher CO2 density, sufficient pressure may not be generated to achieve back flow.

3) Temporary trapped gas saturation may affect penetration and recovery.

4) Technical staff approval required.

## Injection Rate and Surface Treating Pressure.

The injection rate of an acid treatment can greatly influence the placement of the acid and therefore the success of the treatment. The type of acidizing job (matrix, natural fractures or fracture acidizing) usually determines the ideal injection rate.

### Low Injection Rates.

Pump

rates which produce pressures below that required to fracture a formation, usually place the acid in the near wellbore area, within the rock matrix and where present, in naturally occurring fractures. Low rates are recommended to repair skin damage, shallow damage or where it is not intended to fracture the formation, as when treating in proximity to a zone of high water saturation.

Practical factors that influence the rate used, and which require that the rate is kept low are:

- High formation pressures.

- Pressure limitations of the tubular goods.

- Pressure limitations of other well equipment.

### High injection Rates.

Due to the high reaction rates of hydrochloric acid with carbonates present in the formation, higher injection rates are required for deeper penetration of live acid. Since high injection rates customarily involve high pressures, the formation is likely to be fractured during such treatments.

On multi-zone treatments, where several zones accept fluid at different pressures, more complete coverage can be obtained by employing a high rate of injection. (Limited entry techniques).

## Establishing Pump Rate and Surface Treating Pressure.

Sandstone matrix stimulation treatments are performed at less than fracturing pressures. If the formation is fractured, the stimulation fluid takes the path of least flow resistance through the fractures and does not uniformly contact the desired treating interval. This results in uneven drainage of the reservoir and only slight increase in the productivity.

Figure 29 is used to establish the maximum allowable pump rate that will not fracture the reservoir.

Figure 29: Injection Rates (Non-Fracturing) into Permeable Formations at Various Differential Pressures.

( For small volume treatments in partially saturated reservoirs)

The maximum surface treating pressure at the maximum allowable pump rate is calculated as follows:

Maximum Surface Treating Pressure =

(Fracture Gradient x TVD) + Friction Pressure - Hydrostatic - 300 psi

Note : 300 psi is an arbitrary safety factor to assure that the reservoir will not be fractured.

In many sandstone matrix stimulation treatments, the initial injection rates will be substantially less than those predicted when using the above equation and Figure 29, when pumping at the maximum allowable treating pressure. This is caused by permeability damage in the near wellbore area. Once this damage is removed by the acid , the predicted and actual injection rates will be close in value.  When the calculated injection rates are less than 0.5 barrels per minute, the well probably will respond better to a proppant fracturing treatment, than to a sandstone matrix treatment, since the treating time would be extremely long for matrix stimulation.

Example :

Well Depth  (TVD)				9500 		ft

Permeability 					100 		md

Formation Thickness (MD)			10 		feet

Fracture Gradient				0.65 		psi/foot

Fluid in Hole					15%:4.0% 	HCl:HF

Hydrostatic Pressure Gradient		0.476 		psi/foot

Formation Pore Pressure			4500 		psi

Tubing Size					2-3/8 		inch

Formation Fracture Pressure	= 	Fracture Gradient x Depth (TVD)

= 	0.65 x 9500

= 	6170 psi

Hydrostatic Pressure		= 	Hyd. Pressure Gradient x Depth (T.V.D.)

= 	0.476 x 9500

= 	4522 psi

Differential Pressure		= 	Fmtn. Frac Press - Formation Pore Press - 300 psi

= 	6170 - 4500 - 300

= 	1370 psi (P)

Formation Flow Capacity		= 	Formation Thickness (MD) x Permeability

= 	10 x 100

= 	1000 md. ft.

Maximum Injection Rate		= 	2.3 Barrels Per Minute. (From Figure 29)

Friction Pressure			= 	Friction Pressure From Ref. Figures

Specific Gravity x Depth (MD)/1000

= 	90 x 1.08 x 9500/1000

= 	922 psi.

Surface Treating Pressure		= 	(Fracture Gradient x TVD) + Friction

Pressure  - Hydrostatic - 300 psi

= 	6170 + 922 - 4522 -300 psi

= 	2270 psi

Therefore maximum surface treating pressure is 2270 psi at a maximum treating rate of 2.3 barrels per minute.

## Shut-In Times.

Shut-in time is the length of time a well is closed in after a stimulation treatment is completed, before flow back is initiated. This time is determined by the type of acid used and by downhole factors such as formation type, bottom hole temperature and bottom hole pressure.

After an acid solution has been neutralised by reaction with the formation, it is no longer a stimulation fluid. However it may become harmful to the formation permeability if allowed to remain downhole.

### Hydrochloric Acid with Limestone.

Hydrochloric acid reacts so rapidly with limestone that it is essentially neutralised by the time the acid has been completely placed. This generally holds true at all ranges of temperature and pressure. Since limestone formations incorporate varying amounts of insoluble materials that can plug  permeability, if allowed to come to rest, it is important to remove the neutralised hydrochloric acid as soon as it is spent. Shut-in times with such formations is zero.

### Retarded or Emulsified Acid.

When chemically retarded acids such as the Sta-Live systems, and emulsified acids such as SRA-3 are used, the reaction time of the acid can exceed the displacement time. Here a shut-in time of one to two hours is recommended for maximum stimulation. The shut-in time may be extended where there is sufficient bottom hole pressure to promote rapid clean-up.

### Organic Acid and HCl Acid Mixtures.

Mixtures of acetic or formic acid with hydrochloric acid delay the time required for complete spending of the acid mixture and thus require a shut-in time.

### Sandstone Acidizing with HCl: HF Mixtures.

The acid mixtures required to stimulate sandstone reservoirs sometimes create special problems. For example, the reaction rate of hydrofluoric acid with sand, clay and silt is much slower than the practical injection rate. But because of detrimental side effects such as the formation of Na3SiO6, CaF2, and NaF, the shut-in time should be limited to the time required to prepare the well to flow back.

Other harmful effects can result from excessive shut-in times with sandstone reservoirs. One such effect is the reaction of acid solutions with water sensitive clays. While acid is not believed to swell these clays, its reaction does dislodge portions of the clay that can act to block the flow of fluids into or out of the well.

Another potentially harmful effect stems from the reaction of hydrochloric acid on the soluble carbonate material that consolidates the sand grains in most sandstone formations. Reaction with hydrochloric acid can remove this consolidating material, breaking down bonds between the sand grains. The result can be an unconsolidated sand zone with a lowered permeability. To avoid this problem careful attention should be paid to the strength of hydrochloric acid selected based on temperature and carbonate content of the formation.

Where excessive shut-in times cannot be avoided, such as when sucker rods or downhole pumps have to be run to flow back the well, it is advisable to overflush the acid treatment with brine or water, a sufficient distance out in to the formation, to prevent deposition of undesirable reaction by-products in the near wellbore area.

Table 11: Formation Damage Treatment Options for Different Drive Mechanisms.

| Formation Damage Mechanism | Treatment |
| --- | --- |
| Water Drive |  |
| Accelerated water production after acidizing. | Selective acidizing SAF-I,II,III |
| Asphaltene damage | Paravan/Solvent soaks |
| Water production | Aquatrol 1,2 or 3, Direxit |
| Scale formation | Depends on composition |
| Gas Drive |  |
| Water block | Reduce surface tension |
| Paraffin damage | Paravan/Solvent soaks |
| Solution Drive |  |
| Water block | Reduce surface tension |
| Inadequate formation pressure | Carbon Dioxide or Nitrogen |
| Organic formation damage | Paravan/Solvent soaks |
| Combination Drive | All of the above |

## Bottom Hole Temperature Design Factors.

- 1.	Select acid strength according to Maximum Strength recommended (see

Table 1, page 13).

2.	Acid reaction rate is exponential with temperature therefore, there is a very high potential for acid attack of tubulars.

3.	High temperature  acidizing can result in wettability damage from corrosion inhibitors and surfactants. Use mutual solvents in the postflush. (EGMBE, US-40, US-2).

4.	Live acid penetration is reduced in high temperature wells due to increased acid reaction rates. This can create problems of reduced near wellbore permeability as a result of compressive strength reduction. This is caused by the total removal of the matrix binder in the formation ( for example; carbonates, silicates, clays etc.).

5.	Damage from reaction precipitates is more likely in high temperature wells.

6.	Corrosion inhibitor concentrations are critical at higher temperatures. Reduce the contact time and cool down the well whenever possible.

7

.	Acid contact times in the near wellbore area should be no more than four hours at low temperatures (less than 180° F (82° C)). At higher temperatures, contact times should be limited to the time required to prepare the well for flow back.

Table 12: Temperature Considerations.

| Situation | Temperatures |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 150°F | 165°F | 170°F | 180°F | 185°F | 200°F | 240°F |
| Emulsion Blocks | L | L | M | M | M | U | U |
| Organic Deposition | L | L | M | M | M | M | U |
| Asphaltene Coking | U | U | U | U | U | M | L1 |
| Bacteria | L | L | M2 | M2 | M2 | U | U |
| CaCO3 Scale | L | L | L | L | L | L | L |
| BaSO4 Scale | L | L | L | M | M | M | U |
| CaSO4 Scale | L | L | L | M | M | M | U |
| Rigid Emulsion Film | L | L | L | L | L | M | M |
| Are Retarded HF Systems Economical | L | L | M | M | M | U | U |

L = Likely      M = Maybe      U = Unlikely

1	API gravity must be less than 32° and asphaltene to maltene ratio should be  1:1.

2	Microbiological damage only occurs at these temperatures as a result of contaminated gel or sea water and/or surface water contact containing no biocide.

## Formation Fluid and Rock Characteristic Considerations.

1.	Are sensitive clays present. What volume and what kind of clay. What kind of clay sensitivity (Swelling, migrating, water entrapment).

2.	Are iron bearing minerals present and in what volume.

3.	Are feldspars present and in what volume and what type.

4.	What is the solubility in HCl Acid.

5.	Is the formation consolidated or unconsolidated. What is the matrix binding or cementing material. Is the formation susceptible to sand production.

6.	Is the formation geology homogeneous vertically and horizontally. What is the horizontal and vertical permeability (Anisotropy)

7.	What is the porosity.

8.	What is the water saturation.

9.	Is there a distinct oil/water contact. Is there a distinct oil/gas contact.

10.	What is the crude oil API gravity. What is the crude oil viscosity at down hole conditions.

11.	What are the paraffin and asphaltene contents of the crude oil.

12.	Does the crude have natural emulsifying tendencies.

13.	What is the formation brine pH.

14.	Does the formation water have a scaling tendency.

15.	What is the bottom hole static temperature (BHST).

16.	What is the reservoir pressure (BHP)

17.	What is the bottom hole frac pressure (BHFP).

## Mineralogy Design Factors.

1.	If solubility in hydrochloric acid is greater than 18%, use HCl only.

Do not use hydrofluoric acid (HCl:HF). (See Table 17, page 101)

2.	If solubility in hydrochloric acid is greater than 10%, then the standard HCl volume of half the HCl:HF volume is usually adequate.

3.	If the total clay content of the formation is less than 5.0%, then use 7.5% HCl : 1.5% HF.

4.	If the total clay content of the formation is greater than 5.0%, then use 12% HCl : 3.0%. HF.

5.	If iron bearing minerals are present (chomasite, siderite, haematite or pyrites), use iron control additives at volumes determined from core tests or spent acid returns. (Total iron in parts per million divided by five will give an estimate of the ferric iron content).

6.	Presence of iron bearing minerals will cause asphaltene precipitation. The use of anti-sludge additives or solvent preflushes depending upon the severity of the sludging potential.

7.	If chlorite clays are present, increase the iron control additive concentrations in the HCl preflush.

8.	If illite clays are present and the permeability is less than 120 md, reduce the surface tension to at least 30 dynes per centimetre.

9.	If the feldspar content is less than 20 %, use 12% HCl : 3% HF.

10.	If the feldspar content is greater than 20 %, use 7.5 % HCl : 1.5 % 	HF.

11. 	If the formation is susceptible to de-consolidation, use low strength HCl and HCl : HF concentrations.

Clays and silts may remain to plug the permeability after have been acidized. Suspending Acids (e.g. MMR acid) have therefore been developed which "suspend" these undissolved particles, thereby helping to remove them from the formation. Fines stabilising agents (FSA-1) have also been developed to “lock” the fines, in particular clays, in the formation matrix. The special suspending agents (MMR agents) can be added to many of the different acid treating systems that are designed for the removal of specific damage mechanisms.

Both hydrochloric acid and hydrofluoric acid systems have been developed for the suspension and removal of formation fines and invasion particles from mud etc.

Table 13: Treatment Options for Formation Damage Caused by Indigenous Minerals.

| Mineral | Potential Problems | What To Avoid | Treatment Options |
| --- | --- | --- | --- |
| Kaolinite | Mobile Fines | High Flow Rates High Draw Downs | Rate Control. Reduce downhole pressure drop. <5%-Acidize - Standard HCl/HCl:HF >5% - BJ Sandstone Acid with ClayMaster |
| Illite | Mushing | Fresh Water Systems | Permeability > 120 md Acidize - Standard HCl/HCl:HF Permeability < 120 md use surfactants to reduce surface tension to 30 Dynes/Cm2 |
| Smectite (Montmoril-lonite) | Swelling | Fresh Water Systems | Acidize - Standard HCl/HCl:HF Perform Immersion Tests to see if alcohol/acid blends or other additives are necessary to prevent swelling. |
| Chlorite | Iron Hydroxide Precipitate, Hydrous Silicate, Amorphous Alumino Silicate Scale | Oxygen Rich Systems: Ph>2.8 Inadequate Sequesterant. Buffered Acid | Acidize - 7.5-1.5% HCL:HF or Acetic:HF with properly evaluated sequestering agents, Preflush with 5% Acetic. Overflush as required. |
| Mixed Layer Illite/ Smectite | Swelling | Fresh Water Systems | Acidize - Standard HCl/HCl:HF |
| Feldspars | Silica Precipitation | High % of HF Acid | > 20%- 7.5% Hcl, 7.5:1.5 HCl:HF < 20%- 10% Hcl, 12:3 HCl/HCl:HF Preflush with 15% Hcl. |
| Chamosite | Iron Hydroxide Precipitate | Oxygen Rich Systems: Ph>2.8 | Acidize - Standard HCl/HCl:HF with properly evaluated sequesterants |
| Formation Fines (F’spars, Qtz.) | Mobile Fines | High Flow Rates | Preflush - 7.5% HCl Acidize - 7.5:1.5 HCl:HF. Use BJ Sandstone acid for damage > 6 inches. Overflush - as required. |
| Calcite, Dolomite | Calcium Fluoride Precipitation | Contact of calcium ion with HF Acid. | >20% - HCl Only. <20% - Standard HCl/HCl:HF Preflush with Hcl for 3 feet of radial penetration. |
| Siderite, Hematite | Iron Hydroxide Precipitation | Oxygen Rich Systems | Acidize - Standard HCl/HCl:HF with properly evaluated sequesterants |
| Pyrite | Iron Hydroxide Precipitate. Asphaltene ppt. | Oxygen Rich Systems | Preflush - 25 Gal/ft Xylene + 10% Acetic Acidize - Standard HCl/HCl:HF with properly evaluated sequesterants |

## Permeability Design Factors.

Acid Volumes see

Table 14.

- If the permeability is less than 120 md reduce surface tension of acid and flushes to 30 dynes/cm2 or less to prevent water blocks.

- If present, multiple permeabilities should be considered when designing diverter/acid stages for proper diversion and volume control.

Several studies have been conducted on acid volume optimization.  The general consensus of published literature is that jobs using less than 75 gal/foot of perforations are about 30% successful, while jobs where 75-125 gal/foot  is used have a success rate of  75%.  Jobs of over 200 gal/foot have had good results (75%), but few have been done due to cost considerations.  The table below provides a rule of thumb volume recommendation based on permeability.  Field studies have shown tha best job results occur on reservoirs with 5 md. permeability or better.  HF jobs on sandstones of less than 2 md. are not recommended.

Table 14: Acid Treating Volumes Based on Permeability.

| Avg. Permeability Without Damage Ku (md) | Normal HCl Range Gal/ft (Lt/Mt) | Normal HCl:HF Range Gal/ft (Lt/Mt) | Comment |
| --- | --- | --- | --- |
| < 0.1 md | 15-25 (185-310) | Not Recommended | 1,3,4 |
| 0.1 - 2.0 md | 25-50 (310-620) | Not Recommended | 1,3,4 |
| 2.0 - 10 md | 35-75 (430-930) | 75-100 (930-1240) | 3 |
| 10- 50 md | 50-100 (620-1240) | 100-150 (1240-1865) | 3 |
| > 50 md | 50-100 (620-1240) Formic Acid 100(1240) | 100-200 (1240-2480) | 2 |

1) 	Volumes should be selected based on core tests.

2)	Volumes can exceed 100 gallons per foot if necessary without releasing excessive fines.

3) 	Volumes can be modified if indicated by field test results

4)	Use acid for perforation cleaning only.

Laboratory and pilot test data are converted for field use by expressing the recommended treating volume as gallons per square foot (see Figure 32, page 100). In the laboratory a known volume of treating fluid is flowed across a given cross-sectional area of a core sample. The are to be treated in field applications is determined by the radial area of the edge of the treating radius. Once the radial area is determined, it is multiplied by the recommended treating volume (in gallons per square foot) and by the height of pay zone interval to be treated. In order to simplify these area calculations Figure 30 can be used.

Figure

Figure

Figure 30: Recommended Volume of HCl:HF in Gallons / Sq. Ft. of Pay.

Example :

Treating Radius 	= 	5.0	feet

Recommended Treating Volume 	= 	20	gallons per sq.ft.

Treated interval 	= 	10 	feet

From Figure 30 Gallons per foot of treated interval 	= 	630 	gallons

Volume of HCl:HF required = (630 x 10)	=	6300	gallons
In estimating the damage radius of a well in the absence of well test data the following can be used as a guide to estimate treating volumes and pump rates:

- Permeability is 5.0 md or less - assume damage zone thickness to be 3.0 inches. (Refer to Table 15).

- Permeability is greater than 5.0 md - assume damage zone thickness to be 6.0 inches. (Refer to Table 16).

Table 15:  Volume of 12:3 HCl:HF Required to Treat a 3.0 Inch Damage Radius - Gallons Per Foot of Pay.

| Pump | Temperature |  |  |  |
| --- | --- | --- | --- | --- |
| Rate | 100° F | 150° F | 200° F | 250° F |
| BPM/ft | Gal/ft | Gal/ft | Gal/ft | Gal/ft |
| 0.001 | 70 | 80 | 100 | 120 |
| 0.005 | 50 | 65 | 75 | 80 |
| 0.010 | 50 | 55 | 65 | 75 |
| 0.025 | 50 | 50 | 55 | 65 |
| 0.050 | 50 | 50 | 50 | 50 |
| 0.100 | 50 | 50 | 50 | 50 |
| 0.200 | 50 | 50 | 50 | 50 |

Table 16:  Volume of 12:3 HCl:HF Required to Treat a 6.0 Inch Damage Radius - Gallons Per Foot of Pay.

| Pump | Temperature |  |  |  |
| --- | --- | --- | --- | --- |
| Rate | 100° F | 150° F | 200° F | 250° F |
| BPM/ft | Gal/ft | Gal/ft | Gal/ft | Gal/ft |
| 0.005 | 350 | 350 | 350 | 350 |
| 0.010 | 275 | 350 | 350 | 350 |
| 0.025 | 225 | 300 | 350 | 350 |
| 0.050 | 175 | 250 | 325 | 350 |
| 0.100 | 140 | 200 | 260 | 320 |
| 0.200 | 130 | 160 | 210 | 260 |

## Porosity Design Factors.

In sandstone matrix acidizing, the formation porosity is used for volume calculations only:

1.	The post-flush or overflush volume should be calculated for 4.0 to 5.0 feet of radial penetration based on porosity.

2.	Porosity can be used to determine penetration of the live HCl if the solubility in HCl is known. If HCl solubility is greater than or 	equal to 10.0% then the standard HCl volume of half the HCl:HF volume is usually inadequate to prevent precipitation of reaction by-products.

3.	HCl volume is based on HCl solubility. The HCl preflush volume should be sufficient to remove all HCl soluble material in a two foot radius from the wellbore.

4.	HCl:HF volume requirements should be calculated based on a four hour contact time. Any longer contact time in the near wellbore area will result in damaging precipitates.

Where the formation damage is suspected to be shallow (0 to 2.0 feet) an estimate of the treating volume of acid required can be obtained from Figure 31 based on treating radius and formation porosity.

Where it is suspected that damage is deep (greater than 2.0 feet) the following equation can be used to estimate the volume of acid required:

Pore Volume 	= 	3.142 x R2 x H (P) x 7.4806

Where:

R	= 	Treating Radius in Feet.

H	= 	Formation Height in Feet

P	= 	Porosity Expressed as a Decimal.

Example: 20 vertical feet of 15% porosity formation, with a required treating radius of 7.0 radial feet.

Pore Volume 	= 	3.142 x (7)2 x 20 x 0.15 x 7.4806

= 	3455 gallons

= 	(173 gallons per foot of pay)

Figure 31: Gallons Per Foot of Treating Fluid for Differing Porosities.

### Optimising HCl:HF Acid Strength from Core Flow Studies.

A more exact way of designing acid volume (instead of using porosity and permeability “rules of thumb”) is by core flow tests.

To optimise the strength of HCl:HF mixtures used in sandstone stimulation, tests are carried out with several hydrochloric and hydrofluoric acid mixtures. These tests are run using sandstone cores at simulated bottom hole temperatures. The hydrofluoric acid in the acid mixtures serves two essential purposes:

- Dissolves and disperses clays present in the sandstone.

- Dissolves the silica coating that covers many carbonate deposits present in the formation allowing the hydrochloric acid present to react with these deposits.

Removal of the clays can lead to large increases in the flow improvement ratio, whilst removal of the silica coating, allowing dissolution of the carbonate deposits increases formation solubility. The overall effect of these two actions is significant increases in reservoir productivity.

To simulate actual flow conditions in a reservoir, initial permeability of the core is established by flowing brine through the core sample. An acid stimulation fluids are then flowed through the core in the opposite direction. Core permeability after acidizing (return permeability) is then measured by flowing brine or other fluids in the original direction. In this way flow of fluids to and from the wellbore are simulated as they would occur in the producing formation.

From the test results obtained, core permeability after stimulation (final permeability, Kf) can be compared with the initial permeability (Ki), and expressed as a flow improvement ratio:

Kf  	=	Flow Improvement Ratio.

Ki

Figure 32 shows the effect of various HCl:HF acid concentrations on the flow improvement ratio of Berea Sandstone (a universal laboratory testing medium). Clearly the maximum flow improvement ratio was obtained with a mixture of 15.0% HCl and 4.0% HF acid.

From the data produced with Berea Sandstones, it can be seen that acid concentrations of less than 15.0% HCl  and 4.0% HF are not as efficient at dissolving clay and silicates, therefore the flow improvement ratios are lower.

Figure 32: Flow Improvement Ratio of Various Treating Solutions in Berea Sandstones.

Hydrofluoric acid concentrations above 4.0% should not be used. As can be seen in Figure 32 above, the 15.0% HCl to 7.0% HF mixture, deteriorates the core causing a decrease in permeability.

The use of high concentrations of hydrofluoric acid cause sand formations to become unconsolidated, and can create problems by promoting the production of sand. This process is caused by the removal of too much of the silica and carbonate cementing materials present in the formation.

Generally a few pore volumes of 15.0%:7.0% mix of HCl:HF acid will unconsolidate a core, whereas many pore volumes of the 15.0%:4.0% mixture will leave the same core intact. In addition, a 15.0%:7.0% mix of hydrofluoric acid will form a large volume of precipitates. This is particularly true where the solution is made up from liquid hydrofluoric acid as opposed to ammonium bifluoride. The reason for the lesser problem with the ammonium salt is that, when the ammonium ions are dissociated and in solution they act as a chemical buffer which helps maintain a low pH thus minimising  precipitation.

- If representative cores are not available, the optimum volumes and concentrations should be determined by using local experience and guidelines presented in this manual, such as the Berea sandstone curves given in Figure 32. Since the ratios for a given reservoir can vary substantially, no generalised set of flow improvement ratios are presented for various formations.

## Reservoir Solubility.

As a general rule, formations of less than 10.0 per cent solubility are not usually stimulated with HF or hydrochloric acid.   Also, if the formation is greater than 20% soluble in Hcl, HF is not recommended.

Limestone and dolomite formations react at high rates with hydrochloric acid and at moderate rates with formic and acetic acids. The reaction of sandstone formations with these acids, however, is limited to the amount of calcareous material present in the formation. Hydrofluoric acid on the other hand, reacts with sand, silt, clay and most drilling muds.  Formulations for the acid treatment of sandstone reservoirs based on carbonate content  are given in Table 17.

Table 17: Acid Selection Based on Formation Carbonate Content and Temperature.

| Percentage of Carbonate in the Formation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Temp | < 5.0% | 5.0 to 15% | Temp | 10 to 15% | > 15% |
| °F (°C) | Treatment | Treatment | °F (°C) | Treatment | Treatment |
| < 200 (93) | 50 gal/ft (620 L/M) 15% HCl followed by HCl: HF | 75 gal/ft (930 L/M) 15% HCl followed by HCl: HF | < 250 (121) | 75 gal/ft (930 L/M) 15% HCl then wash perforations with HCl: HF | 100 gal/ft (1240 L/M) 15% HCl |
| 200-250 (93-121) | 35 gal/ft (430 L/M) 7.5% HCl followed by HCl: HF | 50 gal/ft (620 L/M) 7.5% HCl followed by HCl: HF |  |  |  |
| 250-350 (121-177) | 35 gal/ft (430 L/M) 10% Formic Formic:HCl or Formic:HF | 35 gal/ft (430 L/M) 7.5% HCl + 10% Formic followed by Formic:HF | 250-350 (121-177) | 50 gal/ft (620 L/M) 7.5% HCl + 10% Formic | 50 gal/ft (620 L/M) 7.5% HCl + 10% Formic |
| >350 (177) | 35 gal/ft (430 L/M) 10% Formic or Formic:HF | 35 gal/ft (430 L/M) 10% Formic followed by Formic:HF | >350 (177) | 35 gal/ft (430 L/M) 10% Formic | 35 gal/ft (430 L/M) 10% Formic |

Factors to be considered when choosing the appropriate acid strength are:

- Formation Solubility.

- Time within the formation.

- Corrosion of tubular goods.

- Reaction product effects.

- Sludging and emulsion forming properties.

To benefit from an acidizing treatment, all or part of the formation being treated must be acid soluble. Solubility is a measure of that fraction of the formation that will react with an acid treating solution expressed as a percentage. Although no theoretical basis has been developed for the figures given in Table 18, there is general agreement that low formation solubilities call for low acid strengths and high formation solubilities call for high acid strengths. The figures in Table 18 summarise current field practice.

Table 18: Acid Strength Based on Formation Solubility in HCl.

| Formation Solubility % (Percentage) | Acid Strength % Percentage) |
| --- | --- |
| 0 -10 | 3 - 5 |
| 10 - 20 | 5 - 7.5 |
| 20 - 40 | 7.5 - 10 |
| Greater than 40 | 10 - 28 |

## Preflushes.

Preflushes are often used ahead of an acid treating solution, to prepare or condition the formation, so that it will accept the acid in the most favourable sections and without creating damage.

In stimulating sandstone formations, it is generally recommended that the  hydrochloric and hydrofluoric acid mixtures be preceded by a preflush of hydrochloric acid commonly at a concentration of 10-15% HCl (See Figure 33). This preflush serves two purposes:

- It dissolves carbonates in the formation so that the following hydrofluoric acid will remain active to dissolve the clays and silicates.

- It removes calcium compounds, minimising the formation of insoluble precipitates.

(Refer to Tables Table 17 and Table 18 for other requirements)

Figure 33 can be used to calculate the required volume of 15% HCl Preflush required for a sandstone stimulation based on formation solubility and treating radius.

Example :

10 feet of pay zone with a solubility in HCl of 5.0%.

Required treating radius is 5 feet.

From Figure 33 (next page):

Gallons of 15% HCl required per foot of pay 	= 	310 gallons

For 10 feet of pay 					= 	310 x 10

Volume of preflush required 				= 	3100 gallons of 15% HCl

Figure 33: Recommended Preflush Volume in Gallons of 15% HCl Per Foot of Pay for Various Radii.

Table 18 : Volume of Hcl to Dissolve Hcl Soluble Materials for 2 ft.

Hcl Solubility (%)

| Gallons of Hcl Reqd | Porosity (%) | 2 | 4 | 8 | 10 | 14 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15% HCl | 15 | 24 | 48 | 96 | 120 | 168 | 216 |
|  | 20 | 23 | 46 | 92 | 115 | 161 | 207 |
|  | 25 | 22 | 44 | 88 | 110 | 154 | 198 |
|  | 30 | 21 | 42 | 84 | 105 | 147 | 189 |
| 10% HCl | 15 | 36 | 72 | 144 | 180 | 252 | 324 |
|  | 20 | 34 | 68 | 136 | 170 | 238 | 306 |
|  | 25 | 33 | 66 | 132 | 165 | 231 | 297 |
|  | 30 | 31 | 62 | 124 | 155 | 217 | 279 |
| 7.5 HCl | 15 | 48 | 96 | 192 | 240 | 336 | 432 |
|  | 20 | 46 | 92 | 184 | 230 | 322 | 414 |
|  | 25 | 44 | 88 | 176 | 220 | 308 | 396 |
|  | 30 | 42 | 84 | 168 | 210 | 294 | 378 |
| 5% HCl | 15 | 72 | 144 | 288 | 360 | 504 | 648 |
|  | 20 | 69 | 138 | 276 | 345 | 490 | 621 |
|  | 25 | 66 | 132 | 264 | 330 | 462 | 594 |
|  | 30 | 63 | 126 | 252 | 315 | 441 | 567 |

Other preflushes include:

- Aromatic solvents (eg. xylene) and/or diesel (not for gas wells)  plus a surfactant for removal of hydrocarbon deposits or as carrier fluids for acid retarders and anti-sludging agents.

- Mutual solvents and alcohols, for the prevention and removal of water blocks, improved clean-up and water-wetting, and where clay minerals are present.

- Selective acidizing formulations (SAF) .  A specially treated kerosene or diesel oil preflush. The preflush allows the formation to react with acid in the oil producing interval whilst restricting the invasion of acid into the water producing strata. 
Preflush Design Considerations.

1.	When acidizing sandstones, iron control additives are most critical here because iron bearing minerals are highly soluble in HCl. The ensuing mud acid treatment will come into contact with less iron because these minerals will have been removed (Ferrotrol Agents).

2.	Iron control additives are also important because the preflush will remove any iron scales from the tubulars and carry the into the formation.

3.	If asphaltenes are present, it is important to use Anti-sludging additives in the preflush (NE-32, ASA-251, MSS-100). This is the fluid that will come into contact and mix with the crude oil in the greatest volumes. It is therefore more likely to cause a problem with asphaltenes.

4.	A solvent wash ahead of the HCl preflush is strongly recommended if the crude shows a tendency for sludging. (Xylene, Toluene, and as a last choice Diesel).  If in doubt, use a solvent preflush - it has the side benefit of stripping the sand clean of oil and thus preparing it for HCl and HCl/HF.

5.	If carbonate scales have been deposited in the matrix, the preflush should be a solvent/acid dispersion (One Shot Acid). If the scale is the only damage mechanism involved, mud acid should not be used.

6.	Use a preflush of hydrochloric acid when treating sandstones with mud acid (hydrofluoric acid). The preflush prevents brine contact with the mud acid and potential precipitation of by-products.

hydrochloric Acid; preflush;

7.	As a general rule of thumb, the HCl preflush volume should be half the volume of the mud acid treatment, if the HCl solubility is less than 10.0%. If the HCl Solubility is from 10.0% to 18.0%, the volume should be calculated for removal of all HCl soluble material within a two foot radius of the wellbore. (Refer to Table 17).

## Diverting Treatment Options.

To obtain the best results from most acid stimulation jobs it is important that the acid be distributed over the entire production or injection interval. Without some method of diverting an acid treating solution, most of the acid will enter the most permeable, and often least productive sections of the formation, leaving parts of the producing zone un-stimulated. The following are guidelines for diverting:

1.	Use mechanical diverting whenever possible.

2.	Diverting before a gravel pack should utilise sand and HEC polymer only.

- Divide the zone into treatment stages.

- If the zone is less than 80 feet, base the treatment on 20 ft stages.

- If the zone is greater than 80 feet, base the treatment on 30 ft stages.

- Sand concentration should be 1.0 to 2.0 pounds per gallon.

- New completions will require 12.5 pounds of sand per foot of perforations. Re-completions will require 25 pounds of sand per foot of perforations.

- When the diverter stage reaches the cross-over tool, rate should be increased to 8.33 ft/second.

Example : 60 ft Zone New Completion.

Step 1 		: 	1000 gallons HCl

Step 2 		: 	2000 gallons HCl:HF

Step 3 		: 	3000 gallons 3% Ammonium Chloride

Step 4		:	6 barrels HEC containing 250 lbs 40-60 sand

Step 5		:	Repeat 1, 2, 3, 4

Step 6		:	Repeat 1, 2, 3

Step 7		:	Pump Gravel Pack System.

Diverting all other conditions.

General Best Results:

Oil wells	:	Oil soluble Resin.

Gas wells	:	65 Quality Foam.

### Foam Diverting Techniques.

When using foam diverting techniques, the first stage of the acid solution is injected into the formation as in a conventional acidizing job. This is followed by an aqueous solution of a foam producing surfactant which is displaced into the formation with a compressed gas such as carbon dioxide or nitrogen. Parts of this surfactant adheres to the rock, both in fractures and within the matrix. When this retained solution mingles with and is agitated by the following stream of nitrogen, foam is formed.

As the procedure continues and additional foam is generated, its  resistance to continued movement through fractures and rock matrix increases until the pressure required to sustain additional flow is greater than the pressure required to break down another section of the interval. At this point the rate of injection into the original section is reduced, and the newly opened section accepts the next portion of acid. This procedure is repeated as many times as necessary to stimulate the entire producing zone.

Alternatively, compressed gas is injected into the aqueous fluid stream containing the foaming surfactant at surface and foam is created. As the foam follows the acid pad into the formation the effects of multiple phase flow (liquid and gas) creates resistance to flow and results higher pressures. The higher pressure breaks down further sections of  formation and the newly opened section accepts the next portion of acid.

The advantages of foam diverting techniques over conventional diverting methods using solid blocking materials are as follows:

- Foam produces a block within the formation rather than a solid block at the well bore. It contains no solid particles thus reducing potential for damage to permeability.

- The compressed gas aids in cleaning silt and undissolved particles from the formation and  in the clean-up of fluids.

- Adaptability to a wide range of temperatures 70 °F to 350 °F.

## Selective Acidizing (Water Stimulation Prevention).

Since acidizing solutions preferentially enter water bearing formations, increased production of water is an unwelcome by-product of many stimulation jobs. When the water contact is in the perforated interval, then selective acidizing should be employed to prevent accelerated water production after treatment. When applying the selective acidizing technique, the acid is diverted into the oil bearing zone, resulting in increased oil production, whilst water production remains the same.

Table 19: Selective Acidizing Treatment Options.

| Situation | Treatment |
| --- | --- |
| Oil Well - Non Gravel Packed | SAF, SAF Mark II |
| Oil Well - Gravel Packed | SAF |
| Gas Well | 65 Quality Foam Preflush |

The principal component of the SAF (Selective Acidizing Formulation) system is a specially treated kerosene or diesel oil (not crude oil) preflush, usually  a minimum of 500 gallons in volume. This preflush is injected ahead of a conventional acid stimulation treatment. Preceding the preflush is a spearhead volume of 5.0 barrels of clean, water free crude oil, kerosene, or diesel oil. Between the preflush  and the conventional acid is a pad volume of 2.0 to 5.0 barrels of clean water free crude oil, kerosene or diesel oil. The spearhead and the pad prevent the preflush from contacting any water based fluids and precipitating prematurely, prior to entering the water bearing zone.

With the pay zone isolated by packers or bridge plugs, the SAF Mark II preflush enters the formation as a low viscosity fluid. Upon contact with any water in the formation, the preflush immediately forms an oil soluble precipitate at the water-SAF Mark II interface. This oil soluble precipitate will partially penetrate and effectively plug the permeability of the water producing strata. With this strata effectively plugged, the acid treatment is diverted into the oil producing strata. When the treatment is complete, the well is shut in and produced in the conventional manner. SAF Mark II systems can be used with any type of acid, with any formation and in all wells except dry gas wells.

SAF reacts with the formation water to form a emulsion block (oil external) which will thicken with additional water and thin with additional oil.

## Post-Flush Design Considerations.

1.	Use Ammonium Chloride at a concentration that is compatible with the formation (2.0 to 4.0%).

2.	If damage from precipitation of reaction by-products is suspected, consider using 3.0% HCl as the post-flush for greater pH control.

3.	Use 5.0% to 10% mutual solvent for wettability (US-40, 2). This is most important when high concentrations of corrosion inhibitor have been used.  Above 5% US-40, corrosion inhibitor loading may need to be increased.

4.	The post-flush prevents the mixing of the displacement fluid (brine) with the spent HCl : HF acid.

5.	Use sufficient volume to over-displace the acid treatment to between 4.0 and 5.0 feet radially.

## Over-flushing.

Over-flushing is the displacement of the acid treating solution with more than the volume of fluid required to clear the tubing and casing. This procedure is sometimes desirable and often necessary.  For example, when using retarded acid systems, the reaction time can be longer than the injection time. Greater penetration may then be obtained by over displacing the acid. The exact amount of over-flush used is related to the reaction time of the acid, therefore, for maximum penetration, just enough overflush should be used to keep the acid moving within the formation until it spends.

Occasionally it is not convenient to flow back the well immediately after acidizing. In such cases, the treating fluid should be over-flushed with water or brine to reduce the contact time of live acid on the tubing and casing.

Recommendations:

- An over-flush with hydrochloric acid is recommended in wells that are stimulated with mixtures of hydrochloric and hydrofluoric acids. The over-flush helps prevent plugging precipitates from forming until the acid treatment has been displaced away from the critical region of the near well-bore, thus reducing the effects of this damage.

- 4-5 feet of radial penetration is optimum volume.

- It is strongly recommeded that the overflush fluid contain 5-10% mutual solvent (INFLO-40) to strip the oil wet film left behind by corrosion inhibitors.

## Retarded Acid Systems.

Retarder techniques are used to improve the effectiveness of acidizing treatments by slowing the rate of reaction with the formation to achieve placement of live acid deep into the formation. Several types of retarded acid systems are available :

- Chemically retarded acid (BJ Sandstone Acid).

- Organic acids.

- Mixtures of organic acid and hydrochloric acid.

In order to effectively retard an acid system, an understanding of the factors effecting the rate of reaction of an acid must be considered.

### Reaction Rate Considerations.

Higher reaction rates may be employed to remove local well-bore damage, but a slower reaction is preferred. The intrinsic reaction rate of an acid cannot be changed, but it can be effectively controlled by emulsifying the acid in oil or by interposing a film of oil between the acid and the formation such as in Super retarded acid (SRA) systems.

The effectiveness of an acidizing treatment, to improve the production from any well, depends essentially on the ability of the acid to reduce resistance to flow of the oil or gas from the formation to the well-bore.

The rate at which acid reacts with the impeding material becomes more important as the acid's distance from the wellbore increases. The increase in production from matrix acid treatments depends upon 2 factors :

- Increased permeability.

- Radius of zone of increased permeability from the wellbore.

### Measuring Reaction.

Generally the rate of reaction between limestone and acid is measured by the rate of change in the amount of available acid. In conducting a laboratory test, small samples of acid are drawn from the reaction vessel and titrated. The fraction of available acid consumed in the reaction is then plotted against elapsed reaction time. Since the basic rate of reaction on a given formation at a particular temperature and pressure depends upon the acid concentration, the last part of the reaction can be much slower than the first part. For practical reasons, the end of reaction is taken as the time at which 90% of the acid has reacted rather than 100%.

### Controlling Reaction Time.

The reaction rate of the acid on the formation can be retarded by changing the properties of the acid treatment. Acid systems can be retarded by chemical and physical means or by mixing HCl with organic acid.

Some acids, particularly the organic acids, react more slowly than hydrochloric acid. As there is no known way to change the intrinsic rate of reaction between a particular acid and a soluble rock, it is necessary to introduce "interfering" agents to reduce the rate. This is accomplished by reducing the effective area of contact between the acid and the soluble formation by:

- Foaming the acid with nitrogen

- Interposing an oil film between the acid and the formation.(Chemically Retarded Acids)

- Chemi-adsorption of temporary alumino-silicate film (BJ Sandstone AcidSM).

The effective reaction rate of some acids may be reduced by dissolving them in non-polar solvents (such as the lower alcohols) or in mixtures of this type of solvent and water. This method is not in general use because of the cost of non polar solvents.

Rate of reaction is related to the rate of diffusion of hydrogen ions and reaction products through the solvent containing them, therefore, increasing the viscosity of the solvent decreases the reaction rate (Gelled Acids). This may be accomplished by means of certain organic gelling agents or by the use of inorganic salts. Some surfactants also act to retard the effective reaction rate.

## Organic Acids.

Only two organic acids, acetic and formic, have been used to any extent in well stimulation. Both these acids have the characteristic of relatively low reaction rates without the addition of additives. However, due to the additional cost neither has been widely used as a replacement for hydrochloric acid systems.

Acetic Acid.

Acetic acid is usually used in the range of 5.0% to 20% concentration . In this range 1.7 gallons of acetic acid will dissolve the equivalent of 1.0 gallon of hydrochloric acid. Acetic acid has the advantage of low corrosion rates with steel, and at temperatures below 200 °F (93 °C), no inhibitor is required where contact with the pipe is less than three hours. With inhibitors, acetic acid is used as a breakdown fluid and can be placed in the well prior to perforating. The low corrosion rate without the need for inhibitors allows the use of this acid for stimulating water wells for domestic consumption.

Formic Acid.

The properties of formic acid are generally between those of acetic and hydrochloric acids of the same strength. Normally, formic acid is used in concentrations of 10% or less due to the low solubility of reaction products formed. At a 10% concentration, 1.3 gallons of formic acid are required to dissolve the same quantity of carbonate as 1.0 gallon of 10% hydrochloric acid.

### Mixtures of Organic acid and Hydrochloric acid.

Mixtures of acetic and hydrochloric acid (Super Sol, EQH acids) are used as a compromise between the greater dissolving capacity of HCl and the slow reaction of acetic acid. These delayed reaction acids are normally used in fixed ratios of Hydrochloric to acetic acid (9:1, 8:2, 7:3, and 5:5). A ratio of 9:1 hydrochloric:acetic acid is the fastest reacting of these mixtures. The dissolving ability of each formulation is approximately equal to that of 15% HCl.

These acid mixtures provide retardation, with less corrosion at higher temperatures (above 200 °F (93°C)), and have the additional benefit that they will sequester dissolved iron after the acid spends in hot carbonate formations.

Formic acid is also used with HCl for high temperature applications where a slow rate of reaction is required. When extremely high temperatures are encountered mixtures of formic and acetic can be used.

## BJ Sandstone AcidSM .

(Ref. SPE Paper 37015 by G. Di Lullo and P. Rae, Sandstone Acid Technical Bulletin by G. Di Lullo, A. Sookprasong and J. Hibbeler)

BJ Sandstone AcidSM is a unique, patented retarded acid system for damage removal and stimulation of sandstone formations. It is applicable for moderate to deep penetration and makes use of new material - HV acid.  HV acid is a phosphonic acid complex (HEDP) with five hydroxyl groups attached.  HV will hydrolize ABF in situ as the pH of the acid solution increases during reaction, thus forming a reaction-rate and solubility-controlled acid. This is accomplished through chemical/physical adsorption and complexing/sequestering actions.  Also, HV competes with HF Acid for aluminium ions, forming a protective layer over  clays, which reduces reaction rate and somehow limits their solubility with HF acid.   Regarding quartz (the primary sandstone constituant);  HV acid improves the adsorption of the HF molecule onto the sand grain surface, thus enhancing reaction rate.  HV acid is also able to chelate calcium and magnesium silicates, forming a complex molecule which improves overall silicate solubility and reduces the tendancy for precipitation.  Sandstone Acid has the following beneficial properties:

- Sandstone Acid  allows deeper penetration of live HF acid.  Stim Lab’s comparison tests reveal the BJSSA’s clay coating mechanism and slow HF release resulted in a more uniform distribution of acid stimulation.

- Limits and retards the reaction of clays.  Reaction rate is retarded by a unique film forming process on clay minerals.  This film is temporary is alumino-silicate based, and is removed by a post flush of weak HCl.

- Higher dissolving power  on quartz compared to conventional HCl:HF acid mixtures of the same strength.  HV acid’s strong anionic charge improves adsorption capacity of HF on (+) charged sand grains and also holdis more silica salts in solution with its chelation ability.

- Delayed release of HF.  Multiple groups in HV have different “release” activity levels, so as HF reacts on the formation and activity changes, H+ is liberated and HF is formed in situ.

- Less precipitation of damaging by-products.  BJSSA’s chelating and dispersing properties, “hold” in solution and intefere with crystal formation - greatly inhibiting precipitation.  Slow release of HF also reduces the tendency for super-saturation

- Strong water wetting properties.  BJSSA is strongly anionic and thus water wets sandstones, making it easier for fluids to be produced.

- High working temperature range.  BJSSA does not rely on low pH to prevent fluoride and flurosilicate precipitation, therefore excess HCl is not needed and corrosion is much reduced.  The fact that HV is a weak organic acid and the slow HF release mechanism also aids in reduction of corrosion.

### Background Relating to BJSSA

Hydrochloric:hydrofluoric acid mixtures have been extensively used to treat sandstone reservoirs, however, their application has been restricted to restoring the natural permeability of the formation. A combination of rapid spending with clays and silicates, matrix unconsolidation in the near wellbore area and subsequent precipitation of various reaction by-products has limited the usefulness of mud acid for matrix stimulation treatments.  Several existing "retarded HF acids" mixtures have been tried with mixed results. The marginal reduction in reaction rate of these acid systems could not overcome the huge contrast in surface area between clays and quartz, which dominate HF’s reaction rate. Also, various “in-situ-generated” acids were developed and pumped, with questionable to poor results due to premature or improper mixing of the solutions, both in the tubulars and in the formation.

The framework of sandstone formations is normally made up of quartz, silicate grains, feldspars, chert and mica. These minerals are bonded together by secondary minerals that precipitated from connate water, occupying partially the original pore spaces, for instance, over growth quartz, carbonates and pore lining clays. The porous fraction and hence the permeability of the formation rock is sometimes reduced by invasion of water base filtrates, either from drilling, completion, workover or treating fluids. These fluids can damage the matrix by swelling and dispersing clays or even by inducing the precipitation of scales. Also, in high permeability formations under high differential pressure some particles can invade the formation and plug some pores.

The dissolution of these materials will enlarge and interconnect pores, allowing the produced fluids to move easily. Kinetics of mud acid reaction with sandstone is heterogeneous, complex and difficult to predict. However, the overall reaction rate depends mainly on the nature of the surface reaction within the pore. Unlike quartzitic fines, clays are unstable and easily protonated thus very reactive and therefore dissolved at a faster rate by HF acids. Nevertheless, when reacted with HF acids under isolated condition, both clays and quartz have similar solubility. The solubility of these minerals with 3% HF mixtures is close to 8 times lower than the amount of limestone dissolved per gallon of 15% HCl. Thus to produce in sandstone reservoirs equivalent stimulation results to those obtained in limestone, the acid volume must be several times larger.

During matrix stimulation treatments, the main factors that contribute to matrix dissolution by injected acids are the availability of minerals and their surface area. Therefore pore lining and flocculated clays, carbonates, scales and invading particles are more vulnerable to acid attack than any framework minerals. Moreover, clays have much higher surface area than quartz, hence increasing their dissolution rate, leaving quartz minerals virtually untouched. Thus, as more acid is injected, more clays and cementitious materials are dissolved especially in the big pores and the formation becomes progressively weaker until it eventually re-compacts to lower porosity and permeability levels.

The by-products of sandstone dissolution  by HF acids are amorphous compounds and complex fluosilicates, fluoaluminates and fluoride salts which by nature have extremely low solubility. They reach saturation at very low concentrations and produce precipitates with varied consistencies.  Consequently, to prevent permanent damage from occurring, the dissolving power of HF acid is adjusted so that no more than minor damage to the formation can occur. Actually engineers have limited HF acid volume and concentration so much that today, jobs are designed just to overcome damage (skin equal to zero) and true stimulation (negative skin) is out of the question. As secondary precipitation and unconsolidation effects on well productivity are difficult to predict, they are not accounted for in the design phase. Thus, there is a big discrepancy between the theoretical and the actual PI obtained in the field from these acid jobs .

### Retarded or Slow reacting acids

Slow reacting acids such as fluroboric, fluroaluminic and hexa-fluro-phosphoric were developed between the 70’s and the 80’s to increase the depth of permeability alteration. Although results were better, they were not substantial and not even commercially applicable on undamaged formations. Most of these acid systems rely on the use of weak organic acids and their secondary reactions to slowly generate HF. During sandstone acidizing treatments the reactions are mass transfer limited, i.e. : reaction rate is limited by the rate at which HF can reach the reactive surface. Thus, in a radial flow pattern live acid penetration is marginally increased (Fig. 6B) and unconsolidation and precipitation effects slightly retarded. Sequential treatments such as an HCl acid stage followed by a fluoride salt stage were tried to improve acid penetration. They rely on adsorption of the hydronium to the clay surface during the acid stage and generation of HF later while pumping the fluoride salt stage. Uneven distribution and placement of the fluids in the matrix and high clay dissolution produced mixed results from these treatments. Self generating mud acid eliminated these hurdles as it hydrolyzed in the formation to yield hydrogen fluoride. Unfortunately under these conditions, rock solubilization is limited to the acid in the pore space, which is insufficient to enlarge the pores enough to provide cost effective increases in well productivity. To counteract this limitation, self generating mud acid treatments were sometimes preceded with mud acid, creating again uncertainties as regards unconsolidation and precipitation problems.

The porosity improvement obtained (ratio between: original porosity + dissolved clays and solids, divided by the original porosity) is limited to the space occupied by clays and carbonates, which is a minor portion of the rock. Without destroying matrix integrity, post stimulation porosity ratios are thereby limited to 1.4 and penetration depth of less than 6 inches. This new porosity ring can improve the PI by no more than 10 percent  (Fig. 1) which in most of the cases is not worth the risk and jobs are designed only to remove damage. Because of these limitations, mud acids, retarded acids and in situ generated acids are unable to produce outstanding stimulation results on sandstone formations. It is clear from figure 1 that there must be a balance between live acid penetration and porosity ratio for a successful acid treatment. A porosity ratio between 1.6 and 2 and rx above 3 will produce healthy flow improvements. The only way to obtain higher and deeper porosity ratios would be to enhance quartz dissolution around the pores, as quartz occupies the major fraction of sandstone formations. An ideal acid then to "truly stimulate sandstone formations", must dissolve quartz faster, limit solubility of clays and/or carbonates and reduce or delay formation of secondary precipitates.

### Other Retarded Systems.

SGMA

One of the oldest retarded acid systems is SGMA (Self Generating Mud Acid), which is a registered trademark of Shell Oil Co.  The retarding effect of this system depends upon the rate at which methyl formate reacts with water to produce formic acid.  This reaction is slow at low temperatures (100 deg. F) and is extremely fast at elevated temperatures (200 deg.F).  The best temperature range for SGMA is 160 - 180 deg. F.  The acid mixture contains methyl formate, ammonium floride, and water.  The methyl formate reacts with water to produce formic acid which reacts with ammonium floride to produce HF acid.  This sysyem has not been used much since the inception of RMS, Clay Acid, and RHF in the early 80’s.

Another system no longer used is a sequential process in which Hcl is injected followed by an amonium floride solution.  The hydrogen ion (H+) from the Hcl is attracted to the cationic sites of the formation clays.  When the fluoride ions (F-) from the ammonium floride solution reaches the clay it combines with the hydrogen ions and forms HF acid on the clay.  Also, some floride ions will be attracted to the anionic sites on the clays.  The next acid stage will combine hydrogen ions with the adsorbed fluoride ions forming more HF acid.  The HF dissolves clays and exposes more cationic and anionic sites.  About 5 or 6 alternating stages of Hcl and ammonium floride are used.  There are no temperature limitations on this process.  The disadvantage of this process is that smaller quantities of clay are dissolved and the clay must have good ion exhange capacity.

Clay Acid

Dowell’s Clay Acid (Fluoroboric acid) is an acid that reacts with water to produce HF acid.  This is a four step reaction and all four fluorides are released from the fluroboric acid.  Clays are said to be “fused” in place, thus allowing deeper penetration.

Step 1		HBF4		+	H2O			HF+ HBF3OH

Step 2		HBF3OH	+	H2O	 		HF + HBF2(OH)2

Step 3		HBF2(OH)2	+	H20			HF + HBF(OH)3

Step 4		HBF(OH)3	+	H2O			HF + HB(OH)4

The first reaction is the only one that is retarded, the other three occur very rapidly, especially above 150 deg. F.  This limits is usefulness, and it is also known that Clay Acid produces more precipitation because it requires extended shut in times.

RHF

Retarded Hydrofluoric (RHF) by Halliburton makes use of aluminum chloride to sequester (or complexes) floride ions, making them available for hydrogen at a slower rate.  AlCl3 is added to a HCL-HF mixture and the reaction occurs as follows;

Step 1		AlF4-		+	H+			HF + AlF3

Step 2		AlF3		+	H+			HF + AlF2+

Step 3		AlF2+		+	H+			HF + AlF1++

The last floride is not released, but stays complexed with the aluminum.  The retarded system is applicable to higher temperature than Clay Acid.

### New HF Acid Mixture

BJ Sandstone Acid, a phosphonic acid complex, hydrolyzes the fluoride salts instead of HCl. This acid has 5 hydrogens available that will dissociate at different stoichiometric conditions, hereafter referred as "HV acid" (five hydrogens). Mixtures of HV acid and NH4HF2 produce an ammonium phosphonate salt and hydrogen fluoride.  Twenty gallons of HV acid per 1000 gallons of water are required to react with approximately 123 pounds of NH4HF2 to produce a 1% HF acid solution.

During preparation of Sandstone acid, an adequate supply of fluoride anions is provided by the ionization of the dissolved fluoride salt, but the formation of a molecule of HF requires a combination of a hydronium ion and a fluoride anion. Since the concentration of hydronium is kept low by the pH of the mixture, so is the concentration of active HF. Therefore, an equilibrium is created by the buffering action of the weak acid and the weak acid salt. When the acid reacts with the formation, the equilibrium of the system is upset and above process is activated again until equilibrium is re-established. Thereafter, as long as the concentrations of the solutes are adequate, the rate of acidization reactions of the solvents is quite constant. When sandstone acid is prepared, only the three strongest hydrogens hydrolyze to produce HF. The rest will not be utilized as they hydrolyze at pH higher than 4, which is not practical at downhole conditions. The strongest hydrogen behaves as HCl and the weakest as Carbonic acid. This buffering effect will slow down the reaction rate with clays and carbonates by several folds, equivalent to retarded acids. Also, the reaction rate can be fine tuned by partially replacing HV acid with another acid. For example, muriatic will form HF at a faster rate than organic acids. Depending on sandstone acid formulation, its starting pH will range between 2.2 and 3.8.

In addition, and more relevant to sandstone stimulation are the following synergistic effects : (1) When the reaction with the formation starts, an aluminum-silicate-phosphonate plating is formed over the clays by chemi-adsorption (Fig. 2). This coating has a thickness narrower than a micron, has low solubility in weak acids (HF acid, carbonic acid) and water, but is rapidly dissolved by HCl and to a lesser extent, by organic acids.

Figure 2 : Comparison of Dissolution Effects

Left side SEM, untreated smectite; center SEM, smectite treated with regular mud acid; right side SEM, smectite treated with regular sandstone acid. Notice the colloidal coating protecting the clay, and on the edax, the low silicon peak and the additional phosphorous peak.

From then on, this film will block further reactions with those clays, limiting their dissolution and, therefore, preventing disintegration of the matrix. Dynamic solubility tests on clays indicate that their dissolution is somehow proportional to the surface area - volume ratio of the clay. For example, on the lower end, smectite and illite dissolution are limited to around 35 percent and on the higher end, kaolinite dissolution is limited to near seventy percent. Because the coating over the clays is acid soluble, small volumes of muriatic acid or formic acid may be used to fine tune clay dissolution and to optimize the design. Figure 3 is a plot of smectite’s solubility versus time for 3% mud acid, sandstone acid and sandstone acid with added HCl or acetic acid.

The slope of the initial portion of  these curves (first ten minutes) measures the reaction rate; the subsequent portions of the graph (flat portion) measure to what extent each acid can dissolve and limit the clay solubility. (2) HV acid, as most phosphonates, has outstanding adsorption and water wetting properties that catalyze HF reaction with quartz. Even though the reaction rate is lower than mud acid, quartz solubility becomes higher as time passes. As can be seen on Figure 4, quartz solubility with sandstone acid is around fifty percent better than with mud acid. Adsorption 3 however, is dependent on the pH of the carrier fluid. Thus in the presence of HCl or acetic acid, the solubility improvement is reduced to near 30 percent. (3) Last but not least, like other anionic phosphonates, HV acid is a superior dispersant, has sub stoichiometric sequestering properties and is an excellent antiscalant, thereby deferring/inhibiting formation of precipitates in the near wellbore area.

Titration of sandstone acid with a solution of sodium silicate (20 Be) to pH 3.8 (12000 PPM Si) showed no formation of precipitates. Moreover, the solution remained stable for over 12 hours at atmospheric conditions. When the same titration test was performed with regular mud acid, the solution became cloudy at 6000 PPM of Silicon and at 12000 PPM malladrite precipitated heavily, even though the pH was still acidic (1.6).

With sandstone acid, as with self generating mud acids,  when the injection is ceased there is still live acid available in the pore spaces. Delaying flowback allows the remaining acid to continue reacting, but only with the quartz, as the clays are protected by the coating, improving the permeability even further.

### Theoretical Production Increase (Discription of Software)

Complete zone coverage is one of the most important factors affecting the success or failure of matrix stimulation treatments. To avoid injection of acids only on the path of least resistance, higher rates are recommended to generate enough differential pressure across the interval, thereby distributing the acid homogeneously. These matrix injection rates however, can not overcome the fast reaction of HF with clays and carbonates. Thus, most mathematical models used to predict production increases consider instantaneous clay dissolution and neglect quartz dissolution. When sandstone acid is utilized, these models must be modified to account for only a limited dissolution of clays and cementitious minerals and slightly higher quartz dissolution rates.

The graphical output of this model, Figures 6A & 6B, plots the PI ratio and acid penetration versus Volume of Acid injected on virgin sandstone formations treated with various HF acids. It assumes (1) clays dissolution is instantaneous but limited between 35% to 95%; (2) the dissolution of quartz is time-dependent; (3) porosity-permeability-PI (Fig. 1) relation proposed by Gatewood/Hall et al is valid and finally (4) that only 65% of the rock is contacted by the acid. This value was selected after evaluating acid returns on core flow tests and studying various porosity distribution on cores which maintained matrix integrity before and after acid stimulation (Fig. 5). Notice that pores with diameter smaller than 8 micron are hardly acidized, however, above this size the curves shift to the right indicating pore enlargement. From this plot, one can conclude that pore enlargement rather than pore interconnection is the main cause for permeability improvement.

### Stim-Lab Core Flow Testing

Frequently, core flow tests are run across short cores (3 inches) or series of cores (up to 3 feet) to help in designing matrix acid jobs. Nonetheless, due to the difference in flow pattern their results must be carefully evaluated and should only be used in a qualitative manner. In these tests the pore volume increases linearly instead of radially, producing optimistic porosity ratios and live acid penetration and overstated matrix deconsolidation and secondary precipitation effects. When testing with mud and retarded acids, long cores are required to allow for a complete acid reaction so that permeability responses are meaningful. Several tests 11 using four cores in series (having a combined length around 9.6 inches) were run to compare mud acid (benchmark) and sandstone acid at 1.5% HF concentration (10 pore volumes each). The Berea sandstone cores used in this study were cut from the same slab, their mineralogy was 82% quartz, 11% feldspars and 7% clays (4% illite, 3% kaolinite and traces of chlorite), having 18% porosity and 64.2 mD Klinkenberg gas permeability. This is a low clay/carbonate content core, thereby porosity ratios obtained after acidification with HF acids will be mainly limited by the amount of quartz and feldspars dissolved and the duration of the test. The results listed on table 1 show the inadequacy of mud acid to treat sandstone formations. It disintegrated half of the first core, stimulated the next two and plugged the last one. Also, based on the compressive strength of the cores (Table 2), we can see that the penetration was limited to no more than 6 inches. Weak sandstone acid + 1% HCl produced a substantial permeability improvement across the complete interval.

Also, the compressive strength data of all cores is equal to 66%  3% of an untreated core indicating even acid distribution. It is obvious from this data that BJ Sandstone Acid was still active on the last core and its performance was limited by its dissolving power (volume and/or concentration). A new set of core flow tests was then performed comparing regular sandstone acid (RSA), regular sandstone acid + 2% acetic acid (RSAA), regular sandstone acid + 1% HCl (RSAH) with commercially available retarded acid systems (3% fluroboric and 3% fluroaluminic). The results are listed on Tables 1 & 2. The retarded acids performed well stimulating all four cores. However, upon analyzing the permeability response and the compressive strength of the cores after stimulation, it became obvious the uneven acid distribution versus penetration depth. Also, the compressive strength of the first core clearly indicates that acid volumes and their concentration could not be increased. An interesting fact is that the permeability improvement of 3% fluroboric acid is only slightly better than weak sandstone acid, which may be attributed to its poor dissolving power with quartz. Like retarded acids, RSAA and RSA stimulated all four cores. RSAH however, proved too strong for the testing period, dissolving quartz and feldspars in excess leading to a zero compressive strength on the first core (this will not happen at higher injection rates and radial flow patterns). The permeability responses from the core flow test with sandstone acid show how easily it can be fine tuned for different mineralogies. Also, if compared with retarded acids, RSA produced overall permeability response 3 times higher. Interesting as well (Fig. 5), is the fact that sandstone acid produces higher porosity enlargement than retarded acids, probably due to higher quartz dissolution, but not as excessive as the combined dissolution of clays and quartz produced by mud acid .

### BJSSA Conclusions:

- Conventional Mud Acid is only adequate for shallow damage removal. Acid volumes and acid concentration should be kept as low as possible to minimize matrix unconsolidation and secondary precipitates.

- It is important to run multitap core flow tests to evaluate acid response. However, results are only qualitative and permeability response and treatment design cannot be extrapolated for field application.

- Retarded acids at least double live acid penetration when compared to mud acid (at the same concentration) on linear core flow tests. In spite of this, when this penetration is converted into radial geometry the final effects on depth of porosity alteration and production are minimal.

- Porosity distribution on untreated cores and acidified cores demonstrate that pore enlargement is the contributing factor for permeability improvement, rather than interconnecting new pores.

Table 1 - Multitap Core Flow Test

| ACID TYPE | PERMEABILITY RATIO |  |  |  |  | TREATMENT |
| --- | --- | --- | --- | --- | --- | --- |
|  | CORE 1 | CORE 2 | CORE 3 | CORE 4 | OVERALL | TIME |
|  | 1.35 inches | 2.5 inches | 2.5 inches | 3.25 inches | 9.6 inches | MINUTE |
| WEAK MUD ACID | 0.04 * | 6.63 | 2.90 | 0.85 ** | 2.67 | 144 |
| WEAK SANDSTONE ACID | 2.55 | 1.54 | 1.85 | 1.74 | 1.83 | 146 |
| REGULAR S. ACID | 49.00 | 1.91 | 1.32 | 1.73 | 8.32 | 144 |
| REGULAR S. ACID + ACETIC | 5.00 | 1.62 | 1.51 | 1.50 | 1.90 | 132 |
| REGULAR S. ACID + HCl | 23.60 | 2.20 | 1.78 | 1.47 | 4.63 | 122 |
| REGULAR FLUROBORIC | 3.50 | 2.10 | 1.46 | 1.76 | 1.96 | 126 |
| REGULAR FLUROALUMINIC | 7.80 | 1.96 | 1.58 | 1.66 | 2.58 | 96 |

*	-	UNCONSOLIDATION

**	-	PRECIPITATION

Table 2 - Multitap Core Flow Test

| ACID TYPE | UNCONFINED COMPRESSIVE STRENGTH (PSI) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | CORE 1 | MIDDLE SECTION | CORE 4 | AVERAGE | AVG. VIRGIN CORE |  |
| WEAK MUD ACID | unconsolidated | 3922 | 5500 | 3140 | 5297 |  |
| WEAK SANDSTONE ACID | 3667 | 3361 | 3489 | 3505 | 5297 |  |
| REGULAR S. ACID | 637 | 4253 | 5526 | 3472 | 5297 |  |
| REGULAR S. ACID + ACETIC | 1681 | 3073 | 3641 | 2798 | 5297 |  |
| REGULAR S. ACID + HCl | nil | 3056 | 4813 | 2623 | 5297 |  |
| REGULAR FLUROBORIC | 1732 | 2623 | 4635 | 2996 | 5297 |  |
| REGULAR FLUROALUMINIC | 1859 | 2623 | 3387 | 2623 | 5297 |  |

KH = 1250 mdft, S=0, Porosity = 12%, 80% Quartz, 19% Clays and 1% Carbonate. Temperature 180°F, 3% HF.

Consider no matrix unconsolidation or secondary precipitation effect.

### Reaction Time -vs- Surface Area

The time for HF acid to completely spend in a pore or a small gap can be estimated by;

t	=	-7.6 w log[1 - (VD/VT)]

2.67 Ra

t	=	time, seconds

w	=	width of gap or diameter of pore, ft;   =   K0.5 * 3.281x10-6

VT	=	Maximum lbs of silicates dissolved per gallon of acid

VD	=	lbs of silicates dissolved at different spent acid concentration

Ra	=	Experimental reaction rate with HF on silicates are;

at 80°F,	Ra = 7.66 X 10-6 lbs/sec. ft2

at 150°F,	Ra = 21.30 X 10-6 lbs/sec. ft2

When acidizing an extremely high permeability formations (65 Darcies), which pore diameters are about 250, the time required for conventional HF acid to dissolve the clays until equilibrium is reached  is approximately 111 seconds at 150°F, but on a  6 md rock this reaction is instantaneous, as the example below illustrates:

Assuming equilibrium is reached after acid spending reach 90%, reaction time is calculated as follows:

1.	65 darcy at 150°F,	t   = -7.6 x 8.33 x 10-4 log {1- [ (0.22 x 0.9)  0.22] }  =  111 seconds

2.67 x 21.30 x 10-6

2.	6.5 md at  150°F,	t   = 1.1 seconds

The conclusions that can be drawn from the analysis of permeability (surface area) are :

1.	HF acid spends proportionally to pore diameter (surface area), therefore in typical sandstone formations, reaction will occur  in a few seconds.

2.	As clays and silicates have a surface area/volume ratio more than 1,000 times higher than quartz they will be completely dissolved, while quartz grains will remain essentially intact.

3.	An acid system that  will react with clays slowly and only to a certain limit will have greater penetration.

### Example Calculation

Because Sandstone Acid limits clay reaction to known values (see Figure-3) a simple method of treatment prediction may be employed.  There are several models for prediction of matrix acid treatments in sandstone formations.  One  is based on instantaneous reaction of clays/fines and a proportional increase in pore size and thus permeability.  Another, more complex model, is based on “retarded” systems which spend over time creating acid fronts with changing acid concentrations. Sandstones with an appreciable amount of clay and/or fines will generally follow the first model.  The following example describes the first model (but with a solubility limit for clays) and is based on a formation without any damage.  With skin damage the treatment results would be much greater.

Given:

- 10 foot interval of 8-1/2” open hole.

- 660 ft. drainage radius, no skin damage.

- 15% smectite.

- 10% porosity, 10 md permeability.

- Treatment-1		-	gal/ft of 12:3 HCl-HF at  2 BPM at 2 bpm.

- Treatment-2		-	gal/ft of equivalent BJ Sandstone Acid at  2 bpm.

Acid penetration can be calculated as follows;  total acid volume = 300 gal/ft x 10 ft = 3,000 gals.; The following formulas describe these reactions; 12:3 HCl:HF can dissolve 0.2 lbs of smectite/gal, therefore the total lbs dissolved is 600 lbs or 4.4 cuft.  Volume of formation clay can be estimated by;

V clays	=	(ra2 - rw2) x (1 - ) x clay% x s.f. x ht.	where; s.f. = solubility factor, ht .= height

__________________________________

ra	=	{V clays  [ x clay% x (1 - ) x ht.]} - (rw)2

_________________________________

so	ra	=	[4.4  (  x  0.15  x  0.9  x 10)] - (0.354)2		=	0.96 ft.

If we use Sandstone Acid at 300 gpf instead of 12:3 HCL:HF, the solubility will be limited to 45% (from Figure 2 at 107 min, 107 = 9,000 gal / 2 bpm)., therefore the acid penetration will be:

_____________________________________

rsa	=	 [4.4  ( x  0.15 x 0.9 x 10 x 0.45)] - (0.354)2	=	1.54 ft.

Treatment effectiveness can now be estimated using the graph below (Figure-7) and Darcy's equation to determine a new average permeability.

Regular acid		Porosity Ratio		= { + clay (%) x  (1-)} /  	=	2.35

Permeability Ratio	= 110  = Kafter/Koriginal; so Ka  	=	1,100 md

Kavg	=	          In (re/rw)     	=		In (660/0.354)	       	=	11.5 md

In(ra/rw) + In(re/ra)		In(0.96/0.354) + In(660/0.96)

Ka	        Ke		       	      1,100	        10

BJSSA	 Porosity Ratio = (0.1 + 0.15 x 0.45 x (1.0 -0.1)/ 0.1 	=	1.61

Permeability Ratio = 12; so Ka 				=	120 md

Kavg								=	12.2 md;

Figure-7

### BJSSA Job Design

BJ Sandstone Acid  solutions are applicable for use in oil, gas, geothermal and injection wells where moderate-to-deep penetrating matrix stimulation is desired. It is not intended as a perf wash treatment or in cases where it is known that only very shallow damage is present.  For these typ jobs Regular Mud Acid is sufficient.

Various formulations of the basic HV : Acid; HF (Acid refers to HCl, Acetic and Formic) mixtures can be used, depending upon mineralogy, temperature and the type of damage to be removed.  Treatment design for acidizing a sandstone formation requires knowledge of the HF acid reactions with formation minerals and reaction products that result.  Generally, these treatments should follow the  guidelines used in matrix acidizing of sandstone formations with HCl:HF acid mixtures.  However, due to its ability to limit and retard reaction with clays and carbonates, high HF concentrations can be safely used.  As the clay solubility can be limited (not simply retarded) from 35% to 65%, higher penetrations can be obtained.  BJ Sandstone Acid is divided into three strength categories based on final HF strength (See below).

Half Strength		15 gpt HV + 15 - 45 gpt Other Acid   + 1.5% HF

Regular		30 gpt HV + 30 - 90 gpt Other Acid   + 3% HF

Strong 		 	60 gpt HV + 60 - 180 gpt Other Acid + 6% HF

Other acid refers to Acetic, Formic or HCl. .

Acid concentrations should be determined based on formation mineralogy (which dictates solubility) rather than by the damaging material plugging the formation.  When ever possible core tests should be done to determine treatment effect.   Two minerlogical considerations in sandstone acidizing are % HCl-soluble minerals and % clay minerals.  Various formulations of acid will dissolve damage, particularly small amounts of damage placed around the perforations and wellbore.

### Guidelines for Acidizing with BJ Sandstone Acid:

1.0	If HCL solubility is 20% or greater, use HCl only.

2.0	If HCL solubility is between 10% and 20%, select acid system through core flow  testing.

3.0	If HCL solubility < 10%, use 2 to 12  pore volumes (3’ damage radius) of BJSSA.

4.0	Mineralogy:

- Low clay (less than 5%), use half or regular strength Sandstone Acid with 30 gpt HCL or 30 gpt Formic (if above 210 degrees F).

- High clay content (greater than 10%), use regular or strong strength Sandstone Acid .

- High chlorite, use half strength Sandstone Acid with 15-30 gpt Acetic.  Preflush with 5% Acetic acid.

- High feldspar (greater than 20%), use half strength Sandstone Acid with 15-30 gpt Acetic or Formic.

### BJSSA Fluid Design / Mixing Notes

If the formation contains a high percentage of iron compound (Fe+3) such as hematite, magnetitemonite, it is necessary for all acid solutions to contain an iron reducing agent (enough to reduce 3,000 ppm of Fe+3 to Fe+2) in addition to the iron sequestering agent (Ferrotrol 300 or 300L).  When HCL is used, 2 gpt Ferrotrol 270 + 1 gpt Ferrotrol 271 is recommended.  For Formic and Acetic solutions Ferrotrol 210 at 5-10 ppt  is recommended.

All Preflushes and Overflush  formulations must incorporate enough Ferrotrol 300 or Ferrotrol 210  to sequester or reduce a minimum of 2,500 ppm iron.  If mineralogy is unknown or treating string is old and rusty, then use enough Ferrotrol to sequester 5,000 ppm iron in all fluids entering the formation.

Because of the limited solubility with clays, all Sandstone Acid formulations must contain 1 to 5 gpt Claymaster.

Fresh water should always be used for mixing Preflush, Sandstone Acid, and Overflush.   Field-water containing high levels of sulphate, calcium, sodium or potassium ions must not be used.

Sandstone Acid requires agitation or circulation of the water to facilitate the proper mixing of additives, dissolving of ammonium bifluoride (ABF), and proper mixing of HV acid, mineral and/or organic acid.  Mixing and storage tanks should be plastic or rubber-lined.

ABF dissolves faster in fresh water than in acid solutions.  Therefore, it should be added to mix water containing corrosion inhibitor, Claymaster, Ferrotrol, Inflo and NE-additives.  HV acid is added after complete ABF dissolution. Finally, the mineral or organic acid is added, and circulation continues until time of pumping.  The amount of mixing water should be equal to the required water minus all liquid additives.

The amount of ABF (96% purity, minimum), concentrated HCl (34% strength), Formic (88-90% strength) and/or Acetic (96% strength) required for mixing the base Sandstone Acid at various strengths are shown in the BJ Mixing Manual.

Corrosion inhibitor loading for Sandstone Acid is listed in Table-2, and corrosion inhibitor tables in the Mixing Manual are also applicable.

As with standard HF formulations, never use fibre glass mixing tanks or steel tanks with fibre glass coating.  Stainless steel tanks (361SS or better), polyethylene tanks and polyethylene lined steel tanks are all suitable for Sandstone Acid preparation.  If steel tanks are used (not recommended), additional iron and corrosion control are recommended.

## Acid Strength.

The strength of acid used is dependent on the solubility of the formation, with some basis placed on previous experience in a given area and common sense. (Refer to Table 17, page 101

If the formation has less than 10% solubility 3.0% to 15% hydrochloric acid is used.  Stronger hydrochloric acid strengths (15% to 28%) are used  to obtain more live acid further from the wellbore for deeper penetration. Where organic or mixed acids are used, strengths equivalent to the dissolving power of hydrochloric acid are used.

Table 20: Relative Retarding Action of Different Systems.

| Regular Acid | Mixture of HCl + Organic | Chemically Retarded Acid | Physically Retarded Acid |
| --- | --- | --- | --- |
| Least Retardation  Most |  |  |  |
| HCl Acid | Super-Sol (EQH) Acids | Sta-Live Acid Systems | Gelled Acids and Crosslinked Acid. |
|  |  |  | SRA-3 Emulsified Acid |
| HCl:HF |  | Retarded Mud Sol | BJ Sandstone Acid |
|  |  | SGMA |  |

# Applications of Nitrogen In Acidizing.

- Atomized Acid.

- Nitrified Acid.

- Foamed Acid.

## Atomised Acid.

Inject fluid as fine mist of droplets in gas phase:

- Where water blocks are present or are likely to occur during acidizing.

- Better removal due to higher mobility of gas.

- High velocity of expanding gas aids clean-up of fines etc. from the producing formation.

- High penetration of acid into smaller rock crevices and fractures.

- Increased coverage due to expanded acid volume.

Gas to liquid ratios.

- Use the correct amount of acid to remove all soluble material.

- Near well-bore blockages:

1:1 or 2:1 should be used (Nitrogen to Liquid).

- Larger areal coverage with given volume of acid:

4:1 to 6:1 should be used.

- Amount of nitrogen used is dependent on:

Bottom hole pressure.

Bottom hole temperature.

Calculated expansion ratio.

## Nitrified Acid.

- Effective and economic due to alleviation of need to swab.

- Due to its low solubility, nitrogen assumes state of compressed gas bubbles. Acts as source of energy inside injected fluids.

- Flow back, expansion of bubbles forces fluids towards well-bore and up to surface. Aids removal of formation fines etc.

- Additional volume due to gas, increases radial penetration for given acid volume.

- Volume of nitrogen in fluid needs to be calculated to determine correct volume of flush to be pumped.

- Each job must be designed individually.

- Use tables to determine ratio of gas to liquid based on well conditions.

## Foamed acid

- Foam is a gas in water emulsion using ± 1.0% surfactant as emulsifier.

- Stable, low fluid loss foams at bottom hole conditions are formed using 52 - 95% gas volume with a continuous aqueous fluid phase.

- Acid recovery is a problem in low pressure reservoirs. Foam gives immediate acid clean-up and eliminates swabbing

- Foam eliminates need for extensive acid reaction rate retardation

- Foam aids removal of formation fines etc.

### Nitrogen Retention.

- Nitrified acid usually contains 500 to 1000 scf/bbl of nitrogen.

- Nitrogen reduces hydrostatic head of acid, and allows natural flow.

- Nitrogen may separate from acid in formation. In this case low pressure wells may  require swabbing.

- Use of up to 1.0% foaming agent allows retention and dispersion  of nitrogen throughout acid.

- As the acid spends, gas in fluid emulsion remains intact.

- Gas provides energy to lift the spent acid.

### Foamed Acid Diverting.

- In large formations, acid will tend to enter most permeable or lowest pressure zone.

- Foamed acid can divert the treatment from one set of perforations to another.

- Foam viscosity can be very high (depends on shear rate).

- Increasing the fluid viscosity by use of foam pads between acid stages will divert the acid to new intervals by increased pressure.

- If all the acid is foamed prior to pumping it will have inherent diverting properties.

### Foamed Acid Applications.

Any well requiring acid stimulation is a candidate for foamed acid or staged nitrified and foamed acid treatments.

- High pump rates are not required and can be undesirable.

- Matrix treatments:

Use 60 to 70% quality foam

- Foamed acid fracturing:

Use 70 to 80% quality foam

Normally foamed acid is preceded by a nitrified acid spearhead to achieve better penetration. After initial penetration of low viscosity nitrified acid, foamed acid is pumped at 65% to 80% quality. This procedure provides:

- Deeper penetration due to low fluid loss.

- Good diversion due to increased viscosity.

In low API gravity oil wells.

- Spearhead with solvents (xylene or toluene) to remove any heavy deposits (asphaltenes or paraffins), and allow the acid to react with the  formation face.

- Allow the solvent to soak for several hours (4 hours) before commencement of the acid treatment.

Wells with scale problems.

- Fill tubing or casing with foamed acid over-flush with 10.0 bbl.

- Shut in well for controlled reaction time.

- Flow back well at highest rate possible.

- If no corrosion test data:

Inhibit acid for 4 hours at BHT

Unload well after 3 hours shut-in.

# Acidizing Additives

## Corrosion Inhibitors

Acid corrosion inhibitors (CI-11, 15, 21, 22, 23, 25, 30) are organic chemical used to reduce the corrosion of well tubulars by acid.  They adsorb on the face of tubing, casing and surface mixing equipment to form a protective oil-wet film which interferes with the production of hydrogen. The primary factors affecting corrosion are;

- Temperature

- Acid concentration / type / volume

- Steel type / surface area of pipe,

- Inhibitor type / concentration

- Agitation (flow reguime / velocity)

- Contact time

- Influence of acid additives (eg. mutual solvent)

- Downhole pressure

Nitrogen compounds such as analine, acridine, phenyl-hydrazine, pyridine and quinoline are combined with acetylinic compounds to make inhibitors.  See BJ’s Mixing Manual for more information on each inhibitor.

Research into corrosion has revealed that mixing different types of inhibitors resulted in a synergy, and that certain other chemicals which alone have no inhibition properties, when added to acid will “boost” their inhibition performance.  BJ’s inhibitor boosters are used at temperatures starting around 200 degrees F and are listed here; Hy-Temp-O (Fromic acid), Hy-TempI (Potassium Iodide), Hy-Temp 400 (Antimony complex).

Presently, there is no API specification for measuring corrosion rate, however, BJ has adopted a standard test and a standard recommended maximum value of weight loss (0.05 lbs/sqft).  For special chrome pipes and coiled tubin 0.02 lbs.sqft is recommended. In effect inhibitor should be doubled for coiled tubing applications.  BJ’s Mixing Manual has loading schedules for normal pipes (N-80 coupons), and Tomball Technology Center has a large data base of corrosion rates and recommended inhibitor loadings for various other metallurgy.  Prior to doing a job on non-conventional steel, consult with Tomball.

## Alcohols

Alcohols are used in acid to lower surface tension, increase vapor pressure thereby improving spent acid cleanup.  The use of alcohols has a particular application on wells of low permeability (< 5md) and low pressure.  Alcohols are nonionic and thus do not adsorb onto the surface of sandstones or carbonates matrix.  Thus alcohols will stay active (reducing surface tension) in solution during the critical time when the acid spends.  Methanol and Isoproponol are examples of alcohols added to acids for this purpose.  Both are commonly used in gas wells at concentrations of 10 to 50%.  An additional advantage to the use of alcohols is in the reduction in water saturation near the wellbore.  This is due to the increase in volatility from alcohol and formation water mixing, whereby future gas production vaporizes entraped formation water.  More information can be found in BJ’s Product Bulliten on Alcohol in Acid Mixtures.

## Foamers

With the advent of foam diversion, BJ has recently made several improvements in acid foaming additives.  Based on a recent Tomball study on various foaming additives, FAW-1, and FAW-21 are the recommended additives for any foamed acid treatments.  They are both amphoteric (can be anionic or cationic) and seem to have a favorable water wetting effect on sandstones (other older additives were oil wetting sand).   Also, the products have the best ability to sustain their properties in the presence of corrosion inhibitors (which have a negative effect on other foaming additives).  This means that they can be pre blended in the acid prior to the job and do not need to be injected on the fly. Reference Tomball Report R00-94-96, for more information on these two products.

## Clay Stabilizers

Formation damage can result from the swellinf, dispersion and migration of clays.  Stabilizers composed of inorganic multivalent cationic salts (Claytreat), and organic polymers (Claytrol-6) are the most commonly used additives to date.   See section on clay stabilizers in Formation Damage Chapter of this manual.

## Iron Control

Iron scales are normally found in the casing and tubing in wells and sometimes as the mineral deposits in the formation rock itself. When hydrochloric acid solutions come into contact with these scales or deposits, the iron compounds are partially dissolved and are carried in solution as iron chloride. As the acid becomes spent, the pH rises above 2.0, allowing the iron chloride to undergo chemical changes and re-precipitate as insoluble iron hydroxide. This re-precipitation can reduce formation permeability and injectivity.  BJ has three types of additives to address iron control; Chelating/Complexing Type, Reducing Agents, and pH Buffers.

Chelating/Complexing:

Citric acid (Ferrotrol 300) is a white granular organic acid material. It is used to "tie up" or chelate dissolved iron scales and prevent re-precipitation of dissolved iron from spent hydrochloric acid solutions. Often, citric acid is used with X-14 (Sodium Acid Pyro-Phosphate, or SAPP) to make the effects of suspension more stable.  SAPP acts as a pH buffer to keep pH low and reduce the chances of insoluble iron ppt.

The amount of citric acid added to the hydrochloric acid system depends upon the amount of iron that is present. The first 50 pounds of citric acid added to 1000 gallons of acid, will sustain 2000 parts per million (ppm) of iron in solution. Each additional 50 pounds of citric acid added will increase its sequestering property by an additional 2000 ppm.

Iron re-precipitation depends upon the retention time of the acid in the formation and the pH of the spent acid solution. In dolomite and limestone formations, hydrochloric acid solutions can spend rapidly and as the pH reaches 2.0, the dissolved iron starts to re-precipitate. Therefore it is important to start production or swabbing operations within an hour after the acid job is complete.  If ferric iron is below 500 ppm, an iron control agent is not required. Other Complexing/chelating agents include; Various salts of EDTA (FE-900. 900L, FE-1000).  FE-800 (Na3 NTA) is also good for higher temperatures but should not be used with HF systems, as the sodium will ppt.  FE-810 (aqueous NTA ) should be used for HF systems at higher temperatures.

Reducing agents convert Ferric to Ferrous and maintain this oxidation state.  Theses include: Erythorbic Acid (FE-210), Sodium Erythorbate (FE-200), and proprietary blends - FE-260L, FE-270L, and FE-271L..  The elimination of Ferric ions prevents the precipatation of Ferric Hydroxide as the acid spends and the pH increases above 2.5.  This also reduces the possibility of asphaltene precipitation and sludging.

pH control additives are used in conjunction with chelating or reducing agents to enhance performance.  Acetic Acid (Z-3) and SAPP (X-14) are the primary materials used in acid solutions to buffer pH.

See the BJ Mixing Manual for an excellent summary of iron control and BJ’s product line.

## Aromatic Solvents

The most common solvent in acidizing is xylene, although others are available, eg. toluene etc.  Xylene is an excellent preflush for sandstone acidizing of oil wells for several reasons.

- It strips the oil from the sand grains allowing acid to contact and react more effeciently.

- It greatly reduces the possiblity of sludging where iron/asphaltene problems exist.

## Mutual Solvents

Mutual solvents (US-40, US-2) are multifunctional nonionic agents which by definition are soluble in oil, acid, water and brines.  They contain strong solvent groups which provide wetting, demulsifing and surface tension reduction properties.  Mutual solvents are frequently used in Preflushes ahead of Hcl or HCL/HF in sandstone matrix acidizing to dissolve organics and aid in water wetting.   The most common mutual solvent used today is EGMBE (Ethylene Glycol MonoButyl Ether).  It is used at concentrations of 2 to 10% by volume.  Combinations of mutual solvent, aliphatic flurorcarbon, and methanol are marketed as flow enhancers.  Mutual solvents become critical when a treated formation has been exposed to large amounts of corrosion inhibitor (which generally oil-wet sand grains).

## Surfactants

Surfactants are chemicals with an oil and a water soluble group, with the ability to alter the liquid-liquid and gas-liquid interface.  Surfactants aid in acid penetration (LT-32), fluid recovery (Flo-Back 20, Inflo100, 150) and are used extensively as non-emulsifiers and demulsifiers (for sandstones; Aquaflow, NE-10, 18, 32, 41, 118, 940).  Other uses are foaming (FAW-1, 4, 18, 21); anti-sludge agents (AS-3,7); Dispersing Agents (S-400, 300, 500, SLA-48 & all FAWs); Suspending Agents (LT-21, EE-100); Micellar Solvents (MS-5, 10, 12, 15, TechniWet-455); Water Block Breaker (NE-118).  Many of these products may be new and the Acidizing Engineer should make himself aware of the total product line to optimise surfactant usage.  See BJ Mixing Manual for detail on surfactants.

Note	:	Other additives such as Scale Inhibitors,  and others are discussed in detail in 			the BJ Mixing Manual.

# Treatment Evaluations.

1.	Monitor the pressure response when the acid contacts the formation and during injection.

- The pump rate should be held constant throughout the job.

If not, the pressure response record is useless.

- The pressure should never increase when injecting the acid.

If it does, the acid is damaging the formation.

- If a pressure increase is seen when acid first arrives at the formation, it is probably plugging from solids that were present in the treating string. (A pipe pickling  treatment should have been performed).  Another possibility is the effect of CO2 generation from the initial reaction with carbonates.  As in the lab, CO2 can cause a temporary pressure drop increase.

- A gradual increase in treating pressure while acid is penetrating the formation, indicates precipitation of reaction by-products.

- A slight increase in pressure should be seen when any diverter contacts the formation.

If not, the diverter is probably ineffective.

2.	Collect and analyse spent acid returns for:

- pH

- Iron content.

- Presence of emulsions.

- Amount, type and size of solids.

- Presence of reaction.

3.	Compare productivity improvement with productivity potential.

Real-time computer analysis of formation skin damage is now possible by using monitoring equipment to measure pressures and rates during the performance of the job. This information is then transferred to a computer for calculation of bottom hole pressure and various other parameters. With the computer program (FracRT)  it is possible to optimise the size of an acid job (or stages) during the actual execution, as the skin damage is seen fall to a minimum. The program calculates and displays:

a.	Damage Ratio Value

b.	Paccaloni Style Injection Pressure vs. Injection Rate Plots.

Data collected during the job can be stored with the monitoring equipment (3600 or 3305 monitors) or on the computer and used for post job reporting and analysis.

# Acid Jobs That Do Not Work.

Acid treatments which fail to stimulate production have usually been troubled by one or more of the following problems :

1.	Using acid on formations which are inadequately perforated, or on sandstones which are not damaged.

2.	Using the wrong type of acid to remove the damage.

3.	Using dirty water in the preflush or overflush.

4.	Lack of Hydrochloric Acid Preflush when using Hydrofluoric Acid with sandstones.

5.	Inadequate mud acid volume, minimum volume should be 50 gallons per foot of pay.

6.	Lack of immediate clean-up with mud acid, even with acid post-flush, allows deposition of precipitates.

7.	Failure to clean the acid or water tanks.

8.	Additive misuse or overuse.

# Technical Reference

## General Chemistry.

### Acids.

Hydrochloric Acid.	HCl + H2O 	 	H+ 	+	Cl-

Hydrofluoric Acid.	HF + H2O 	 	H+ 	+	F-

Acetic Acid.	H3C-COOH + H2O 	 	H+ 	+	H3C-COO-

Formic Acid.	HCOOH + H2O 	 	H+ 	+	HCOO-

Water.	HOH + H2O 	 	H+ 	+	OH-

### Bases.

A base is a compound that when dissolved in (or hydrolysed by) water, produces hydroxyl ions (OH-) as the anion. Some examples of common bases are as follows :

Sodium Hydroxide.	NaOH + H2O 	 	OH- 	+ 	Na+

Potassium Hydroxide.	KOH + H2O 	 	OH- 	+ 	K+

Calcium Hydroxide.	Ca(OH)2 + H2O 	 	2OH-	+ 	Ca2+

Ammonia Gas.	NH3 + H2O 	 	OH- 	+ 	NH4+

### Salts.

A salts is a compound formed by the reaction of an acid with a base. The reaction is usually quite rapid and liberates a large quantity of heat. Some common examples of salt formation are :

Sodium Chloride	HCl + NaOH	 	NaCl	+ 	H2O

Calcium Chloride	2HCl + CaCO3	 	CaCl2 	+ 	CO2  +  H2O

Silicon Tetrafluoride	4HF + SiO2 	 	SiF4 	+ 	2H2O

Barium Sulphate	H2SO4 + Ba(OH)2		BaSO4  + 	2H2O

Magnesium Formate	2HCOOH + Mg(OH)2 		Mg(HCOO)2  +  2H2O

## Reactions of Hydrochloric Acid (HCl).

Hydrochloric	+	Calcium  	   	Calcium 	+ Carbon 	+ Water

Acid                	Carbonate             		Chloride    	   Dioxide

2HCl  		+  	CaCO3	 	CaCl2     	+ CO2  	+ H2O

Hydrochloric 	+	Dolomite 		Calcium  + Magnesium  + Carbon  +  Water

Acid                	  			Chloride     Chloride          Dioxide

4HCl  		+  	CaMg(CO3)2	 	CaCl2     +     MgCl2        + 2CO2+   2H2O

Hydrochloric 	+	Sand           		No appreciable reaction.

Acid

HCl  	+  	SiO2		No appreciable reaction

## Reactions of Hydrofluoric Acid (HF).

Hydrofluoric	+	Calcium		Calcium   	+ Carbon 	+  Water

Acid                		Carbonate        		Fluoride    	   Dioxide

HF 		+  	CaCO3	 	CaF2        	+ CO2     	+  H2O

Hydrofluoric 	+	Sand   		Fluosilicic  	+  Water

Acid                          		Acid

6HF 		+  	SiO2	 	H2SiF6       	+  2H2O

Fluosilicic 	+	Sodium  		Sodium        + Hydrogen

Acid                          		Fluosilicate

H2SiF6 	+  	2Na		Na2SiF6  	+  H2

Hydrofluoric 	+	Bentonite 	   	Fluosilicic 	+ Fluoaluminic + Water

Acid                		Clay		Acid           	   Acid

36HF 		+ 	Al2(Si4O10)(OH)2  		4H2SiF6 	+ 2H3AlF	   + 2H2O

## Reactions of Acetic Acid.

.

Acetic 	+	Calcium   		Calcium 	+ Carbon 	+ Water

Acid		Carbonate                	Acetate    	    dioxide

2HCH3CO2 	+  	CaCO3		Ca(CH3CO2)2 + CO2 	+ H2O

## Reactions of Formic Acid.

Formic 	+	Calcium   		Calcium 	+ Carbon 	+ Water

Acid       		Carbonate                	Formate    	   dioxide

2HCCO2H 	+	CaCO3		 	Ca(HCO2)2 	+ CO2	+ H2O

# Appendix / References