Total JV Catenary Operations

1.11 Planned Disconnect

A planned disconnect procedure is intended to be used when the barge can no longer keep station and must move away from the platform before there is time to POOH and normally rig down the tubing and connections.

A planned disconnect will take approximately 30 – 45 minutes, and may be changed to an emergency disconnect at any time.

The following sequences are to be followed:

1. The Supervisorinstructs the crew to execute a planned disconnect after discussion and consultation with Client Rep on site. (The barge captain and marine crew on the barge are notified of this planned disconnect and so will be at their stations during this period)

2. The injector operator notifies the reel operator, and then stops the injector for sand cleanout operations, CT needs to be picked up above initial top of sand or into tubing if not too far..

3. The injector operator notes the depth of the CT, then sets the pipe and slip rams on the 5-1/8” Combi BOP on the well head (the 3-1/8” Quad BOP will not be activated).

4. The Supervisordirects all fluid and nitrogen pumping to stop and depressurize, and confirm all pressure is zero.

5. All personnel are evacuated from the platform and barge deck except the injector operator, service supervisor, reel operator, and barge supervisor and CT operator stationed on the platform.

6. If there is no danger of collapsing the coil, the reel operator bleeds the pressure off the coil and inflow tests the BHA check valves. (by comparing wellhead pressure to circulation pressure once pumping has stopped)

Note

If there is danger of wellhead pressure collapsing the coil, the injector operator should close the shear and seal rams to control the well before bleeding off internal pressure.

7. Disconnect and stow all data, video, and power cables between the platform and barge on the platform by the CT personnel on the platform using bolt cutters

8. The barge service supervisor disconnects the coflexip hoses from the barge by activating the coflex hose disconnect on the barge. (The coflex hose disconnect procedure can be reviewed separately)

9. The area between the platform and tubing reel is cleared of all personnel, this need to be confirmed by the Senior supervisor.

10. If the inflow test confirms that the BHA check valves are sealing properly, the reel operator shifts the BOP valve to “close” (on the catenary head BOP mounted on the reel.) to cut the tubing (on the barge) then sets the reel brake.

Danger

All personnel must be clear of area between the reel and platform before cutting the tubing. Injury or death may result from being struck by the loose end of the tubing or hazardous fluid discharge.

11. The barge service supervisor reports by radio that disconnect is complete.

12. The injector operator advises the CT operator stationed on the platform to close the manual locks on the pipe and slip rams and closes the kill and flow tee valves to secure the well.

13. The Senior Supervisor directs the barge CT operator to bleed off the pressure in the Injector and priority circuits to zero and to manually disconnected one Injector power hose from the barge end using a pair of pipe wrenches to secure it and slack it off and then using hand to unscrew the cap

14. 12. The second Injector power hose will be disconnected using the hydraulic hose disconnect

15. 13. After engaging the 5-1/8” Combi BOP on the well head, bleed off pressure from the BOP circuit in the coiled tubing cabin and the BOP hoses for both 5-1/8” Combi BOP on the well head and 3-1/8” Quad BOP are disconnected using the hydraulic hose disconnect from the barge end.

16. 14. The Injector control and case drain hoses are also disconnected using the hydraulic hose disconnect on the barge end.

17. 15. Bleed off pressure from the remaining circuits in the coiled tubing cabin and shut down the CT unit

18. The Barge Captain and marine crew complete the anchor retrieving operation, once complete he will notify Client Rep and PESL supervisor.

19. The Supervisor directs the barge captain to move the barge away from the platform after consultation with Client Rep.

1.12 Emergency Disconnect

The emergency disconnect procedure takes place in approximately 20 minutes and is only used if there is no time to follow the planned disconnect procedure. It is important to realize that the planned and emergency disconnect procedures are only designed to release the barge from the platform and vice versa.

Emergencies involving well control should be handled in the conventional manner.

Here are the steps to follow in an event where emergency disconnect is required:

1. The Supervisor instructs the crew to execute an emergency disconnect after discussion and consultation with Client rep on site.

The barge captain initiates a red flashing light and audible alarm.(Barge Captain and marine crew prepare to engage anchor handling equipment to enable recovery of anchors after the disconnect procedure is completed)

2. The Supervisor directs all fluid and nitrogen pumps to stop and

Depressurize. Needs to confirm that all pressure is bled off and zero.

3. The injector operator notifies the reel operator, and then stops the injector, for sand cleanout operations CT needs to be picked up above initial top of sand or into tubing if not too far.

4. The injector operator notes the depth of the CT, then hydraulically closes all BOP rams on the 5-1/8” Combi BOP on the well head (the 3-1/8” Quad BOP will not be activated).

5. All personnel are evacuated from the platform and barge deck except the injector operator, service supervisor, reel operator, and barge supervisor. The operator and supervisor on the platform will be recovered later this needs to be confirmed by the supervisor.

6. The barge supervisor disconnects the coflex hose(s) by activating the coflex hose disconnect on the barge and evacuates the rear of the barge. (The coflex hose disconnect procedure can be reviewed separately)

7. The reel operator shifts the BOP valve to “close” (on the catenary head BOP mounted on the reel.) to cut the tubing (on the barge) then sets the reel brake.

Danger

All personnel must be clear of area between the reel and injector before cutting the tubing. Injury or death may result from being struck by the loose end of the tubing or hazardous residual fluid discharge.

8. All data, video, and power cables between the platform and barge are cut using bolt cutters stowed nearby on the platform by the CT personnel on the platform.

9. The barge supervisor reports by radio that disconnect is complete.

10. The injector operator advises the CT operator stationed on the platform to close the manual locks on all the

rams on the 5-1/8” Combi BOP on the well head and close the kill and flow tee valves to secure the well.

11. The Senior Supervisor directs the barge CT operator to bleed off the pressure in the Injector and priority circuits to

zero and to manually disconnected one Injector power hose from the barge end using a pair of pipe wrenches to

secure it and slack it off and then using hand to unscrew the cap

12. The second Injector power hose will be disconnected using the hydraulic hose disconnect (see picture)

13. After engaging the 5-1/8” Combi BOP on the well head, bleed off pressure from the BOP circuit in the coiled

tubing cabin and the BOP hoses for both 5-1/8” Combi BOP on the well head and 3-1/8” Quad BOP are

disconnected using the hydraulic hose disconnect from the barge end. (see picture)

14. The Injector control and case drain hoses are also disconnected using the hydraulic hose disconnect on the barge

end. (see picture)

15. bleed off pressure from the remaining circuits in the coiled tubing cabin and shut down the CT unit

16. The Barge Captain and marine crew complete the anchor retrieving operation, once complete he will notify Total

Rep and SLB senior supervisor17. The Supervisordirects the barge captain to move the barge away

from the platform after consultation with Client rep.

Note

Once an emergency disconnect is initiated, it cannot be reversed. If communications are lost, the disconnect

sequence is not affected.

1.13 Emergency Disconnect due to poor Weather conditions

A realistic weather update for the campaign duration will be provided by Total. Upon commencement of the operation

a daily update will be given and reviewed by Total and SLB team on site. A maximum heave height will be agreed by

all parties (catenary operations have been performed in 1.8m heaves in other locations). Once the weather conditions

are not acceptable to continue operations Total site Supervisor will give the directive to rig down equipment from the

platform. Once this is done the barge Captain and marine crew can then engage the anchor handling equipment and

tug to retrieve the anchors and allow the barge move away from the platform until weather conditions improve.

In the event that there is no time to rig down completely the equipment before the onset of bad weather condition,

then a planned disconnect procedure will be initiated similar to 1.11

A planned disconnect will take approximately 30 – 45 minutes, and may be changed to an emergency disconnect at

any time. The following sequence to be followed:

1. The Supervisorinstructs the crew to execute a planned disconnect after discussion and

consultation with Client rep on site. (The barge captain and marine crew on the barge are notified of this

planned disconnect and so will be at their stations during this period)

2. The injector operator notifies the reel operator, and then stops the injector for sand cleanout operations CT

needs to be picked up above initial top of sand or into tubing if not too far.

3. The injector operator notes the depth of the CT, then sets the pipe and slip rams on the 5-1/8” Combi

BOP on the well head (the 3-1/8” Quad BOP will not be activated).

4. The Supervisordirects all fluid and nitrogen pumping to stop and depressurize, and confirm

all pressure is zero.

5. All personnel are evacuated from the platform and barge deck except the injector operator, service

supervisor, reel operator, and barge supervisor and CT operator stationed on the platform.

20. If there is no danger of collapsing the coil, the reel operator bleeds the pressure off the coil and inflow tests

the BHA check valves. (by comparing wellhead pressure to circulation pressure once pumping has stopped)

Note

If there is danger of wellhead pressure collapsing the coil, the injector operator should close the shear and seal rams

to control the well before bleeding off internal pressure.